# LEM1_9 Variants

James Brakefield

Brakefield Research © 2014

- LEM: Logic Emulation Machine

2.a) Characterization:

1-bit data, 9-bit instructions

Bit accumulator and carry bit

Data memory consists of single bits

Instructions from block RAM: 1024 by 9 for Altera, also 2048 by 9 for Xilinx

Absolute addressing only

Goals:	small LUT count (<100), high frequency of operation, one or two pipeline stages

2.b) Instance:	LEM1_9min:

Supports 2K instruction ROM and 64 bit data RAM

Bit instructions to/from accumulator

XXXnnnnnn

Also bit operations between accumulator and carry bit

0000xxxxx

No branches or conditional control flow

Implemented in 2003

Device independence in 2014

2.c) Instance:	LEM1_9accum:

Supports 2K instruction ROM and 64 bit data RAM

Bit instructions to/from accumulator

XXXnnnnnn

Also bit operations between accumulator and carry bit

00000xxxx

Conditional branches and call using two word instructions

000xxxxnn	nnnnnnnnn

Call and return via very small address stack

2.d) Instance:	LEM1_9stk:

Supports 2K instruction ROM and 64 bit data RAM

Bit instructions to/from top of bit stack

XXXnnnnnn

Also bit stack operations

0000xxxxx

Conditional branches and call via two word instructions

000xxxxnn	nnnnnnnnn

Call and return via very small address stack

2.e) Instance:	LEM1_9stk:

Supports 2K instruction ROM and 32 to 256 bits of data RAM

Bit instructions to/from top of bit stack of 16+ locations

Return address stack of 16+ locations

XXXRnnnnn: R is 1 for a replace operation (pop on replace op)

000Rnnnnn: LD/STP	(load & push, store & pop)

001Rnnnnn: LDC/ST	(load complement & push, store & no pop)

010Rnnnnn: AND	(and to TOS)

011Rnnnnn: OR	(or to TOS)

100Rnnnnn: XOR	(xor to TOS)

101Rnnnnn: ADC	(add with carry to TOS)

1100nnnnn: increment memory by carry

1101nnnnn: decrement memory by carry

1110xxxaa aaaaaaaaa: CALL/JMP/JCC/JCS/JA0/JA1

111011nnn nnnniiiiR: data memory and op-code expansion

1111sccaa: swap X nop/complement/clear/set X C/A

111100000: for no-swap, C/A both NOP is the return instruction

XXXR00nnn is top of stack relative

XXXR01nnn is offset for data pointer?

XXXR1nnnn is data RAM

Memory allocation for scrolling 7-segment “Hello World”

000xxx	stack locations 1..5

000101	bit at address pointer location

000110	bit at address pointer location with adr auto inc

000111	bit at address pointer location with adr auto dec

001xxx	8-bit address pointer

0100xx	4-bit 7-segment decode register (eight outputs)

0101xx	1st digit on display (low end of large counter)

01100x	digit select (higher up on large counter)

011010	enable for digit select

Other		working data RAM

2.f) Instance:	LEM1_9ptr:

Supports 64+ word instruction ROM and 32 to 256 bits of data RAM

Bit instructions to/from top of bit stack of 4+ locations

Three bit RAM pointers with auto inc/dec using 9 data memory locations

Return address stack of 4+ locations

Parameterization: return address stack depth, instruction address width, bit RAM size, bit stack size

XXXRnnnnn: R is 1 for a replace operation (pop on replace op)

000Rnnnnn: LD/STP	(load & push, store & pop)

001Rnnnnn: LDC/ST	(load complement & push, store & no pop)

010Rnnnnn: AND	(and to TOS)

011Rnnnnn: OR	(or to TOS)

100Rnnnnn: XOR	(xor to TOS)

101Rnnnnn: ADC	(add with carry to TOS)

1100nnnnn: increment memory by carry

1101nnnnn: decrement memory by carry

1110xxxaa aaaaaaaaa: CALL/JMP/JCC/JCS/JA0/JA1

1110111nn nnnniiiiR: data memory and op-code expansion

111011000 0nnnnnnnn: initialize stack pointer

1110110xx 0nnnnnnnn: initialize pointer xx

1110110xx 1nnnnnnnn: unused

1111sccaa: {null/swap}x{nop/complement/clear/set}x{C/A}

111100000: for no-swap and C/A both NOP is the return instruction

XXXR00nnn is top of stack relative

XXXR01nnn is offset for data pointer?

XXXR1nnnn is data RAM

Pointer mechanism

000xxx	stack locations 1..5

000101	bit at address 1st pointer location

000110	bit at address 1st pointer location with adr auto inc

000111	bit at address 1st pointer location with adr auto dec

001000

001001	bit at address 2nd pointer location

001010	bit at address 2nd pointer location with adr auto inc

001011	bit at address 2nd pointer location with adr auto dec

001100

001101	bit at address 3rd pointer location

001110	bit at address 3rd pointer location with adr auto inc

001111	bit at address 3rd pointer location with adr auto dec

Memory allocation for scrolling 7-segment “Hello World”

0100xx	4-bit 7-segment decode register (eight outputs)

0101xx	1st digit on display (low end of large counter)

01100x	digit select (higher up on large counter)

011010	enable for digit select

Other		working data RAM

2.g) Instance:	LEM1_9stg:

Bit string dominated ISA:

xxxrpssuu:			instructions using pointers (ss /=11)

xxxrp1100 nnnnnnnnn:	instructions with absolute bit addresses

xxxrp11uu nnnnnnnnn:	instructions with pointer offset

xxx	op-code (move, and, or, xor, adc, andc, movc)

111	other op-codes (call, branch, load pointer, expansion)

r	replace, e.g., result to memory

p	enable pop/push (“r” selects pop/push) stack

ss	pointer mode: (uu), (uu)++, (--uu), (uu)+nnnnnnnnn

uu	pointer selection (stack is pointer #0)

1110xaacc:		set/clear/swap A and/or C, return,

1111sccaa: 		{null/swap}x{nop/complement/clear/set}x{C/A}

111100000: 		for no-swap and C/A both NOP is the return instruction

111111c1a: 		four available/redundant op-codes

Load/adjust pointers, call & branch, instructions using memory adrs

1111xxxaa aaaaaaaaa:	CALL/JMP/JCC/JCS/JA0/JA1

1111110uu nnnnnnnnn:	initialize data/stack pointer

1111111uu nnnnnnnnn:	adjust data/stack pointer

2.h) Instance:	LEM4_9stk:

Usable as a four-function calculator

Supports 2K instruction ROM and 32 to 256 digits of data RAM

4-bit data operations to/from top of digit stack of 16+ locations

4LUT instruction, small return address stack

Return address stack of 16+ locations

XXXRnnnnn: R is 1 for a replace operation (pop on replace op)

000Rnnnnn: LD/STP	(load & push, store & pop)

001Rnnnnn: LDC/ST	(load complement & push, store & no pop)

010Rnnnnn: AND	(and to TOS)

011Rnnnnn: OR	(or to TOS)

100Rnnnnn: XOR	(xor to TOS)

101Rnnnnn: ADC	(add with carry to TOS)

1100nnnnn: increment memory by carry

1101nnnnn: decrement memory by carry

1110xxxaa aaaaaaaaa: CALL/JMP/JCC/JCS/JAZ/JANZ

111011xaa aaaaaaaaa: JPL/JMI or by left shift/rotate thru carry?

1110110aa aaaaa0000: 4-bit LUT instruction?

1110110nn nnnnniiii: rotate/shift instructions (i /= 0)?

1110111nn nnnnniiiR: data memory expansion?

11110xxxx: LDI

11111ccaa: nop/complement/clear/set C/A

111110000: both C & A nop is the return instruction

XXXR00nnn is top of stack relative

XXXR01nnn is offset for data pointer?

XXXR1nnnn is data RAM

Memory allocation for scrolling 7-segment “Hello World”

000xxx	stack relative locations

00100x	4-bit 7-segment decode registers (eight outputs)

00101x	8-bit address pointer (unused)

001100	digit at address pointer location (unused)

001101	BCD enable, 7-seg enable, digit sel (from counter)

001110	digit at address pointer location with adr auto inc

001111	digit at address pointer location with adr auto dec

010000	1st digit on display (low end of large counter)

Other		working data RAM

2.h) Instance:	LEM1_9risc:

Supports 2K instruction ROM and 512 bit data RAM

Bit instructions among eight bit “registers”

XXXdddsss

Load/store instructions

11Xnnnnnn

Conditional branches and call using two word instructions

000xxxxnn	nnnnnnnnn

Call and return via very small address stack

3.b) Instance:	LEM1_9min:

Supports 2K instruction ROM and 64 bit data RAM

Bit instructions to/from accumulator

XXXnnnnnn

Also bit operations between accumulator and carry bit

0000xxxxx

No branches or conditional control flow

opHLT		"000000000"			wait for system clock

opCACC	"000010000"			clear A, clear C

opCASC	"000010001"			clear A. set C

opSACC	"000010010"			set A, clear C

opSASC	"000010011"			set A, set C

opCC		"000010100"			clear C

opSC		"000010101"			set C

opCA		"000010110"			clear A

opSA		"000010111"			set A

opOR2C	"000011000"			A | C to C

opNA		"000011001"			negate A

opNC		"000011010"			negate C

opNANC	"000011011"			negate A, negate C

opAND2C	"000011100"			A & C to C

opC2A		"000011101"			copy C to A

opA2C		"000011110"			copy A to C

opXAC		"000011111"			swap A and C

opST		"001nnnnnn"			store A at memory location “nnnnnn”

opLD		"010nnnnnn"			load A from memory location

opLDC		"011nnnnnn"			load A complement from memory location

opAND		"100nnnnnn"			AND memory location into A

opOR		"101nnnnnn"			OR memory location into A

opXOR		"110nnnnnn"			XOR memory location into A

opADC		"111nnnnnn"			sum to A, carry to C

3.c) Instance:	LEM1_9accum:

Supports 2K instruction ROM and 64 bit data RAM

Bit instructions to/from accumulator

XXXnnnnnn

Also bit operations between accumulator and carry bit

00000xxxx

Conditional branches and call using two word instructions

000xxxxnn	nnnnnnnnn

Call and return via very small address stack

opNOP		"000000000"			waste a clock cycle

opRTN		"000000001"			pop return stack into PC

opCACC	"000010000"			clear A, clear C

opCASC	"000010001"			clear A. set C

opSACC	"000010010"			set A, clear C

opSASC	"000010011"			set A, set C

opCC		"000010100"			clear C

opSC		"000010101"			set C

opCA		"000010110"			clear A

opSA		"000010111"			set A

opOR2C	"000011000"			A | C to C

opNA		"000011001"			negate A

opNC		"000011010"			negate C

opNANC	"000011011"			negate A, negate C

opAND2C	"000011100"			A & C to C

opC2A		"000011101"			copy C to A

opA2C		"000011110"			copy A to C

opXAC		"000011111"			swap A and C

opBR		"0001000nn"	"nnnnnnnnn"	branch to location “nnnnnnnnnnn”

opCALL	"0001001nn"	"nnnnnnnnn"	branch to location “n”, save PC

opBRA0	"0001010nn"	"nnnnnnnnn"	branch to location “n” if A zero

opBRA1	"0001011nn"	"nnnnnnnnn"	branch to location “n” if A one

opBRC0	"0001110nn"	"nnnnnnnnn"	branch to location “n” if C zero

opBRC1	"0001111nn"	"nnnnnnnnn"	branch to location “n” if C one

opST		"001nnnnnn"			store A at memory location “nnnnnn”

opLD		"010nnnnnn"			load A from memory location

opANDC	"011nnnnnn"			AND complement of memory location into A

opAND		"100nnnnnn"			AND memory location into A

opOR		"101nnnnnn"			OR memory location into A

opXOR		"110nnnnnn"			XOR memory location into A

opADC		"111nnnnnn"			sum to A, carry to C

3.d) Instance:	LEM1_9stk:

Supports 2K instruction ROM and 64 bit data RAM

Bit instructions to/from top of bit stack

XXXnnnnnn

Also bit stack operations

0000xxxxx

Conditional branches and call via two word instructions

000xxxxnn	nnnnnnnnn

Call and return via very small address stack

A is top of data stack and C is second from top on data stack

opNOP		"000000000"			waste a clock cycle

opRTN		"000000001"			pop return stack into PC

op0		"000010000"			push 0 to A

op1		"000010001"			push 1 to A

opnA		"000010010"			complement A

opDUP		"000010011"			duplicate and push A

opPOP		"000010100"			pop A

opSWAP	"000010101"			swap A and C

op		"000010110"

op		"000010111"

opXOR		"000011000"			pop C, A xor C to A

opXORC	"000011001"			pop C, not A xor C to A

op		"000011010"

op		"000011011"

opOR		"000011100"			pop C, A | C to A

opORC		"000011101"			pop C, not A | C to A

opAND		"000011110"			pop C, A & C to A

opANDC	"000011111"			pop C, not A & C to A

opBR		"0001000nn"	"nnnnnnnnn"	branch to location “nnnnnnnnnnn”

opCALL	"0001001nn"	"nnnnnnnnn"	branch to location “n”, save PC

opBR0P	"0001010nn"	"nnnnnnnnn"	branch to location “n” if A zero, pop A

opBR1P	"0001011nn"	"nnnnnnnnn"	branch to location “n” if A one, pop A

opBR0		"0001110nn"	"nnnnnnnnn"	branch to location “n” if A zero

opBR1		"0001111nn"	"nnnnnnnnn"	branch to location “n” if A one

opST		"001nnnnnn"		store A at memory location “nnnnnn”, pop A

opLD		"010nnnnnn"		push A, load A from memory location

opANDC	"011nnnnnn"		AND complement of memory location into A

opAND		"100nnnnnn"		AND memory location into A

opOR		"101nnnnnn"		OR memory location into A

opXOR		"110nnnnnn"		XOR memory location into A

opADC		"111nnnnnn"		sum to A, carry to C

3.e) Instance:	LEM1_9risc:

Supports 2K instruction ROM and 512 bit data RAM

Bit instructions among eight bit “registers”

XXXdddsss

Load/store instructions

010ddd0xx nnnnnnnnn

Conditional branches and call using two word instructions

001dddxnn nnnnnnnnn

001xxx1nn nnnnnnnnn

Call and return via very small address stack

opNOP		"000000000"			waste a clock cycle

opRTN		"000001000"			pop return stack into PC

6 unused codes

op02D		"000ddd001"			clear D

op12D		"000ddd010"			set D

opDC		"000ddd011"			negate D

4*8 unused codes

opBRA0	"001ddd0nn"	"nnnnnnnnn"	branch to location “n” if D zero

opBRA1	"001ddd1nn"	"nnnnnnnnn"	branch to location “n” if D one

opBR		"0100001nn"	"nnnnnnnnn"	branch to location “nnnnnnnnnnn”

opCALL	"0100011nn"	"nnnnnnnnn"	branch to location “n”, save PC

6*4 unused codes

opLD		"010ddd000"	"nnnnnnnnn"	load D from memory location “n”

opLDC		"010ddd001"	"nnnnnnnnn"	memory location “n” negated to D

opST		"010ddd010"	"nnnnnnnnn"	store D to memory location “n”

opSTC		"010ddd011"	"nnnnnnnnn"	store not D to memory location “n”

opANDC	"011dddsss"			AND complement of S into D

opAND		"100dddsss"			AND S into D

opOR		"101dddsss"			OR S into D

opXOR		"110dddsss"			XOR S into D

opADC		"111dddccc" "nnnnnnnnn"	sum to D, carry to C