LEM1_9 processor

James C. Brakefield

Summary:

Supports 64-2048 word instruction ROM and 16-32 bits of data RAM

Single bit data instructions to/from data RAM & accumulator

Return address stack of 4+ addresses

Parameterization: return address stack depth (4-32), instruction address size (6-11) & data RAM size (16-32).

Bit string dominated ISA:

000Rnnnnn:		LD/ST	(load/store)

001Rnnnnn:		LDC/STC(load complement/ store complement)

010Rnnnnn:		AND	(and)

011Rnnnnn:		OR	(or)

100Rnnnnn:		XOR	(xor)

101Rnnnnn:		ADC	(add with carry)

1100nnnnn:		increment memory by carry

1101nnnnn:		decrement memory by carry

xxx	op-code (move, and, or, xor, adc, andc, movc)

111	other op-codes (call, branch, load pointer, expansion)

r	replace, e.g., result to memory

nnnnn	absolute address

xxxR11111 nnnnnnnnn:	escape to larger data address

1110sccaa: 		{null/swap}x{nop/complement/clear/set}x{C/A}

111000000: 		no-swap and C/A both NOP is the return instruction

111011c1a: 		four available op-codes

Call & branch, instructions using memory adrs

1111xxxaa aaaaaaaaa:	CALL/JMP/JCC/JCS/JA0/JA1

Implementation:				6-LUTs:		min	max	Current	Form

Instructions: 2048x9 dual port block RAM							18K block RAM

PC: 5 to 11 bit register with N, RS and PC+1inputs			05	11	08	Dff+LUT

RS (return stack): (4-32) x (6-11) simple dual port LUT RAM		04 	08	04	32x6SDP

RSptr:2-5 bit register with RSptr+/- inputs				02	05	03	Dff+LUT

DM (data memory): single port LUT RAM				01	08	01	64x1

A & C bits							02	02	02	Dff+LUTs

Inst decode							10	10	10	LUT

Adders (2PC5-11 & 1RSP2-5)					16	36	19	LUT

MUXing (2 each 4:1 for PC5-11, RSP2-5 & AC2)			18	36	26	LUT

LUT Totals							58	116	73	LUT

Actual LUT totals	6-LUT, area setting,   140MHz				60	Artix7-3

Actual LUT totals	6-LUT, speed setting, 150MHz				84	Artix7-3

Actual LUT totals	4-LUT, area setting,     64MHz				170	Spartan3-3

Comments:

Use both ports of instruction ROM so 18-bit instructions execute in one clock.

Possible data memory expansion via 18-bit inst. and reserving an “n” value as escape code