LEM1_9ptr processor

James C. Brakefield

Summary:

Supports 64-2048 word instruction ROM and 32-512 bits of data RAM

Single bit data instructions to/from data RAM & accumulator

Return address stack of 4+ addresses

Data pointer RAM has four data pointers, leave pointer #0 at zero for absolute addressing

Parameterization: return address stack depth(4-32), instruction address size (6-11) & data RAM size (32-512).

P instruction bit is unused since data stack omitted

Bit string dominated ISA:

xxxrpssuu:		instructions using pointers

xxxrp11uu nnnnnnnnn:	instructions with pointer offset

000Rpssuu:		LD/ST	(load/store)

001Rpssuu:		LDC/STC(load complement/ store complement)

010Rpssuu:		AND	(and)

011Rpssuu:		OR	(or)

100Rpssuu:		XOR	(xor)

101Rpssuu:		ADC	(add with carry)

1100pssuu:		increment memory by carry

1101pssuu:		decrement memory by carry

xxx	op-code (move, and, or, xor, adc, andc, movc)

111	other op-codes (call, branch, load pointer, expansion)

r	replace, e.g., result to memory

p	enable pop/push (“r” selects pop/push) stack (not implemented)

ss	pointer mode: (uu), (uu++), (--uu), (uu+nnnnnnnnn)

uu	pointer selection

1110sccaa: 		{null/swap}x{nop/complement/clear/set}x{C/A}

111000000: 		no-swap and C/A both NOP is the return instruction

111011c1a: 		four available op-codes

Load/adjust pointers, call & branch, instructions using memory adrs

1111xxxaa aaaaaaaaa:	CALL/JMP/JCC/JCS/JAC/JAS

1111110uu nnnnnnnnn:	initialize data/stack pointer

1111111uu nnnnnnnnn:	adjust data/stack pointer

Implementation:				6-LUTs:		min	max	Current	Form

Instructions: 2048x9 dual port block RAM							18K block RAM

PC: 5 to 11 bit register with N, RS and PC+1inputs			05	11	11	Dff+LUT

RS (return stack): (4-32) x (6-11) simple dual port LUT RAM		04 	08	08	32x6SDP

RSptr:2-5 bit register with RSptr+/- inputs				02	05	05	Dff+LUT

PtrM (data pointer memory): single port LUT RAM			04	09	04	32x2SP

DM (data memory): single port LUT RAM				01	08	01	64x1

A & C bits							02	02	02	Dff+LUTs

Inst decode							20	20	20	LUT

Adders (2PC5-11, 1RSP2-5 & 1idx4-9)				16	36	33	LUT

MUXing (2 each 4:1 for PC5-11, RSP2-5, idx4-9 & AC2)		26	54	48	LUT

LUT Totals							80	153	132	LUT

Actual LUT totals	Kintex-7-3, area mode, 170MHz				105	6-LUT

Actual LUT totals	Kintex-7-3, speed mode, 180MHz				127	6-LUT

Actual LUT totals	Spartan-3						259	4-LUT

Comments:

Use both ports of instruction ROM so 18-bit instructions execute in one clock.