LEM4_9 processor

James C. Brakefield

Summary:

Supports 32-2048 word instruction ROM and 16-32 digits of data RAM

Single digit data instructions to/from data RAM and accumulator

Return address stack of 4+ addresses

Parameterization: return address stack depth (4-32), instruction address size (5-11) & data RAM size (16-32).

Digit adder can do either binary or BCD additions.

Digit string dominated ISA:

000Rnnnnn:		LD/ST	(load/store)

001Rnnnnn:		LDC/STC(load complement/ store complement)

010Rnnnnn:		AND	(and)

011Rnnnnn:		OR	(or)

100Rnnnnn:		XOR	(xor)

101Rnnnnn:		ADC	(add with carry)

1100nnnnn:		increment memory by carry

1101nnnnn:		decrement memory by carry

xxx	op-code (move, and, or, xor, adc, andc, movc)

111	other op-codes (call, branch, load pointer, expansion)

r	replace, e.g., result to memory

nnnnn	absolute digit address

11100nnnn: 		load A with n

111010000: 		C/A both NOP is the return instruction

111010acc: 		{nop/complement/clear/set C}x{nop/complement A}

11101100x: 		set digit addition to binary/BCD

11101101x: 		arithmetic shift left/right

11101110x: 		rotate A left/right

11101111x: 		rotate A & C left/right

Call & branch, instructions using absolute memory address

1111xxxaa aaaaaaaaa:	CALL/JMP/JCC/JCS/JAZ/JANZ

Implementation:				6-LUTs:		min	max	Current	Form

Instructions: 2048x9 dual port block RAM

PC: 5 to 11 bit register with N, RS and PC+1inputs			05	11	08	Dff+LUT

RS (return stack): (4-32) x (6-11) single port LUT RAM		04 	08	04	32x8

DM (data memory): single port LUT RAM				02	08	04	32x4

A & C bits							05	05	05	Dff+LUTs

Inst decode							20	20	20	LUT

Adders (2PC5-11, 1RSP2-5, 1idx4-9, 2A4)				24	44	41	LUT

MUXing (2 each 4:1 for PC5-11, RSP2-5, idx4-9 & AC5)		32	60	54	LUT

LUT Totals:							95	165	147	LUT

Actual LUT count	Kintex-7-3, area mode, 200MHz				158	6-LUT

Actual LUT count	Kintex-7-3, speed mode, 195MHz				144	6-LUT

Comments:

Use both ports of instruction ROM so 18-bit instructions execute in one clock (implemented).

Non BCD digits are ignored (left as is, carry likewise) (not implemented)?