# 2N10

James C. Brakefield

© 2014

A Serial binary & decimal Microprocessor

Operating seven bits at a time

Motivation:

With programmable logic (FPGA chips) it is possible to embedded custom microprocessor architectures into a chip along side of any additional interface circuits.  By embedding a microprocessor into the FPGA the slower control aspect of a combined hardware/software project can be implemented in software and the higher speed aspect implemented in programmable logic.  The FPGA approach also allows full customization of any embedded microprocessor: The instruction set can be augmented or trimmed as warranted.

The following exhibits a processor family of efficient, yet modest complexity.  Both binary and Binary Coded Decimal (BCD) can be supported.  Addresses and data can be independently configured as binary or BCD.  Variable length arithmetic is supported: Each byte holds a flag bit and either seven binary bits or two BCD digits.

The microprocessor is designed to allow both a BCD and a binary mode of operation.  BCD can be used where binary to/from decimal conversion is disadvantageous.

BCD coding allows a naïve approach to machine language and computer internals. Memory content, register values and addresses are in decimal thereby avoiding any conversion to/from binary.  This allows the developer to make do without assemblers, compilers and such (at least for coding small problems).

Text:

Description:

This is a high performance BCD computer.  The following design supports 2,000,000 digits of memory.  The BCD arithmetic is serial and variable length.  Two BCD digits are encoded into seven bits.  An eighth bit is used as a flag to terminate a BCD digit pair string at the most significant end (MSE).  Digit pair strings are addressed by BCD addresses at the least significant end (LSE).  Addresses increase from LSE to MSE.  Negative numbers are stored in 10’s complement form.

The instruction stream also uses seven-bit BCD plus flag-bit coding.  An instruction string is scanned LSE to MSE.  The A & B registers address the instruction operands and the C register addresses the result.  After the end of the instruction, A & B point to the next digit pair after the operands and C points to the next digit pair after the result.

The memory interface is an eight-bit wide asynchronous RAM with 2^n * 8 bits of memory.  In the minimal configuration “n” is twenty.  In the halted state an external device can probe memory.  The arithmetic compute cycle is: read operand digit-pair, read second operand digit-pair, compute and store result digit-pair.

The instructions are two or more digits.  Additional digits contained with the instruction may contain an address, an offset, or an immediate value.

Registers (6-digit address registers, 8-digit instruction register: 178-226 bits total):

Bits	Reg.	Description

24	IA	instruction address

7	I	instruction

24	A	operand address

24	B	operand & destination address

24	C	result address (optional)

8	D	data at A

8	E	data at B

24	L	loop counter

24	SP	data stack pointer (optional)

24	RS	return stack pointer (optional)

6	S	status register

carry/borrow

overflow

zero

positive

BCD/binary mode(optional)

interrupt mask

interrupt present

4	ST	processor state (binary code)

Function units

IA, A, B & C have BCD increment

L has a BCD decrement

D & E feed a BCD digit-pair arithmetic logic unit (ALU)

IA, I, A, B, C & L feed a mux,

the mux feeds two BCD to binary converters

which addresses memory

I has a digit enable such that instruction digit pairs are loaded into succeeding positions

SP & RS (optional) have increment/decrement

Processing states

Halt

Load 1st instruction digit pair (op-code)

Load 2nd instruction digit pair (LSE address)

Load 3rd instruction digit pair

Load 4th instruction digit pair (MSE address)

Load 1st indirect address digit pair

Load 2nd indirect address digit pair

Load 3rd indirect address digit pair

Store 1st address digit pair

Store 2nd address digit pair

Store 3rd address digit pair

Read D

Read E

Compute

Write ALU result

Interrupts

Illegal instruction

Long instruction

Interrupt input signal

Halt instruction

Number encoding

Unsigned BCD numbers are in radix-100 with the flag bit set for the MSE.  Signed BCD numbers are in radix-100, first flag bit is sign and MSE flag bit determines size (negative numbers are a minimum of four BCD digits).  Negative numbers are stored in 100’s complement.  Floating-point numbers have a two digit signed exponent and a signed mantissa (minimum BCD float is two digit exponent and four digit mantissa).  Mantissas are “digit” normalized so MSE of mantissa is greater than 10.

A BCD encoding

Two BCD digits are converted to seven binary bits (100 values of 128 possible values):

The LSE bits of each digit form the LSE binary bits (bits 1 & 0)

If both digits are < 8, the four remaining BCD bits are bits 5…2 of the binary value

If any digit is > 7, bits 6 & 5 of the binary value are 10

Bit 4 indicates which digit is > 7

If both digits are > 7, bits 6…2 are 11000

Alternative BCD encoding

Numbers are in radix-100.  Display unit does conversion to radix-10.  Flag bit used for MSE end indication, sign of signed integers and exponent sign of floating-point numbers.

A BCD to memory address encoding

Three BCD digits are converted to ten binary bits (1000 locations of 1024 available locations):

The LSE bits of each address digit form the LSE address bits (address bits 2…0)

If all three digits are < 8, the six remaining BCD bits are bits 8…3 of the binary address

If any digit is > 7, bit 9 of the binary address is one

Bits 8 & 7 indicate which digit is > 7

If two digits are > 7

Bits 8 & 7 are one and bits 6 & 5 indicate which digit is < 8

If all three digits are > 7, bits 9…3 are 111110

Alternative BCD to memory address encoding

Memory addressing is in binary.  Conversions to/from radix-100 and radix-10 are hidden.

Instruction set (two digit op-code, additional digits are a 9s complement value)

Register load & load immediate (16 codes)

Load A, B, C or L immediate

Load A, B, C or L indirect

Load A, B, C or L relative

Store A, B, C or L absolute

Register movement (12 codes)

Move register (A, B, C or L to A, B, C or L)

Branches (28 codes)

Relative & absolute

Condition true or false

Carry, zero, positive, overflow, flag in D, flag in E, always

Subroutine call (2 codes) (return address stored in memory as a branch instruction)

Relative & absolute

Looping (1 code)

Decrement & branch relative if positive

Arithmetic (16 codes)

Add & subtract

With & without result extension

Immediate & non-immediate

Two address & three address

Multiply, multiply-add, multiply-subtract & divide (16 codes) (optional, three address)

With & without result extension

Immediate & non-immediate

Memory moves & compares (16 codes)

Immediate & non-immediate

Move & compare

By count & by flag

With & without source expansion

Byte operations (14 codes)

And, nand, or, nor, xor, and1c, or1c, xnor logical connectives between bytes at A & B

Add & subtract between bytes at A & B

Load D from @A++

Load D from @IP++

Load E from @B++

Push/Pop (optional, 19 codes)

Push/pop A, B, C, D, E, L, SP, RS

Call, Call relative, Return

# Appendix:		Memory efficiency

Memory data density of mapping two BCD digits to 7-bits is log2 of 100/128 or 94.91%

Memory address density of mapping six BCD digits to 20-bits is (1000/1024)^2 or 95.37%

The product is 90.52% net memory efficiency for combined BCD arithmetic and addressing

# Appendix:		Register display interface

When this design was first formulated, FPGAs where relatively expensive compared to CPLDs.  Thus the design included provision for a CPLD implementation.  This appendix is part of the original CPLD design provision where individual registers where connected to dedicated IO pins:

In a CPLD the registers are connected to I/O pins with a tri-state connection possible.  By connecting all the register digits in parallel and by having a one-of-n digit-pair enable, a multiplexed seven-segment display is possible.  Assume a five-digit display, a two-digit op-code and a three-digit address.  A selection switch controls which address register is enabled.  The output enable serves as the digit select/drive, the digit bus is connected to a hex-decimal decoder.

The additional pin requirement is two times eight segment outputs, four-digit-pair drive outputs, three inputs from the selection switch.  Giving a total of 23 pins.  The register pins number 6 * 6 * 4 + 2 * 4 or 152 pins (six registers of six digits each: IA, A, B, L, T, DEF & the two op-code digits).

## Memory interface

The memory interface is 20+ address lines, eight data lines, CE, OE and WE for a total of 31+ pins.  External operation of memory is possible in the Halt State.  When halted and memory-disable are asserted, the memory interface is in high impedance (with resistor pull-ups on all 31+ signals).

## External control interface

The halt and step signals allow halting and single stepping.  Combined with the register display external debugging is possible.  Asynchronous reset allows initialization of all registers.

Signal count: reset, clock, halt, step, memory-disable, memory-address, memory-data, CE, WE & OE (36 total).

High performance in speed is by doing BCD arithmetic two digits at a time and high performance in memory density is by storing two digits in seven bits and high performance in memory density is by using 1000 of 1024 memory locations.  Additional performance can be had by doing four or eight digits at a time, and/or by using dual and triple port memory (with 3-port memory: read, compute & write done simultaneously).

Some op-codes (e.g. optional ones) are four digits long.