Combined “C” oriented and dual stack instruction set (CnSIS)

James Brakefield

Contents

Introduction

8-bit instruction units

Operate

Load/Store

Branch/Call

Op-codes

Simplest configuration

16-bit instruction for ROIS

Speed-ups

Implementation

Architecture

Compilers

Instruction Set

Introduction:

Intent is to establish an expandable and efficient instruction set which supports stack oriented computation and traditional “C” language memory organization.  Stack frames may or may not live on the ascending return stack (RS).  Data stack is descending.

The top of the data stack (TOS) is the accumulator.  The top of the return stack is the PC.  Additional dedicated registers are the stack pointers (SP and RSP), the level or “C” frame pointer (FP) and the thread or base pointer (TP).  A loop counter (LC) is also desirable.  The instruction register (IR) is cleared by non-prefix instructions.  The prefix instruction or any other instruction shifts the current IR left by eight and inserts the eight-bit prefix data or instruction at the LSB end.

IR	Instruction Register, prefix data accumulates here

TOS	Accumulator and virtual top of data stack

SP	Pointer to rest of data stack, e.g., pointer to the second on stack

PC	Program counter and virtual top of return stack

RSP	Pointer to rest of return stack where the return addresses are located

FP	Frame pointer, points to the location where previous FP is located

TP	Thread pointer, nominally the location for per thread global values

LC	Loop counter, used for subtract one and branch not zero instruction (SOB)

Consideration should be given to the augmentation of the data width by two or three bits which carry status information.  For integers it would be carry and overflow bits.  For floats it would be exact, overflow, underflow and rounding mode.  It would be expected that the data stack and return stack would support these per “register” bits.  Frames, bases and general memory would not support extra bits.  On a 16-bit implementation, a 12-bit PC would allow four status bits on call & return.

A given implementation may chose an 8, 16, 32 or 64-bit word size.  Those instructions that specify a data size that is less than the word size use only the lesser significant part of the word.  Those instructions that specify a data size greater than the word size need not be implemented (e.g. they cause traps).

8-bit instruction units:

- Operate				0RQXXXXX	R=return, Q=2nd operate, X=op-code

- Operate Prefix				1RPXSZMM	R=replace; M=mode; P=pop TOS

- Case Limit Prefix			1LLLLLLL	L= upper limit, lower limit is 0

- LUT Sizes Prefix			1DDDSSSS	D=2^0..6; S=1..16

- CC Prefix				1CCCNNNN	C=condition code, N=offset

- 1st Address/Immediate Prefix		1AAANNNN	A=register, N=offset

- Additional Address/Imm Prefixes	1NNNNNNN	N=additional offset bits

MM=(a),(a++),(--a),((a))	AAA=SP+N,RSP-N,PC+N,PC-N,FP+N,TP+N,+N,-N

SZ=8,16,32,64		CCC=Z,C,OV,P,GT,GE,…

- Operate									0RXXXXXX

  - With one location prefix				1AAANNNN	0R0XXXXX

  - With two location prefixes		1NNNNNNN	1AAANNNN	0R0XXXXX

  - With second operate and no location prefix		1RPXSZMM	0R1XXXXX

  - With 2nd operate & location prefix	1AAANNNN	1RPXSZMM	0R1XXXXX

  - With 2nd operate…	1NNNNNNN	1AAANNNN	1RPXSZMM	0R1XXXXX

XXXXXXX:	op-codes and op-code modifiers

AAA:	Relative to Thread pointer, Frame pointer, RSP, DSP or +/-PC and +/- Absolute

MM:	Allows use of relative location as a pointer

R:		Is operate to leave result at TOS or at relative location

P:		Pop or don’t pop the data stack after execution

Q:		Operate prefix if is prefix present, otherwise, 6th X bit

R:		Return after execution

SZ:		8/16/32/64-bit data (16/32/64/128 for floating-point)

NNNNNNNNNNNNN:	0…15, 0…2047, 0…262143 offset or absolute address

- Load/store								0RXXXXXX

  - With one prefix					1AAANNNN	0R0XXXXX

  - With two prefixes			1NNNNNNN	1AAANNNN	0R0XXXXX

  - With second operate and no location prefix		1RPXSZMM	0R1XXXXX

  - With 2nd operate & location prefix	1AAANNNN	1RPXSZMM	0R1XXXXX

  - With 2nd operate…	1NNNNNNN	1AAANNNN	1RPXSZMM	0R1XXXXX

AAA:	Relative to Thread pointer, Frame pointer, RSP, DSP or +/-PC and +/- Absolute

M:		Load/store

SZ:		8/16/32/64-bit data

NNNNNNNNNNNNN:	0…15, 0…2047, 0…262143 offset or absolute address

- Branch, call								0RXXXXXC

  - With one prefix, standard addressing			1AAANNNN	0R0XXXXC

  - With two prefixes, standard addressing	1NNNNNNN	1AAANNNN	0R0XXXXC

  - With one prefix, PC relative addressing			1CCCNNNN	0R0XXXDC

  - With two prefixes, PC relative		1NNNNNNN	1CCCNNNN	0R0XXXDC

  - With second operate and no location prefix		1CCCSZMM	0R1XXXXC

  - With 2nd operate & location prefix	1AAANNNN	1CCCSZMM	0R1XXXXC

  - With 2nd operate…	1NNNNNNN	1AAANNNN	1CCCSZMM	0R1XXXXC

AAA:	Relative to Thread pointer, Frame pointer, RSP, DSP or +/-PC and +/- Absolute

CCCC:	Condition codes (for relative branch): carry, overflow, sign, zero, …

NNNNNNNNNNNNN:	0…15, 0…2047, 0…262143 offset or absolute address

- Op-codes								0RXXXXXX

- NOP

- Software trap

- Return (restore previous PC and status)

- Return from interrupt

- Restore frame ptr & PC & status bits

- Push previous frame pointer

- Subtract one and branch non-zero

- Subtract TOS from LC and branch > zero

- Drop 1 word from DS

- Drop 2 words from DS

- Drop 3 words from DS

- Drop 4 words from DS

- Dup 1 word on DS

- Dup 2 words on DS

- Dup 3 words on DS

- Dup 4 words on DS

- Move 1 word from DS to RS

- Move 2 words from DS to RS

- Move 3 words from DS to RS

- Move 4 words from DS to RS

- Negate

- Absolute value

- Push Zero

- Push Negative one

- Shift logical

- Shift arithmetic

- Rotate

- Bit set (within bit banded range)

- Bit clear (within bit banded range)

- Bit change (within bit banded range)

- Field extract

- Field extract & append

- Field insert

- LUT

- Case dispatch

- Leading zeros count

- Ones count

- Test

- And

- Or

- Not

- Exclusive Or

- Add

- Subtract

- Compare

- Multiply

- Extended multiply

- Divide

- Remainder

- Divide with quotient and remainder

- Floating add

- Floating subtract

- Floating multiply

- Floating divide

- Floating add-multiply

- Floating compare

- Integers (number & exponent) to float

- Float to integers (integer & exponent)

- Branch/Call conditionally

- Branch/Call unconditionally

Above list of instructions not intended to be inclusive or permanently allocated.  Variants shall be tolerated and expected.

- Simplest configuration:

  - 8-bit data

  - 64x8: data stack (DS), return stack (RS) & instruction memory (IM) = 24 LUTs

  - ALU: ~16 LUTs?

  - Registers: 8-bit: TOS; 6-bit: SP, PC, RSP, FP & TP; 12-bit IR = 50 DFF

  - Associated adders: PC, SP, RSP, FP/BP = 24 LUTs

  - No loop counter, no multiplier

  - Single clock per instruction, e.g., no multi-cycle instructions

- Using 16-bit instruction word formats

  - ROIS

Operate				00XX XXXX   DDDD SSSS

Load/store local			01RR MMMN NNNN SSSS/DDDD

Branch, call, immediate		10AA NNNN NNNN  NNNN

Prefix				11NN NNNN NNNN  NNNN

  - Multi-accumulator

Operate				0XXX XXAA MMMR NNNN

AA:		Accumulator(1 of 4)	MMM:		Relative address	R:	Return

Prefix				1RXX AASZ NNNN NNNN

R:		Replace operation	AA:		Indirection		SZ:	Size

Is a single data stack sufficient?

- Speedups

  - Pipelining of course.

  - Have the program memory 32-bits wide.  Process the first instruction and its prefixes simultaneously.  Only gets as far as one instruction per clock.

  - Use a 16-bit instruction size so there are fewer prefixes and try to do dual issue.

  - Use a ROIS instruction set (fewer memory read/writes).

  - Add co-processor(s) and support long instructions with sections for each co-processor (e.g. a way to do multiple dispatch).

- Implementation

  - Pointers

The pointer registers are: Program counter, Top of data stack, Top of return stack, Frame pointer and Thread pointer.  For starters assume no replace operations, so only writes to either return stack or data stack are either stores or pushes.  Can get by with single ported LUT RAM for both stacks, however, dual ported LUT RAM gives better performance (more done per clock cycle).  Need two adders (reason: move to return stack from data stack instruction).

  - TOS ALU

Inputs: TOS, SOS, Data stack, Immediate, PC, RTA, Return stack, Counter.  Outputs to: SOS, Data stack, PC, Return stack (Frame, Thread or RTA relative), Frame pointer, Thread pointer.

Operations: add, subtract, and, or, xor, forward, shifts, multiply, insert & extract fields.

First operand: TOS or negative TOS, shifted left or right

Second operand: Data RAM or Return stack RAM read ports, immediate, zero

Register for status bits with a minimum of carry and overflow

  - PC ALU

Increment by one, relative address, load from TOS, Return address or immediate

  - Index ALU

Frame, Thread, Return stack or Data stack pointer plus displacement; load from TOS

  - Counter adder

Minus one or minus TOS

  - Data stack pointer ALU

Increment, decrement, hold

  - Return stack pointer ALU

Increment, decrement, hold

  - Return stack address

Index ALU or Return stack ALU

  - Data stack address

Index ALU or Data stack ALU

  - Instruction RAM address

PC ALU

- Architecture Unification  (will be moved to and replace Introduction)

  - Architecture character:

The market being addressed is soft-core processors using FPGA fabric.  As such a number of small processors running at high clock rates can be preferable to a larger, more powerful processor running at a slower clock rate and using more resources.  Indeed, the fixed size FPGA resources for RAM create sweet spots for program and data capacity.  Ideally one wants to use only one or a few Block RAMs per processor.  Use of a DSP block for the ALU can lead to low logic resource usage and to high clock speeds.

The architecture character herein is oriented towards a generic programming language style, that being that of algebraic languages with evaluation of expressions and subroutines.  This leaves out those special features for functional programming or event driven architectures.  And the architecture character is oriented towards a simple mapping between algebraic language operations and those operations in the instruction set.

The overriding architecture character is that a range of processors are possible using a range of resources and implementation complexities.  At its core this class of architectures uses an accumulator, data and return stacks.  And IO registers to connect to the outside world.  As memory needs increase one adds data memory and instruction memory.  As these memories get larger, they migrate from LUT RAM to Block RAM(s).  As performance needs increase, data widths increase and co-processors are added.

The instruction set is very fluid: using prefix fields, both long-complex and short-simple instructions can be handled efficiently.  As the instruction register can always be made larger, adding additional parameters and op-codes can be done without changing much besides instruction decode.

Short instructions use default values for things such as data size, operand and destination locations (e.g. on the stack for minimal instruction size).  Larger word size versions can be upwards compatible with their smaller brothers by including an instruction field for data size.

  - Instruction character:

The basic instruction has a small, say 8 or 16-bit unit.  Instruction can be lengthened by prefix units and is gathered in the instruction register by a shift and insert sequence.  The entire instruction register is cleared after each instruction.  The entire instruction register is always in use.  The basic instructions are short with additional parameterization being zero.  Thus a short format load immediate op-code may only load zero but with additional prefixes can load a succession of larger data values up to or greater than the accumulator size.  This gives instruction memory economy for the simple modes for various op-codes.

The instruction memory can be made wider than the unit size so that prefix look ahead can gather all the prefixes in fewer clock cycles.  And instruction memory can have a unit size different from the data unit size.

Processing efficiency follows from each instruction doing several things.  Thus the simple operations use the accumulator and the two stacks.  More elaborate operations access data memory via Thread and Frame pointers and do replace operations to the selected memory location.  A return bit can be included in the instruction so that a distinct return operation is seldom needed.  The Thread and Frame pointers can be operated in auto-increment/decrement mode for sequencing through data memory.

Dual stack processors are easy targets for simple compilers.  For short, simple subroutines the dual stack model works well and has low call/return overhead.  Larger subroutines, which seem to be the norm, require a stack frame where local data and local parameters are kept.  For the architecture considered herein, entry to a “large” subroutine requires pushing the Return address and the Frame pointer, copying parameters from the data stack to the frame, and initializing local data.  Exit from a “large” subroutine requires copying results to the data stack, popping the frame pointer and the return address.  It is envisioned that compilers will use optimization criteria to choose between stack based and frame based subroutine structure and also optimize when to in-line subroutines.  The subroutine call for a “small” subroutine can be very fast via the use of LUT RAM for the Return stack and most “small” subroutine returns can be merged into the last subroutine instruction execution via a “return” bit in the final subroutine operation.

  - The memory structure is:

    - Data stack and its pointer, via simple dual port LUT RAM

    - Return stack and its pointer, via simple dual port LUT RAM

    - Global/Thread/Frame memory, via a single port RAM

For small data memory needs, a LUT RAM is efficient.  For larger data memory, block RAM or external RAM can be provisioned.  Efficient use of block RAM requires adding pipeline stage(s).

    - Program memory, mostly read only and can be wide

Certain alignment strategies can be used to provide high performance for long instructions.  With minimal decode or shifting alignment, long instructions can be dispatched rapidly, e.g., minimizing prefix cycles.

    - Co-processor register files, via a multi-port read, single port write LUT RAM

Each register file caters to a specific class of instructions such as floating-point, multi-media or loop/address counters (with counter, delta, refresh value & loop start address).  Multiple instruction issue possible using long instructions containing multiple co-processor operation fields.

The co-processor logic is built around the co-processor register file.  Thus multiple identical co-processor instances are possible and several distinct co-processor types are possible.  For multi-cycle co-processor operations a variety of interlock possibilities exist including a queue for operations in waiting.

  - The register structure is:

    - Accumulator, the center of data activity, at least until co-processors are added

    - Program counter, the center of program flow

    - Instruction register, herein can be small to very large

    - Thread pointer, a place for subroutine data such as large numbers of parameters

Dual usage as an auto increment/decrement pointer

    - Frame pointer, a place for thread specific data

Dual usage as an auto increment/decrement pointer

    - Global memory:

Normally, global memory starts at address zero and enumerates all read/write memory locations.  Memory mapped IO at high memory.

  - Growth paths:

    - Starting very basic:

- The simplest processor is a small state machine.  States are given names by enumeration and each state has the expressions for computing and going to the next state.  State machines without an ALU will be characterized as “small”.  There is no instruction set.

- No instruction memory, only the ALU and registers. This is the realm of micro-code and medium size state machines.  E.g., there is no instruction set other than the micro-code signals.  Without a formal ALU it can’t be considered general purpose.  If a register file (with say two read ports and one write port) is used instead of an accumulator one has a register oriented computer instead of a memory oriented computer.

- At some point the need for subroutines and organized processing, with intermediate data, brings in an accumulator with data and control stack RAMs.

- At some further point, need for code compactness leads to an instruction decoder and instruction memory.

- At some further point the need for subroutine working space leads to adding a Frame pointer.  It could point to within the control stack.  Design complexity favors a distinct memory for frame data and/or global data.

- Adding a Thread pointer allows multi-threading with tread changes requiring multiple sets of Thread, Frame, Return stack and Data stack pointers along with the other state such as the PC, Accumulator and Status register.

    - Starting with no Thread or Frame pointers: All addressing is global

For some embedded applications, a global data pool is possible.  An example is when there is no subroutine recursion, so each subroutine can have a section of global memory dedicated for its parameters, results and local data.

    - Starting with no co-processors

Typical.

    - Starting with no Return stack

For some embedded applications, only one or two levels of subroutine calls occur.  It is then efficient to allocate D flip-flops for the return address as opposed to a return stack RAM and its pointer register.

    - Starting with no Global data memory, use data stack only

Rare, insignificant.

    - Starting with no Data stack or Return stack

Changes the basic nature of the instruction set.  All operations are to/from data memory (e.g. an accumulator design).  Can be useful for comparative analysis.

- Compilers and optimization

  - For embedded applications with small amounts of code, global variables are simpler than stack frame variables.  Compiler should be bright enough to decide between the various forms.  C2stack supports thread space separate from global space, so this optimization can generate three forms: global or thread, level or frame and data stack allocation of variables and values.

  - A sufficiently bright compiler should be able to inline subroutine calls on a case by case basis (e.g. if the parameters of the call allow simplification of the subroutine code and if there are few calls of this sort – e.g. do a cost benefit analysis).  Also in the bright category is choosing between using a stack frame and not using a stack frame (and thereby avoiding the stack frame overhead).  For the C2stack architecture the stack frame overhead consists of memory cycles to save/restore the previous stack frame and memory cycles to copy the parameters from the data stack to the return stack.

- Instruction set

  - Registers

  - Addressing modes

  - Instruction formats

  - Instruction list

    - Add

E.g. has both a data stack (DS) and a return stack (RS)

Provides a frame pointer (FP) for stack frames and supports a data stack.

A register oriented instruction set (ROIS) is considered a design variation wherein there are multiple accumulators.  A ROIS can be overlaid by rearranging bits.  A better way to do ROIS is to use 16-bit instruction words for a ROIS and include stack support via two of the registers in the register set.

Which can be a temporary on the return stack

E.g., Harvard architecture: data memory separate from program memory and may have different word size.

LUT and block RAM have short access times compared to external DRAM.

It might be advantageous to make instruction alignment to half-words rather than bytes at cost of NOP fillers

Multi-port register files are available in FPGAs and simple register oriented instruction sets (ROIS) are easy to implement.  A compiler for ROIS must be more complicated to do efficient register allocation.  Instruction and data memory are usually shared and implemented in Block RAM.

Historically the choice was to add a single, all purpose, read/write memory.  Instruction fetch and data load/store occurred to the same memory.  Many instructions took several memory cycles to complete. Opinion: The history of the single read/write main memory computer frames the evolution of main stream computer architecture.

With FPGAs and their variety of memory resources, dedicated memories for each purpose make sense.  And most instructions can complete in a single clock cycle.  Herein the major memory types are: data stack, return stack, data memory and program memory.  Add register files if co-processors are warranted.

Why four memory sections when one will do?  It’s a matter of total memory bandwidth: Here, a given instruction can interact with the four memories simultaneously.  As opposed to current architectures, famous for the “von Neumann bottleneck”, where instruction and data caches are needed to improve the single path to memory.  And overcome the slow speed and long access time of that massive memory.