Hello Jim,

I have progressed a little bit with the code. Re-wrote what I call the "Address Generator" block, with assocciated package files. You can view the synthesis report from ISE 14.7 for a SP LX45 part with -1 speed grade:

=========================================================================

Synthesizing Unit <hay4stk_addr_gen>.

Related source file is "C:\Users\Leo\Google Drive\ExperimentalProcessor_hay4stk\hay4stk_addr_gen.vhd".

Found 8x16-bit dual-port RAM <Mram_Addr_RAM> for signal <Addr_RAM>.

Found 16-bit register for signal <EAR_Reg>.

Found 16-bit adder for signal <addr_alu_result> created at line 62.

Found 3-bit 4-to-1 multiplexer for signal <Addr_RAM_wr_addr> created at line 80.

Found 3-bit 4-to-1 multiplexer for signal <Addr_RAM_rd_addr> created at line 86.

Found 16-bit 4-to-1 multiplexer for signal <Addr_out> created at line 93.

Summary:

inferred   1 RAM(s).

inferred   1 Adder/Subtractor(s).

inferred  16 D-type flip-flop(s).

inferred   5 Multiplexer(s).

Unit <hay4stk_addr_gen> synthesized.

=========================================================================

HDL Synthesis Report

Macro Statistics

# RAMs                                                 : 1

8x16-bit dual-port RAM                                : 1

# Adders/Subtractors                                   : 1

16-bit adder                                          : 1

# Registers                                            : 1

16-bit register                                       : 1

# Multiplexers                                         : 5

16-bit 2-to-1 multiplexer                             : 2

16-bit 4-to-1 multiplexer                             : 1

3-bit 4-to-1 multiplexer                              : 2

=========================================================================

The reported theoretical frequency in synthesis is 265.904MHz.

I'm attaching the files for you to review the code.

Best Regards,

Leonardo.