The 12X Computer Family

Hybrid microprocessor family with two or more Addressable Stacks

James Brakefield

Contents

Introduction

Instruction Encoding

Op-codes

Speed-ups

Implementation Decisions

Architecture Unification

Compilers and Optimization

References

Glossary

The Extended nX_uP Family

Debug and test sequence

Introduction:

The characterization of this architecture is two addressable stacks and Forth/Accumulator oriented instruction set.  As such it encompasses both accumulator style computers and stack style computers.  The instruction set is geared for 12, 24 or 48-bit word size.  The Forth stack operators are a subset of the instruction set.  There are one or two accumulators (Data Top-Of-Stack and an optional Floating-point TOS).  The floating-point and locals stacks may or may not be separate from the data and return stacks.

The addressing modes are program counter relative, top-of-stack relative, locals (e.g. frame) pointer relative and absolute.  Each instruction has the option to pop/push the associated accumulator.  That is, if the option bit is not invoked, the instruction has the semantics of an accumulator instruction.

The architecture has a combined stack and accumulator flavor.  Due to the multiple stacks and the frame pointer, compactly encoded offset locations give high coding density on the same order as the best load/store (RISC) architectures.

Intent is to establish an expandable and efficient instruction set which supports stack oriented computation as well as traditional “C” language memory organization.  Due to the presence of multiple accumulators, superscalar or multiple instruction dispatch is possible allowing increased performance compared to typical stack architectures.

Another goal is fast and efficient subroutine call/return.  A call pushes the address of the next instruction onto the return stack and branches to the subroutine.  Within the subroutine most instructions have an R-bit, which if set pops the top of the return stack into the program counter.  Parameters for the subroutine are on the (data) stack.  A frame pointer is provided allowing simple access to the parameters.  Within the subroutine a special instruction can be used to push the previous frame pointer and update the frame pointer to the beginning of the parameters.  Likewise a special instruction exists to restore the previous frame pointer, adjust the stack pointer and optionally do a return.

The instruction sets for 12, 24 and 48-bit word sizes vary from each other in that the default operations are with the 12, 24 or 48-bit word size; with load and store instructions for the other sizes.  Also, the instruction sets may vary in the operations provided and in the size of the instruction set.  Also, a variety of possible instruction encodings are shown.

The intended FPGA implementation is with a single LUT RAM for the stacks and Block RAM for “main memory”.  Thus, an instruction can be read from Block RAM memory, the operation performed and the results clocked into LUT RAM in a single clock cycle.  Block RAM data access (read or write) requires an additional main memory cycle (unless Block RAM is dual ported with separate instruction read and data read/write ports and that the data memory cycle is pipelined).

12X_uP is a research computer architecture employing a hybrid mixture of accumulator, stack and load/store features.  By restricting the instruction set usage (e.g. using a subset) one can effectively have an accumulator machine, a stack machine or a C machine.

Data movement between the stacks and main memory is via load/store instructions.

The same FPGA resources will implement a RISC instruction set.  Thus all three instruction set types are available: Stack and accumulator via the first instruction set and RISC via a second instruction set (the first using a 12 or 24-bit instruction and RISC, using a 24-bit instruction).  All share the same load/store approach to main memory.

An overall programmer’s model consists of:  The program counter (PC), the instruction register (IR) and a condition code register (CCR).  There is an overall memory map which includes program memory, data memory and possibly memory mapped input/output (IO) ports.  There are one or more operating units (OU) consisting of the accumulators, arithmetic-logic units and their associated stacks.  Each stack consists of addressable memory, a stack pointer and an adder for adding an offset to the stack pointer when referencing deeper into the stack.

For comma coded instruction length: The instruction register (IR) is cleared by after instruction execution.  An instruction suffix chunk shifts the current IR left by eleven and inserts the twelve bit suffix chunk at the LSB end.

PC	Program Counter

IR	Instruction Register, instruction suffix chunks accumulate here

CCR	Condition Code register, has operational mode bits as well as carry and overflow bits

Traditionally not used in Forth mode, used here with constraints.

TOS	Accumulator and top of the data stack.  Located in either LUT RAM or in DFF.

SP	Pointer to data stack, e.g., in Forth the stack pointer

RP	Return stack pointer

FP	Frame or locals pointer

FLP	Floating-point stack pointer

FR	Floating-point accumulator, top of floating point stack.  Located either in LUT RAM or DFF.

A given implementation may chose an 8, 9, 12, 16, 18, 24, 32, 36, 48 or 64-bit word size with 4X, 8X, 9X or 12X addressability.

When implementing data widths of 8X within a 12X word size, addressing can be either 12X or 8X.  With 12X addressing: 8 and 16-bit data have 33% memory waste, 32 and 64-bit have 11% memory waste.   With 8X addressing: 12-bit data has 25% waste, 24 and 48-bit data 0% memory waste.

- One or more accumulators and addressable stacks

Each accumulator is associated with a particular stack.  The instruction Displacement Field (DF: MMM along with NNN…) provides an offset for any of the stack pointers for the second operand.  The purpose of multiple accumulators is to allow implementation of simultaneous issue of multiple instructions in a stack environment.  I.e., a sequence of instructions using distinct accumulators can issue simultaneously.  The motivation for the instruction displacement field is to minimize the stack manipulations used to get operands into suitable positions for calculations.

This leads to an associated and unique computer architecture perspective: Different categories of instruction architectures, in general, are different ways of abbreviating memory locations.  The rational is to save space in the instruction stream.  Full addresses can be, say, 48 bits in size.  An abbreviated memory value can be held on just a few bits.  Zero bits if it is implied.  A three address instruction could require 144 bits for the addresses.  With abbreviation, one operand addresses and a result can be encoded in as few as five bits.

A single accumulator abbreviates the location it was loaded from.  A register in a register file abbreviates the location it was loaded from or will be written to.  The stack machine abbreviates both the top of the stack and the nearby stack values.  So much so, that register numbers are unnecessary.  Here, in addition to normal stack operations, stack offsets are abbreviations for values deeper on one of the stacks.  And frame or thread offsets are abbreviations for the locations near the frame or thread pointer.

12X_uP has one or two accumulators which represent the top of the data and floating-point stacks.  Offsets to the program counter, top of stack accumulators or to thread or frame registers are size encoded to minimize instruction lengths.  Compared to load-store instruction architectures, AKA RISC architecture, with 16 to 32 general purpose registers, here there are one or two accumulators and two to four memory areas accessible with short displacements offering 16 or more fast access “registers”.  It is both more compact than load/store encoding and if more abbreviations are needed they can be arranged by use of additional prefix chunks (e.g. larger displacement).

  - 24-bit instruction encoding, accumulator size specified

Complete instruction	XXXX XXrP	SZRM sNNN NNNN NNNN

X:	Op-code	SZ: Data size 	M: data stack pointer/locals pointer

r: 	Replace op	R:   Return	P:   if set: if r=0 do push, if r=1 do pop

s: 	Offset sign	N:  Offset or immediate or op-code expansion

  - 12-bit instruction encoding: Two 12-bit instructions or one 24-bit instruction

24-bit instruction		XXXX XXX0 EPRM sNNN NNNN NNNN

Two 12-bit instructions	XXXX XPr1 MNNN XXXX XPrR MNNN

X:	Op-code	M: data stack pointer/frame pointer

E:	replace op	R:   Return	P: if set do push if r=0, do pop if r=1

s: 	Offset sign	N:  Offset or immediate or op-code expansion

To fit two instructions, the return bit was sacrificed in the first of the instruction pair.

  - 12-bit chunk style (comma coded) instruction encoding

Operate								1NNN XXXX XPEM

First Prefix					0XXM RSZN NNNN 1NNN XXXX XPEM

Adt’l Prefixes  	0NNN NNNN NNNN 0XXM RSZN NNNN 1NNN XXXX XPEM

E:	Replace operation	X:	Op-code 	SZ:	Size

R:	Return			P:	If set: if r=0 do push, if r=1 do pop

M:	Data stack pointer/locals pointer	N:	Offset/op-code expansion

  - RISC instruction encoding (24-bit instructions)

Register to register				XXXX XXXXX DDDDD RRRRR SSSSS

Immediate1				XXXX XXXXX DDDDD RRRRR sNNNN

Immediate2				XXXX XXXXX DDDDD sNNNN NNNNN

Immediate3				XXXX XXXXX sNNNN NNNNN NNNNN

X:    Op-code		D:    Result register

R:    1st operand register	S:    2nd operand register

N:    immediate operand	s:     N’s sign (2’s complement)

  - 6-bit instruction encoding

There are only 64 instructions which include stack operations, prefix codes, immediates, and stack or memory references.  By pairing stack operations with stack references many of the 12-bit chunk instructions are encoded by such pairs allowing the pair to dispatch together.

The concept is that a MNNN code creates a stack reference which is pushed onto the data stack.  A following stack operation distinguishes between stack values and stack references.  A further generalization is to  support memory references.

A chunk per clock implementation is straight forward.  Dispatching multiple chunks per clock is likely to require greater decode logic and uses the same data path.

The low 64 memory locations are considered as the chunk values for execution purposes.

Immediates: 6, 12, 24 & 48 bit immediates; the values 0, +1 and -1

MNNN codes: 16 MNNN codes

Stack operations: remaining 41 codes

CALL	SP^->PC, PC+1->vRP

RTN	RP^->PC

BRcc	if cc then SP^+PC->PC

Bcc 	if cc then SP^->PC

ADD	(SP^)+(SP)->SP		normal stack-op if (SP) & (SP+1) are values

- 12X_uP Op-codes			XXXX XRPE MNNN

Traps (user defined op-codes & interrupts)	0000 0NNN NNNN		X=0

N expansion (adds two X bits)?			0000 1XXN NNNN		X=1

N expansion (N10, N17, N24)?			0000 1NNN NNNN		X=1

Relatives (6-bit signed displacement)

If s=1 and NNNNN = 0 then absolute address is at PC+1?

Call, Branch (BR=Call+Rtn)			0001 0RsN NNNN		X=2

Branch (BRZ, BRNZ) 				0001 1vsN NNNN		X=3

Load/Store					001X XRPX MNNN		X=4..7

Load/store indirect								X=4

Load/store indirect with post increment						X=5

Load/store indirect with pre-decrement						X=6

Load/store with address offset							X=7

Immediates (5-bit signed immediate)		01XX XRPs NNNN		X=8..14

If s=1 and NNNN = 0 then N is at PC+1?

Load immediate

Add

And

Or

Xor

Shift

Compare

Forth Ops					0111 10NN NNNN		X=15

Lut

Case

Move Locals

Copy Locals

Pop Locals

T>R, R>T, PC>R, PC>T

T>PC, Call TOS (exec), CASE2, PACK, UNPACK

CCR>T, T>CCR

Conditional Skips

TOS tests

R@, D@, !, D!, +!

MOD, /MOD, */MOD, */

UM*, UM/MOD, MAX, MIN

ABS, NEG, D+, DNEG

<, =, >, 0<, 0=, 0>, D<, U<

SWAP, `, DO

HALT

Conditional functions				0111 11XX vNNN		X=15

Conditional branch

Conditional skip

Generate 0/1

Generate & return

Conditional call

Conditional return

ALU operations					1XXX XRPE MNNN X=16..31

If R=1 pop return stack to PC (after execution)

If E=1 swap operands, result goes to stack location (MNNN)

If E=0 and P=0: result goes to current top of data stack

If E=0 and P=1: result pushed onto data stack

If E=1 and P=0: no change to data stack pointer

If E=1 and P=1: top of data stack dropped

MNNN=0..6: SP+N, 7: PC+1 imm, 15:RP, 8..14:L-N (choices for 2nd operand)

Pick/Pock

Add

Subtract

Multiply

Divide

And

Or

Xor

Compare/Fltg-pt Cmp

INC/DEC/CLR/SET

Fltg-pt Add

Fltg-pt Sub

Fltg-pt Mul

Fltg-pt Div

IN/OUT

Push/Pop frame ptr/DropN/INITstk

  - 12X_uP Op-code descriptions

The (IA) instruction address is PC+sNNNNN, unless s=1 and NNNNN=0, then the IA is the memory value at PC+1.  sNNNNN is in two’s complement.

The (IM) value is sNNNN, unless s=1 and NNNN=0, then IM is the memory value at PC+1.  sNNNN is in two’s complement.

MNNN is a location in LUT RAM. If MNNN=0..6, then SP+N, if =7: PC+1 has the value, e.g. an immediate value, if =15:RP, if =8..14:FP-N.  These are the choices for 2nd operand.

The (OP) value is the LUT RAM value at MNNN.

TOS is the top value on the data stack, e.g. LUT RAM value at SP.

SOS is the second value on the data stack, e.g. LUT RAM value at SP+1.

The CCR bits include carry, signed overflow, sign bit (MSB), least significant bit (LSB).  The zero condition may be generated from the MSB, LSB, and the remaining ten result bits.  E.g. to eliminate a logic delay in the critical path, the zero status of the result bits may be a logic combination of several “hidden” CCR bits.

The E bit affects non-symmetrical operations such as subtract and divide.  If the E bit is set for a subtraction, TOS is subtracted from OP rather than OP from TOS.  Some non-symmetrical operations, such as compare, do not use the E bit.

The following instructions are in numerical order with indentation of instructions with same first five bits (e.g. both CALL and BR have an op-code of 2 with the following instruction P-bit being zero for CALL and one for BR.

      - R bit

For most instructions, after the rest of the instruction finishes, if the R bit is set, the return address on the return stack is popped and becomes the address of the next instruction.

      - E bit

For most instructions that generate a result using TOS and an operand from an offset location on the stack, if the E bit is set, the result replaces the operand from the stack offset location.

      - P bit

For those instructions that use an E-bit, if the P-bit is not set, TOS either gets the result or remains intact (accumulator mode).  If the P-bit and the E-bit is set, then TOS is dropped from the stack.  If the P bit is set and the E-bit is clear, then the result is pushed onto the stack.

    - TRAP

Push PC+1 onto the return stack and set PC to the contents of location NNNNNNN.

    - EXTND

NNNNNNN is to be used by the following instruction to augment its X’s and N’s.

    - CALL

Push PC+1 onto the return stack and set PC to 4*(PC/4+sNNNNN).  First subroutine instruction must be on a quad word boundary so that a greater relative range is available at the expense of some unused instruction memory.

      - BR

Branch to IA (e.g. set PC to IA).

    - BRZ

Branch to IA if the CCR Z bit is set, otherwise continue at PC+1

      - BRNZ

Branch to IA if the CCR Z bit is clear, otherwise continue at PC+1

    - LD/LDoff /LDpp/LDnn (op-codes of 4, 5, 6 and 7)

A set of four instructions that load a value from main memory.  OP has the main memory address.  For OPpp OP is incremented after the load.  For OPnn OP is decremented before the load.  For OPoff TOS is added to OP to form the address.  OPpp and OPnn modify OP.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.

      - ST/SToff /STpp/STnn (op-codes of 4, 5, 6 and 7)

A set of four instructions that store a value into main memory.  OP has the main memory address.  For OPpp OP is incremented after the store.  For OPnn OP is decremented before the store.  For OPoff SOS is added to OP to form the address.  OPpp and OPnn modify OP.  Depending on the P bit, TOS is either popped or left intact.  The return bit, “R”, is supported.

    - LDI

Load or push IM.  Depending on the P bit, the IM value either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.

    - ADDI

Add IM to TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.

    - ANDI

And IM with TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR, except carry and overflow bits, is updated.

    - ORI

Or IM with TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR, except carry and overflow bits, is updated.

    - XORI

Xor IM with TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR, except carry and overflow bits, is updated.

    - CMPI

Compare IM with TOS.  Depending on the P bit, the TOS can be popped.  The return bit, “R”, is supported. The CCR, except for the overflow bit, is updated.

    - SHFT

Shift TOS by the shift count IM.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  As a zero shift count is unneeded and a logical right shift of 11 can be accomplished by the 1CCC instruction, arithmetic left and right shifts of 1 thru 11 take 22 codes and logical right shift of 1 thru 10 take ten codes; the five bit immediate suffices to encode all useful logical and arithmetic shifts.  The return bit, “R”, is supported.  The CCR, except for the overflow and carry bits, is updated.

    - MISC

Variety of stack operations such as swap, exec, return …

        - SWAPmnnn

Exchange TOS and SOS.

        - Execute

Execute subroutine at address L(LN).

        - TBD

      - BRCCC

Branch to address at PC+1 if condition XXX is true.

        - LOOPCCCvnnn

Branch to L(RP) if condition is true.

        - SKCCCvnnn

Skip following instruction of condition is true.

        - 1CCCvnnn

Push 1 if condition is true, otherwise push 0.

        - 1RCCCvnnn

Push 1 if condition is true, otherwise push 0.  Do a return.

The complete set of basic conditions is: EV/OD: even/odd, Z/NZ: zero/non-zero, CC/CS: carry clear/set, OV/NOV: overflow/no overflow, LO/HS: low/high-same, M/P: minus/plus, GT/LE: greater-than/less or equal, HI/LS: high/low-same.

      - MISCnnnn

Nice to have op-codes, currently no place to put them.

        - Case Branch

N is max TOS value.  N branch addresses follow the instruction.  If TOS > N the CCR overflow bit set and execution continues following the N branch instructions.

        - Case Call

N is max TOS value.  N subroutine addresses follow the instruction.  If TOS > N the CCR overflow bit set and execution continues following the N branch instructions.  If TOS <= N, the TOSth subroutine is executed and on return execution continues following the N subroutine addresses.

        - Extract

Combination right shift and mask out of low bits.  Two results.

        - Pack

Combination left shift and ORing in of the low bits.

        - MULI

Multiply immediate.

        - LUT

TOS is selector of a portion from OPND.

        - RTNCCCvnnn

Do a return if condition is true.

        - CALLCCCvnnn

Call subroutine with address at PC+1 if condition is true.

    - PICK

Load or push OP.  Depending on the P bit, OP either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.

      - POCK

Store TOS at LUT RAM location MNNN.  Depending on the P bit, TOS is either popped or left intact.  The return bit, “R”, is supported.

    - IN

Read input port NNNN.  Depending on the P bit, the value either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.

      - OUT

Post TOS to output port NNNN.  Depending on the P bit, TOS is either popped or left intact.  The return bit, “R”, is supported.

    - PUSHFP

Push the FP register onto the return stack.  Change FP to SP+NNNN.

      - POPFP

Change SP to FP-NNNN.  Pop the FP register from the return stack.  The return bit, “R”, is supported.

      - DROPn

Add NNNN to SP.  Equivalent to drop NNNN times.  The return bit, “R”, is supported.

      - INITstk

Push the following NNNN words onto the stack.  The return bit, “R”, is supported.

    - INC

Increment the value at MNNN.  “R”, is supported. The CCR, except for the carry and the overflow bits, is updated.

      - DEC

Decrement the value at MNNN.  “R”, is supported. The CCR, except for the carry and the overflow bits, is updated.

      - CLR

Clear the value at MNNN.  The return bit, “R”, is supported.

      - SET

Set the value at MNNN to all 1’s (e.g. to -1).  The return bit, “R”, is supported.

    - ADD

Add OP to TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.

    - SUB

Subtract OP from TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.

    - MUL

Signed multiply of OP and TOS.  Depending on the P bit, the 12-bit result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.  The CCR overflow bit is set if the product exceeds the truncated 12-bit result.

    - DIV

Signed divide of OP into TOS.  Depending on the P bit, the 12-bit result either replaces TOS or is pushed onto the data stack.  Results accessed via input port after a delay.  The CCR is updated as part of input port read.  If divisor is zero, the overflow bit is set and the result is TOS.

    - AND

And OP with TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR, except carry and overflow bits, is updated.

    - OR

Or OP with TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR, except carry and overflow bits, is updated.

    - XOR

Xor OP with TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR, except carry and overflow bits, is updated.

    - CMP

Compare OP with TOS.  Depending on the P bit, the TOS can be popped.  The return bit, “R”, is supported. The CCR, except for the overflow bit, is updated.

      - FCMP

Floating-point compare OP with TOS.  Depending on the P bit, the TOS can be popped.  The return bit, “R”, is supported. The CCR, except for the overflow bit, is updated.

    - FADD

Floating-point add OP to TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.

    - FSUB

Floating-point subtract OP from TOS.  Depending on the P bit, the result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.

    - FMUL

Floating-point multiply OP and TOS.  Depending on the P bit, the 12-bit result either replaces TOS or is pushed onto the data stack.  The return bit, “R”, is supported.  The CCR is updated.  The CCR overflow bit is set if the signed product exceeds the truncated 12-bit result.

    - FDIV

Floating-point divide OP into TOS.  Depending on the P bit, the 12-bit result either replaces TOS or is pushed onto the data stack.  Results accessed via input port after a delay.  The CCR is updated as part of input port read.  If divisor is zero, the overflow bit is set and the result is TOS.

Instruction set issues (9/23/2015):

Not enough code space: The XXXX XRPE MNNN format only has space for 32 opcodes

Can be solved by 16 or 24-bit instructions

Pruning based on experience?

N = 7 is escape to value at PC+1, e.g. use immediate value following the instruction

Use shorter field for immediate instructions

Use N expansion instructions to also add X bits

Small branch range for relative branches and calls (+31, -31)

Can be solved by 16 or 24-bit instructions

Ambiguity for word list representation (e.g. two representations needed):

Words as addresses

Words as machine code

Standard CFA & PFA format: CFA has machine code, PFA has word list?

Too many Forth primitives: e.g. essentially two distinct instruction sets

Efficient implementation of common operations

SWAP: Single LUT RAM write per clock requires DFF TOS for 1 clock realization

ROT, ROLL: No single instruction

Block moves for data between return stack and data stack (for parameters mostly)

Forth implementation issues

Hash table for all constants of any type

Standard Forth dictionary search

Quotations: word string in dict. parameter field

Objects: type, size, value, hash link if constant

Locals & loop data: either stack supported

Use of skip instructions and CCR register

ANSI Forths: SwiftForth, VFX, iForth, Gforth, …

Beg/borrow from other Forths:

8th:		JSON

Gforth:		Locals

Strongforth:	Type checking

FMITE (LOTD):	Higher-order functions

Forth types (open ended)

A design alternative is to have memory and the stacks encode data types.  This is as opposed to only the compiler having access to data types.  As commodity memory width is a multiple of 16 bits, one can easily provide four type bits per each 12-bit word.

Boolean			size inferred

Int (three or four sizes) 		size inferred

Uint (three or four sizes) 	size inferred

UTF-12 string			size included

Float (two or three sizes) 	size inferred

Complex (two or three sizes) 	size inferred

Array				size included

Character string			size included

Word list			size included

Code segment (executable)	size included

Dictionary entry		size inferred

Vocabulary array		size included

Objects: any of the above with:

Type field, size field if not inferred, constancy link (all constants in hash table) & datum

Hash table entry for constants:	Has object pointer, is hash of type plus value

Vocabulary entry:	Has PFA link, CFA link, Name link and immediacy

On a small target processor, host processor can maintain most or all of the metadata (type codes).

- Speedups

  - Multi-stage pipelining, of course.

  - Have the program memory 48-bits wide.  Process the first instruction and its prefixes simultaneously.  With 12-bit chunks one can have one to four instructions per word and multiple dispatch.

  - Implement parallel ALUs, one each for each addressable stack (e.g. simplify multiple dispatch for superscalar implementation).

  - Use a larger chunk size so there are fewer prefixes.

  - Have the compiler arrange multiple instructions per word so multiple dispatch always on (e.g. NOPs inserted as necessary by compiler and entire 48-bits of instructions always dispatched).

  - Use NOP space for immediate chunk data that is used later.

  - Add co-processor(s) and support long instructions with sections for each co-processor (e.g. another way to do multiple dispatch).

  - Implement multiple threads via pipeline and pipeline barrel.  High speed without pipeline interlocks.

  - Waste space on the data stack: have a unique stack location for each result.  Use a valid bit per stack location that permits instruction dispatching prior to operands being computed.

- Implementation decisions

  - Block RAM versus LUT RAM for the stacks

The LUT RAM approach keeps the stacks in LUT RAM and the program and data in block RAM.   Block RAM memory bandwidth is limited to instruction reads and data memory read/write.  LUT RAMs allow asynchronous operation, meaning that a single phase instruction cycle is possible: Read instruction, decode instruction and forward stack read addresses to the LUT RAM, forward LUT RAM read data to ALU, write ALU result to LUT RAM or data RAM.  Instructions with data RAM reads or writes require an additional phase.

The alternate implementation is to use a single dual port block RAM for all RAM usage: program memory, stack memory and data memory.  The processing cycle consists of two phases:

- Writing the result of the previous instruction to memory while also reading the next instruction.

- Reading the two operands and computing the result.

Phase A includes instruction decode and address generation.  Not all instructions require phase B, in particular the conditional branches.  Some instructions will require a phase C, such as doing a subroutine return as part of an ALU instruction.

As block RAM can cycle faster than needed for the two phases, it is possible to do two or more instructions in about the same amount of time by pipelining the address generation and the ALU computation.

This block RAM approach is similar to the historical three address computer.  Due to the dual ports and performance of FPGA block RAM, this is simple to implement and gives high performance.

  - Go for a minimal design

There are arguments for going for either the smallest possible design or going for the highest performance design.  The figure of merit is DMIPS per LUT.  Or equivalently, DMIPS per unit silicon area, which factors in the area taken by multiplier/DSP units and block RAMs.  Of course LUTs from different FPGA families are not necessarily equivalent.  The ultimate figure of merit is DMIPS per dollar using the prorated chip area.

The DMIPS per dollar and DMIPS per unit silicon area metrics also allow comparison with FPGAs with hard core processors such as the PowerPC, ARM Cortex A9 or Cortex A3, or with non-FPGA embedded processors.

Naturally some applications call for very little data RAM or instruction memory.  Other applications call for a full 32-bit processor with floating-point, caches and/or memory management.

Interestingly enough there are examples of high performance FPGA soft core processors at the minimal level, at the single processor pipelined level and at the multi-processor/barrel approach.  Thus it is possible to get a high figure of merit for any of the three design approaches.

Minimal treatment of the stack RAM is to not support spilling or refilling.  A LUT RAM in 6LUT FPGA technology can provide 64 locations for the combined return and data stacks.

Given that 12X_12uP is a new architecture, a simple or minimal implementation is the lowest risk and fastest to implement choice.  A minimal implementation also offers the greatest insight into LUT count versus instruction set complexity or cycle time.

  - Use a single pipeline stage

This mode of operation uses LUT RAM with asynchronous read for fetching operands.  Thus in one clock cycle an instruction is read, is decoded, the operands are read, the operation performed and the results along with the updated PC registered at the beginning of the next clock cycle.

A pipeline design with additional stages will result in a higher clock speed and greater performance.  However, there is greater complexity which is best left to when after the single pipeline stage design is up and running.  A full pipelined “barrel processor” design, with one processor per pipeline stage, is a straight forward way to get maximum clock rate.

  - Keep the accumulators in RAM (LUT or Block)

Keeping the accumulators in RAM eliminates the complexity of updating discrete accumulator registers and pushing them to RAM.

  - Use block RAM for instruction memory

The smallest block RAM allows for several hundred instructions, a reasonable number.  Reading instructions from a synchronous RAM is compatible with a single pipe stage implementation.

  - Use dual port or quad port LUT RAM for the stacks and accumulators

A Xilinx series 6 or 7 FPGAs support quad port LUT RAMs (three asynchronous read ports and one asynchronous read/synchronous write port).  For the 12X_uP instruction set one can use all four ports: one port for pulling the return address from the return stack, a read port for one operand of an instruction, a read port for the second operand and a write port for the result data.

Some Altera FPGAs support a simple dual block RAM “MLABs” (with asynchronous read and a separate synchronous write).  By operating three MLABs in parallel one can obtain three asynchronous read ports and a single synchronous write port (always writing the same location and data to the three MLABs simultaneously).

Micro Semiconductor, formerly Actel, and Lattice Semiconductor FPGAs provide similar capabilities as the Xilinx and Altera FPGAs (block RAM and LUT or MLAB RAM).

  - Use discrete pointer registers

The pointer registers are: Program counter, Data stack pointer, Locals pointer and Return stack pointer.  These are individual D flip-flop registers.  The alternative, to introduce another LUT RAM to hold these registers, is ruled out by the instruction set modifying more than one pointer per instruction.

  - Use block RAM for all operands and results

With a two stage pipeline and a full dual port RAM, the sequence is: 1st pipeline stage reads the next instruction and writes the previous result.  2nd pipeline stage reads the two operands and computes the result.  Address calculation fitted into first stage.  ALU operation fitted into second stage.  Delay exceptions or resource constraints force additional pipeline stages.  A conditional branch can be fitted into a single pipeline stage if it uses CCR results from completed instructions.

  - Instruction memory data width

Three instruction formats are defined: one for six to nine bit instruction memory width, one for twelve to eighteen bit width and one for 24 to 36-bit width.  Using block RAM for instruction memory, any width is readily supported.  Additional block RAM width is useful for faster instruction issue (prefix chunks can often be processed in parallel with the operation chunk).

Even wider instruction block RAM width is useful for dual instruction issue, e.g. superscalar, designs.

  - Using instruction memory for data memory

Given that instruction memory is block RAM and therefore has synchronous reads, and supports dual port operation (first port for instruction reads, second port used for data RAM read/write), the single stage pipe design does not allow a block RAM data read to occur in the same cycle as the instruction read.  However a block RAM data write can occur at the end of the cycle.

Given that it takes one cycle to post the read address and the next cycle to read from the block RAM, a variety of arrangements can be made for data read: Use two instructions, one to post the read address and the next instruction to process the data.  Or use a memory pointer register (in IO space) which auto increments after each data read and fetches data at the next location.

  - Interrupts

Interrupts shall push sufficient information (program counter, condition code register, etc.) to transparently resume the program at the point of interrupt assuming no stack overflows.

  - Condition code register (CCR)

The conditional branch instructions specify an accumulator and a condition code.  There are arguments for and against condition code registers.  Here, it was decided to stay with a full condition code register.  Thus, the CCR contains carry, signed overflow, zero and sign for each accumulator.  As the CCR with status bits for each accumulator takes more than 16-bits, there is room for a least significant bit, a parity bit and a default stack direction bit.  Instead of parity bit, use a default bit to indicate stack pointer not in stack mode (must use P bit to force pop/push).

The CCR also contains defaults such as the default size field for instructions that do not specify a data size (8, 12, 16, 24, 32, 48 or 64 bits).  Additional CCR bits for floating point are probably necessary.  Interrupt enable and Interrupted bits also present.

  - Main ALU

Operations: Add, Subtract, AND, OR, XOR, Shifts, Multiply, Insert field, Extract field

First operand: Accumulator

Second operand: LUT RAM read port(s), block RAM read port(s), EA or immediate, zero, CCR, IO input port

  - Effective address ALU

An accumulator is always one operand and an effective address or data at the effective address is usually the second operand.  The effective address is a location relative to one of the stacks, relative to the frame or thread pointer, relative to the PC or an absolute memory address.

A two cycle instruction is provided in the instruction set where the first cycle accesses and possibly modifies an address at the effective address and the second cycle uses the address at the effective address to access the operand to complete the instruction.

  - Implementation resources (Spartan-6)	          LUTs:  min sz	full sz	Other

Two columns of LUT counts are shown: 2nd column is for full block RAM address and 1st column is number for quad port LUT RAM address.  E.g. the smaller number is feasible.

A variant with LUT RAM for instruction memory is feasible: however, it both increases the LUT count and decreases the available program size drastically.

    - PC address ALU (PC_ctrl)					10	16

Inputs: PC+1, Op2, Return Address; Operations: select one

    - AFTP mux (AFTP_ctrl)						10	16

Inputs: ALU output, FP, TP, PC+1; Operations: Select one

    - Accumulator address mux (SPn_ctrl)				  6	16

Inputs: SPn; Operations: Select one

    - Source EA accumulator address mux (SPm_ctrl)		  6	16

Inputs: STKn; Operations: Select one

    - Source EA PC/FP/TP address mux (FTP_ctrl)		12	16

Inputs: PC, FP, TP, zero; Operations: Select one

    - Source EA ALU (EA_alu_ctrl)					12	16

Inputs: N, SPm, FTP; Operations: SPm, SPm+/-N, FTP+/-N; generates EA

    - Destination EA ALU	(EAW_ctrl)				  6	16

Inputs: SPn, EA; Operations: SPn, SPn+1, SPn-1, EA

    - Quad port RAM							64	64

Inputs: SP0, SPn, EA, WEA, DID

    - IO input mux (IOin_ctrl) 					16	16

Inputs: 32-bit Input port & CCR; Operations: Select one: CCR, CCR>>16, IOport, IOport>>16

    - ALU input mux	(Op2_ctrl)					16	16

Inputs: BRAMDO, QPMDO, EA, IOinMX; Operations: Select one; generates Op2

    - Boolean unit (Bool_ctrl) 					16	16

Inputs: QPMDOB, Op2; Operations: a, not a, b, not b, zero, a&b, a|b, a^b

    - ALU (ALU_ctrl)		 					16	16

Inputs: bool, op2; Operations: bool + op2, bool + not op2

    - QPM write data mux (DID_ctrl)				16	16

Inputs: AFTP mux, DSP48, DSP48>>16, DSP48|(DSP48>>16); Operations: Select one

    - Stack pointer “n” input ALU (SPn_ctrl)			  6*4	16*4

Inputs: Op2, EA, SPn; Operations: Op2, EA, SPn+1, SPn-1

    - FP or TP input mux (FP_ctrl, TP_ctrl)			  6*2	  8*2

Inputs: Op2, EA; Operations: Select one

    - Multiplier (DSP48)								    1

Inputs: QPMDOB, Op2; Operations: 2’s complement multiply

    - Instruction Block RAM (32x512 & 16x1024)					    1

Inputs: PCALU, DID, EA; Operations: Sync dual port RAM

    - Instruction decode						~40	~40

    - Registers: PC, Stack pointers, FP, TP, CCR 	10,4*6,12,12,32  90-144Dff

~282	~376

  - Data access to instruction block RAM (optional)

When using quad port LUT RAM to hold the stack values, a second port to the block RAM can be used for read/writes to the block RAM.  Writes can take place at the end of an instruction cycle.  Read address can be posted at the end of an instruction cycle but the data will not be available until the next cycle.

  - Implementation of various data sizes (optional)

Support of the various data sizes is simplest if it only applies to block RAM reads and writes.  A four way shifter can be used to scale the offset to match the data size.  Data input and output sifters are also required.  And control signals to implement the byte write enables.

  - Barrel shifter (optional)

  - LUT logic (optional)

  - Extract/Insert logic (optional)

  - Bit banding logic (optional)

  - Case and/or Threaded code dispatch (optional)

- Architecture Unification

  - Architecture character:

The market being addressed is soft-core processors using FPGA fabric.  As such a number of small processors running at high clock rates can be preferable to a larger, more powerful processor running at a slower clock rate and using more resources.  Indeed, the fixed size FPGA resources for RAM create sweet spots for program and data capacity.  Ideally one wants to use only one or a few Block RAMs per processor.  Use of a DSP block for the ALU can lead to low logic resource usage and to high clock speeds.

The architecture character herein is oriented towards a generic programming language style, that being that of algebraic languages with evaluation of expressions and subroutines.  This leaves out those special features for functional programming or event driven architectures.  And the architecture character is oriented towards a simple mapping between algebraic language operations and those operations in the instruction set.

The overriding architecture character is that a range of processors are possible using a range of resources and implementation complexities.  At its core this class of architectures uses accumulators and stacks.  And IO registers to connect to the outside world.  As memory needs increase one adds data memory and instruction memory.  As these memories get larger, they migrate from LUT RAM to Block RAM(s).  As performance needs increase, data widths increase and co-processors are added.

The instruction set is very fluid: by using prefix chunks, both long-complex and short-simple instructions can be handled efficiently.  As the instruction register can always be made larger, adding additional parameters and op-codes can be done without changing much besides instruction decoding.

Short instructions use default values for things such as data size, operand and destination locations (e.g. on a stack for minimal instruction size).  Larger word size versions can be upwards compatible with their smaller brothers by including an instruction field for data size.

There is a second aspect to instruction set encoding.  As the IR is zeroed after instruction execution and can be made as large as needed, the order of bits in the IR can be adjusted so that op-code bits interleave with mode bits and offset bits.  One can examine the compiler generated code and optimize the IR bit ordering so that the most necessary bits occur last in the instruction stream.  This minimizes the number of prefix chunks in the instruction stream, e.g. minimizes either static or dynamic program length.

  - Instruction character:

The basic instruction has a small, say 8, 12 or 16-bit “chunk”.  Instruction can be lengthened by prefix chunks and is gathered into the instruction register by a shift and insert sequence.  The entire instruction register is cleared after each instruction.  The entire instruction register is always in use.  The basic instructions are short with additional parameterization being zero.  Thus a short format load immediate op-code may only load zero but with additional prefixes can load a succession of larger data values up to or greater than the accumulator size.  This gives instruction memory economy for the simple modes for various op-codes.

The instruction memory can be made wider than the chunk size so that prefix look ahead can gather all the prefixes in fewer clock cycles.  And instruction memory can have a bit size different from the data granularity.

Processing efficiency follows from each instruction doing several things.  Thus the simple operations use an accumulator and one of the stacks or other accumulators for the operands and put the result back into the accumulator.  More elaborate operations access data memory via Thread and Frame pointers and do replace operations to the selected memory location.  A subroutine return bit can be included in the instruction so that a distinct return operation is seldom needed.  The Thread and Frame pointers can be operated in auto-advance (T bit) mode for sequencing through data memory.

Dual stack processors are easy targets for simple compilers.  For short, simple subroutines the dual stack model works well and has low call/return overhead.  Larger subroutines, which seem to be the norm, require a stack frame where local data and local parameters are kept.  For the architecture considered herein, entry to a “large” subroutine requires pushing the Return address and the Frame pointer, copying parameters from the data stack to the frame, and initializing local data.  Exit from a “large” subroutine requires copying results to the data stack, popping the frame pointer and the return address.  It is envisioned that compilers will use optimization criteria to choose between stack based and frame based subroutine structure and also optimize when to in-line subroutines.  The subroutine call for a “small” subroutine can be very fast via the use of LUT RAM for the Return stack and most “small” subroutine returns can be merged into the last subroutine instruction execution via setting the “return” bit in the final subroutine instruction.

  - The memory structure is:

    - Unified address space in that instruction memory, data memories and stack memories part of a single address space.  Data, frame and thread memory varies with implementation.

    - Accumulators, their stacks and pointers, stacks via LUT or Block RAMs

    - Program memory, mostly read only and can be wide

Certain alignment strategies can be used to provide high performance for long instructions.  With minimal decode or shifting alignment, long instructions can be dispatched rapidly, e.g. minimizing prefix cycles.

    - Co-processor register files, via a multi-port read, single port write LUT RAM

Each register file caters to a specific class of instructions such as floating-point, multi-media or loop/address counters (with counter, delta, refresh value & loop start address).  Multiple instruction issue possible using long instructions containing multiple co-processor operation fields.

The co-processor logic is built around the co-processor register file.  Thus multiple identical co-processor instances are possible and several distinct co-processor types are possible.  For multi-cycle co-processor operations a variety of interlock possibilities exist including a queue for operations in waiting.

  - The register structure is:

    - Program counter, the center of program flow

    - Instruction register, herein can be small to very large

    - Frame pointer, a place for subroutine data such as large numbers of parameters

Dual usage as an auto increment/decrement pointer

    - Thread pointer, a place for thread specific data

Dual usage as an auto increment/decrement pointer

    - Stack pointer, pointer to associated accumulator.

  - Growth pattern: from state machine to general purpose programmable processor

    - Starting very basic:

- The simplest processor is a small state machine.  States are given names by enumeration and each state has the expressions for computing and for going to the next state.  State machines without an ALU will be characterized as “small”.  There is no instruction set.

- No instruction memory, only the ALU and registers. This is the realm of micro-code and medium size state machines.  E.g., there is no instruction set other than the micro-code signals.  Without a formal ALU it can’t be considered general purpose.  If a register file (with say two read ports and one write port) is used instead of an accumulator one has a register oriented computer instead of a memory oriented computer.

- At some point the need for subroutines and organized processing, with intermediate data, brings in an accumulator with data and control stack RAMs.

- At some further point, need for code compactness leads to an instruction decoder and instruction memory.

- At some further point the need for subroutine working space leads to adding a Frame pointer.  It could point to within the control stack.  Design complexity favors a distinct memory for frame data and/or global data.

- Adding a Thread pointer allows multi-threading with thread changes requiring multiple sets of Thread, Frame, Return stack and Data stack pointers along with the other state such as the PC, Accumulators and Status register.

    - Starting with no Thread or Frame pointers: All addressing is global

For some embedded applications, a global data pool is possible.  An example is when there is no subroutine recursion, so each subroutine can have a section of global memory dedicated for its parameters, results and local data.

    - Starting with no co-processors

Typical.

    - Starting with no Return stack

For some embedded applications, only one or two levels of subroutine calls occur.  It is then efficient to allocate D flip-flops for the return address as opposed to a return stack RAM and its pointer register.

    - Starting with no Global data memory, use data stack only

Rare, insignificant.

    - Starting with no Data stack or Return stack

Changes the basic nature of the instruction set.  All operations are to/from data memory (e.g. an accumulator design).

- Compilers and optimization

  - For embedded applications with small amounts of code, global variables are simpler than stack frame variables.  Compiler should be bright enough to decide between the various forms.  12X_uP can support thread space separate from global space, so this optimization can generate five forms: global, thread, PC relative, frame and stack allocation of variables and values.

  - A sufficiently bright compiler should be able to inline subroutine calls on a case by case basis (e.g. if the parameters of the call allow simplification of the subroutine code and if there are few calls of this sort – e.g. do a cost benefit analysis).  Also in the bright category is choosing between using a stack frame and not using a stack frame (and thereby avoiding the stack frame overhead).  For the 12X_uP architecture the stack frame overhead may consist of memory cycles to save/restore the previous stack frame and memory cycles to copy the parameters from the data stack to the return stack.

- References

- Glossary

- The Extended nX_uP Family

The extended processor family takes the same resources (LUT RAM for stacks or register file, Block RAM for main or cache memory and “load-store” access to main memory) and considers a wide range of word sizes from 6 to 64 bits.  Load and store accommodate a variety of submultiple data sizes (such as 8/16/32/64 or 6/12/24/48 or 9/18/36/72).

Word size and address size are assumed to be similar.  Larger word sizes naturally lead to larger program sizes, higher performance and larger stack or register files.  The same set of op-codes may not be present for the various word sizes.  However, the default ALU operation word size is the register size.

In all word sizes, the RISC instruction set uses 5-bit register designators and at least a 5-bit op-code.

The instruction sets are most easily expressed in a generalized form by using 6, 8, 9 or 12-bit instruction chunks.  The shift and load of the chunks into the instruction register includes the comma bits so that they can be part of the instruction decode.

  - Six bit chunks

The comma code (e.g. termination flag) takes one bit leaving five.  First chunk is all op-code “XXXXX”, second chunk is “E” and “MNNN”.  Third is “R” and “P”, another “X”, one “N” and N’s sign “s”.  Space is made in the 32 op-codes for push immediate (both 6-bit and 12-bit).  Making op-code space for load/push L(LN) is impractical.

For RISC instructions, first chunk is op-code, second chunk is the “D” register, third chunk is the “R” register, fourth chunk is the “S” register and the fifth chunk is more op-code.  Thus, a RISC op-code can have zero (such as increment), one (replace add) or two (three register add) operand meanings.

  - Eight bit chunks

With two more bits per chunk, the “E” and “P” can move to the first chunk.  Additional stack pointers can be addressed in the second chunk, as well as more “N” bits.

  - Nine bit chunks

Similar to eight-bit chunks.

  - Twelve bit chunks

See section 1.3.

  - Experimental Variations

Given that the12X_12uP has reached ~1K LUTs and needs to be rewritten to use a fixed data path, there are the issues of how other approaches compare in LUT count, Fmax and code density.

    - “RISC” using 24-bit instructions and 6-bit register fields

    - Simple Stack Machine using 6-bit instructions

    - 16-bit chunks with evens being subroutine calls and odds being instructions

    - 16-bit chunks with additional bits for: op-code, stack, stack offset & 9.5.3

    - Stack in dual port block memory (two clocks/instruction, stack locations addressable)

    - Program as a VHDL array constant (and it’s optimization) with/without block RAM

    - Effect of moving ALU operations to DSP block

    - TBD

  - TBD

- Debug and Test Sequence

The development sequence is to implement a few instructions and implement a corresponding test.  On power-up the sequence of tests is executed with a halt at the end of any failing test.  The halt displays either the instruction address or the fault code (to an eight-bit or more LED string).  Sequencing beyond the halt is via a push button (there being a “reset” and a “continue” push button).

The initial implementation is for a Zynq “Zed Board” as this allows use of 6LUTs, quad port LUT RAM and the DSP ALU for logic as well as arithmetic.  The LS9 micro-board (Spartan-6) is also usable.

  - Hello World for FPGAs (completed Oct. 9, 2015 using LX9 micro-board)

A 36-bit counter is implemented by three 12-bit words.  Only four instructions are required: LDI, INC, BRNZ/BRZ and OUT.

  - Use push button as continue (completed Oct. 11, 2015 using LX9 micro-board)

A 36-bit counter is implemented by three 12-bit words on the stack.  Instructions required are: LDI, INC, CALL, BR, BRNZ/BRZ, IN, OUT and ANDI.

  - Full instruction set (complete Nov. 2, 2015, no ALU 12-bit imm, no shift, no INITstk)

LUT count				= 994

Without extended CCR conditions	= 926, factoring out inst(0) gave 957

Reordering ccr_evalf case gave 927

Without all CCR conditions		= 844

Without CCR cond, pack & unpack	= 826

  - Revise full instruction set (Nov. 24, 2015)

Added LOOPCCC, removed SKCCC, added EXEC

Merged all specials into a single op-code zone

55 regular (including five floating-point + divide), 38 specials = 93 total

LUT count & Fmax (no XCCR)	= 961 & 62.8MHz (no SHFT, no INITstk, no imm’s)

LUT count & Fmax (with XCCR)	= 980 & 62.5MHz (no SHFT, no INITstk, no imm’s)

  - Two stage pipe: better control of ALUs

First stage reads the instruction; registers pointers, imm’s & control signals for 2nd stage

Multiple states for some inst., flag to indicate CCR about to change

Second stage reads the LUT RAM, does the ALU & registers/writes the results

Both stages interface to separate ports of block RAM

  - Test full instruction set (completed Nov. ??, 2015 using LX9 micro-board)

Do a variety of tests on each op-code with a trap to a common routine that displays a fault code or the return address until user push button pressed and released.  The common routine de-bounces the push button.

  - Optimize data path (completed Dec. ??, 2015 using Zynq ZedBoard)

Create data path multiplexors, use DSP ALU, enumerate the control signals and re-write the VHDL to generate the control signals.

E.g. has both a data stack (DS) and a return stack (RS)

Provides a locals/frame pointer (FP) for subroutine frames and supports merging locals with the data stack.

In its current state the architecture does not provide stack windowing from main memory as a means to automatically handle stack over/underflow.

CCR pushed and popped from the return stack as necessary.  E.g., if a routine modifies the CCR, it must save and restore the previous CCR.

At this time the data stack is a descending stack and the return stack is ascending.  Forth calling convention is that parameters are on the data stack, destroyed by the subroutine, and results are in their place on return.  Methodology for locals is that the subroutine is called and return address is on the return stack.  The locals pointer is pushed onto the return stack and the locals pointer updated to the new locals location.  If locals are on the return stack, parameters are then moved one at a time from the data stack to the return stack.  On exit the locals pointer is popped (recovering the stack space used by the locals) and then the return address popped.  There are complications in Forth in the usage of the return stack for loop counters.  Here the loop counter is considered local and requires the locals pointer push.  The current implementation has the stack frames on the data stack.

There are two ways for the locals pointer to work: It points to the return stack or to the data stack.  If to the return stack, the parameters and other locals must be copied or moved from the data stack.  If to the data stack, parameters and locals are on the data stack and are not moved/copied, however before returning the results must overwrite the parameter area.  Thus, if the subroutine returns few results, it is more efficient to have the locals on the data stack.  The current  instruction set supports locals on the data stack only.

They can be many variations including Harvard architecture, e.g. instruction-size not a sub-multiple of data size.  Exploration of these variations is part of the research process. One of the goals herein is to provide a simple architecture capable of such variations.

The accumulator choice is implied by the instruction: integer or floating-point, the offset location in four bits: one for the stack or locals pointer register select and three for the offset. Most operations take only two unique addresses as the result address is related to one of the operand addresses.  Here the “E” bit is included in the bit count as it specifies one of two locations for the result.

The intent of the Extra-Code is to provide additional variations as well as additional N-bits for each Op-code listed below.

Encoding space is very tight: Sixteen conditions are allowed including the negatives.  Use of the Extend instruction allows additional conditions such as =-1, =-2, =+1, all 1s, signed max and min, etc.

One can always add special instructions that facilitate functional or event operations.

The condition code register (CCR) contains the default value for data size.

E.g., Harvard architecture: data memory separate from program memory and may have different word size.

LUT and block RAM have short access times compared to external DRAM.

It might be advantageous to make instruction alignment to half-words rather than bytes at cost of NOP fillers

Multi-port register files are available in FPGAs and simple register oriented instruction sets (ROIS) are easy to implement.  A compiler for ROIS must be more complicated to do efficient register allocation.  Instruction and data memory are usually shared and implemented in Block RAM.

Historically the choice was to add a single, all purpose, read/write memory.  Instruction fetch and data load/store occurred to the same memory.  Many instructions took several memory cycles to complete.

With FPGAs and their variety of memory resources, dedicated memories for each purpose make sense.  And most instructions can compute in a single clock cycle.  Herein the major memory types are: data stack, return stack, data memory and program memory.  Add register files if co-processors are warranted.

Why four memory sections when one will do?  It’s a matter of total memory bandwidth: Here, a given instruction can interact with the four memories simultaneously.  As opposed to current architectures, famous for the “von Neumann bottleneck”, where instruction and data caches are needed to improve on the single path to a large memory and to overcome the slow speed and long access time of that large memory.