Register oriented instruction set

For comparison against the 12X_uP

James Brakefield

Contents

Introduction

Instruction Formats

Op-codes

Speed-ups

Implementation Decisions

Compilers and Optimization

Debug and test sequence

Introduction:

When devising a soft core microprocessor (uP) with a 24-bit data and address word size the RISC variation is immediately apparent:  Three register fields of up to 64 registers and a 6-bit op-code fits nicely into a 24-bit instruction.

The intended FPGA implementation is with a single LUT RAM for the registers and Block RAM for “main memory”.  Thus, an instruction can be read from Block RAM memory, the operation performed and the results clocked into LUT RAM in a single clock cycle.  Block RAM data access (read or write) requires an additional main memory cycle (unless Block RAM is dual ported with separate instruction read and data read/write ports and that the data memory cycle is pipelined).

Rois24_24uP is meant as a comparison design for the 12X_uP architecture which supports two or more stacks as well as accumulator style instructions.  Rois24_24uP has three variants: the first using six-bit register designators, the second using five-bit register designators (the remaining three bits used to augment the instruction op-code) and the third offering the register mapping found in 12X_uP.

Data movement between the registers and main memory is via load/store instructions.

An overall programmer’s model consists of:  The program counter (PC), the instruction register (IR) and a condition code register (CCR).  There is an overall memory map which includes program memory, data memory and possibly memory mapped input/output (IO) ports.

- Instruction Formats

- Register to register				XXXXXX DDDDDD RRRRRR SSSSSS

- Immediate1				XXXXXX DDDDDD RRRRRR NNNNNN

- Immediate2				XXXXXX DDDDDD sNNNNN NNNNNN

- Immediate3				XXXXXX NNNNNN NNNNNN NNNNNN

- X:    Op-code		D:    Result register

- R:    1st operand register	S:    2nd operand register

- N:    immediate operand	s:     N’s sign (2’s complement) bit

  - 5-bit instruction encoding option

By restricting the register fields to five bits (32 registers, still plenty for RISC) three additional instruction bits become available.  The allocation of these bits is as option bits: a return bit, a CCR update bit and an immediate bit.

Register to register				XXXXXX XDDDDD XRRRRR XSSSSS

Immediate1				XXXXXX XDDDDD XRRRRR NNNNNN

Immediate2				XXXXXX XDDDDD sNNNNN NNNNNN

Immediate3				XXXXXX NNNNNN NNNNNN NNNNNN

X:    Op-code		D:    Result register

7th X: return using contents of register 31

8th X: if = 1: update condition code registers

R:    1st operand register	S:    2nd operand register

N:    immediate operand	s:     N’s sign (2’s complement)

- Op-codes			XXXXXX DDDDDD RRRRRR SSSSSS

Register 0 always reads as zero

PFX instruction loads 18-bits into prefix register.

Used for concatenation to a 6-bit “N” in a following instruction.

Immediate2’s are sign extended.  Immediate1’s & 3’s are not sign extended.

Three address arithmetic:

ADD	Add

SUB	Subtract

ADC	Add with carry

SBC	Subtract with borrow

AND	And

ANDN	And with second operand complemented

OR		Or

XOR	Exclusive Or

MUL	Multiply, least significant 24 bits

MULU	Multiply, most significant 24 bits, unsigned

MULUS	Multiply, most significant 24 bits, signed

DIV	Divide start, (D & R) / S

FADD	Floating point add

FSUB	Floating point subtract

FMUL	Floating point multiple

FDIV	Floating point divide

Immediates (immediate 1 format):

ADDI, SUBI, ADCI, SBCI, ANDI, ANDCI, ORI, XORI,

MULI, MULUI, MULUSI, DIVI, FADDI, FSUBI, FMULI, FDIVI

(MULI instructions replace need for shift instructions):

LSRI	Logical shift right

ASRI	Arithmetic shift right

LSLI	Logical shift left

RORI	Rotate right

Immediate (immediate 2 format):

LDI		Load register with sNNNNN NNNNNN

Immediate (immediate 3 format):

PFX	Load prefix register with 18 bits from NNNNNN NNNNNN NNNNNN

Load/Store:

LD		Load register D with memory location R+S

ST		Store register D at memory location R+S

Load/Store (immediate 1 format):

LDN	Load register D with memory location R+NNNNNN

STN	Store register D at memory location R+NNNNNN

IN		Place contents of input port R+NNNNNN into D

OUT	Transfer D to output port R+NNNNNN

Relative branches (immediate2 format):

BRCC	Branch to PC+sNNNNNNNNNNN if condition DDDDDD true

CALLR	Branch to PC+sNNNNNNNNNNN and save PC at register D

Three address branches:

CALL	Branch to address R+S, save PC+1 at register D

CALLN	Branch to address R+NNNNNN, save PC+1 at register D

JMPCC	Jump to R+S if condition DDDDDDD true

JMPCCN	Jump to R+NNNNNN if condition DDDDDDD true

- Possible speedups

  - Multi-stage pipelining, complicates branches.

  - Have the program memory 48-bits wide.  Process two instructions simultaneously.

  - Implement parallel ALUs.  Probably required by 2.2

  - Have the compiler arrange multiple instructions per word so multiple dispatch always on (e.g. NOPs inserted as necessary by compiler and entire 48-bits of instructions always dispatched).

  - Add co-processor(s) and support long instructions with sections for each co-processor (e.g. another way to do multiple dispatch).

  - Implement multiple threads via pipeline and pipeline barrel.  High speed without pipeline interlocks.

- Variations

The main constraints are LUT RAM for the register file(s) and Block RAM for program and data memory.  In the larger memory space versions block RAM becomes a cache for main memory (which is external to the FPGA).  LUT RAM for a single register file is constrained to have a single write port and may have as many read ports as needed.

The variation dimensions are RTL description (pipeline-depth, data-path, control signals), instruction encoding (op-codes, modes, number of register fields), instruction bit size (24, 16 or 12-bit) and addressable unit (8-bit, 12-bit or word).

All the variations listed below assume that one or more instructions are processed per clock.  The single stage pipe versions of load and store instructions take two clocks.  Two stage pipe versions of conditional instructions take two clocks if the condition code is modified by the pervious instruction.  To the extent that they implemented; multiply, divide and floating-point instructions may take more than two clock cycles depending on the pipeline implementation and result delay interlocks.

  - Minimal implementation (MI)

Only a few instructions are implemented and the program is expressed by a case statement which allows further optimization (program and instruction logic get folded together, optimization includes removing unused instructions and modes).  Fmax very good and LUT count is minimal.

  - Basic implementation (BI)

The basic architecture is a single stage pipe, triple port LUT RAM, single port block RAM.  Instruction implementation is a simple case statement, one entry for each op-code.  Then there is a second case statement to handle the second clock cycle for memory read/write.  Program typically in block RAM.

Synthesis results in many adders and multiplexors which get optimized by translate & map. Clock speed is good and LUT count is high.

The great advantage of BI is the ease of adding or removing instructions (one can simply comment out unused instructions).

  - Data path with control signals

The case statement for instruction specification now generates control signals rather than data operations.  Number of signal names increases and debugging is more difficult.

    - Inferred data path

Adders with similar inputs are coded separately from the instruction case statement.  The multiplexors are inferred as is the optimizations.  Results are good!  Debugging is much simpler than designed data paths with defined control signals.

    - Many adders, multiplexors mostly after the adders

Variation is unexplored at this time.  Variation needed for comparison against DP.  Possibly superior Fmax at small increase in LUT count.

    - Few adders, multiplexors on both sides of adders (DP)

Preliminary indications are that LUT count is dramatically lower than that of BI.  Good control over synthesis results.

  - Two stage pipeline

    - With single port block RAM

    - With dual port block RAM

This variation is expected to give the best performance metric (Instruction work per LUT per millisecond).  Use of more than two pipe stages likely to give marginal improvement to Fmax and result in increased complexity and LUT count.

  - Five bit register designators

    - Additional instruction bits

With three five bit register designators there is room for nine instruction bits.  Planned usage for the additional bits are: return bit and condition code save bit.  The third bit remains for six-bit immediates.  The condition code save bit is the MSB of the D register field and return bit is the MSB of the R register field.

    - Register designation recode

Rather than have 32 fixed location registers it is possible to provide a number of register offsets that correspond to: return address stack, data stack(s) and frame pointer.  Thus one can specify a register relative to these pointers.

Additionally it is useful to specify modification of the offset pointers.  Thus an instruction can specify stack pops (from a source stack) and pushes (to a destination stack).

  - Code compression

The MIPS instruction set uses up to three five bit register designators and a variable size instruction.  Here, the baseline instruction is three six bit register designators and a six bit op-code.  By being 75% of the MIPS instruction size (32-bits), it is 33% more compact and within the realm of “densely coded” instruction sets.

Further compression requires either drastically reduced register designators or fewer register designators.  Here the limit is a single register designator which requires either a stack or accumulator style instruction set.  To the extent that a 16-bit or 12-bit instruction can perform the same amount of “instruction work” the code size is 2.0 or 2.67 times as dense as MIPS.  A true stack instruction set (both operands and result implied) with 6 or 8-bit instructions is not considered since an instruction set with a single register designator performs significantly more “work” per clock cycle.

    - 16-bit instructions

A single 5-bit register designator using offset pointers; a 6-bit op-code with the remaining bits specifying: Return, condition code update, replace operation, stack pop/push and immediate.

    - 32-bit instructions

The register specifiers are enlarged to 8-bits.  Indirect memory references are encoded, a typical encoding is six modes (register, register indirect, register indirect post increment, register indirect pre decrement, register with offset, register with offset and update) and 256/6 registers.  Also supported are immediate, absolute, stack and relative addressing.

    - 12-bit instructions

A single 5-bit register designator using offset pointers; a 5-bit op-code with the remaining bits specifying: replace operation, stack pop/push.

  - Addressable unit size

    - 12-bit

Original design is for 12, 24 or 48 bit word sizes.  With 12-bit addressability the functionality closely matches 8/16/32 uP.  Practical considerations may necessitate 8-bit addressability (UTF-8 in particular).

    - 8-bit

One can dispense with 12-bit items being addressable.  Thus instructions would be either 24 or 16-bit and memory would be 8, 16, 32 or 64-bits wide.  Memory would be in two banks with separate address adders for each bank.  Instruction readout for 24-bit instructions requires a shift circuit to align the instruction.

    - 4-bit

Advantage is that both 12-bit and 8-bit items are directly addressable.  Not considered viable.

    - 1-bit

Advantage is that any bit size item can be directly read from or written to memory.  Not considered viable.

  - LUT RAM word size

    - 12-bit

The intent of a 4K memory space with 12-bit words is small micro-controllers.

    - 24-bit

The intent of a 16M item memory space is embedded systems with program and data that fit.  There is a cost savings over 32-bit uP.

    - 48-bit

The intent of a 48-bit uP is all applications short of very large super-computers.  Its 48-bit floating-point is adequate for many applications.  It holds the sweet spot between 32-bit and 64-bit uP.

    - 16-bit

TBD

    - 32-bit

The 32-bit uP provides 4GB of address space, sufficient for most general purpose computing including full operating systems and large libraries of code.

  - High performance variations

    - Multiple register files

    - Parallel decode

- Implementation decisions

  - Block RAM versus LUT RAM for the stacks

The LUT RAM approach keeps the stacks in LUT RAM and the program and data in block RAM.   Block RAM memory bandwidth is limited to instruction reads and data memory read/write.  LUT RAMs allow asynchronous operation, meaning that a single phase instruction cycle is possible: Read instruction, decode instruction and forward stack read addresses to the LUT RAM, forward LUT RAM read data to ALU, write ALU result to LUT RAM or data RAM.  Instructions with data RAM reads or writes require an additional phase.

The alternate implementation is to use a single dual port block RAM for all RAM usage: program memory, stack memory and data memory.  The processing cycle consists of two phases:

- Writing the result of the previous instruction to memory while also reading the next instruction.

- Reading the two operands and computing the result.

Phase A includes instruction decode and address generation.  Not all instructions require phase B, in particular the conditional branches.  Some instructions will require a phase C, such as doing a subroutine return as part of an ALU instruction.

As block RAM can cycle faster than needed for the two phases, it is possible to do two or more instructions in about the same amount of time by pipelining the address generation and the ALU computation.

This block RAM approach is similar to the historical three address computer.  Due to the dual ports and performance of FPGA block RAM, this is simple to implement and gives high performance.

  - Go for a minimal design

There are arguments for going for either the smallest possible design or going for the highest performance design.  The figure of merit is DMIPS per LUT.  Or equivalently, DMIPS per unit silicon area, which factors in the area taken by multiplier/DSP units and block RAMs.  Of course LUTs from different FPGA families are not necessarily equivalent.  The ultimate figure of merit is DMIPS per dollar using the prorated chip area.

The DMIPS per dollar and DMIPS per unit silicon area metrics also allow comparison with FPGAs with hard core processors such as the PowerPC, ARM Cortex A9 or Cortex A3, or with non-FPGA embedded processors.

Naturally some applications call for very little data RAM or instruction memory.  Other applications call for a full 32-bit processor with floating-point, caches and/or memory management.

Interestingly enough there are examples of high performance FPGA soft core processors at the minimal level, at the single processor pipelined level and at the multi-processor/barrel approach.  Thus it is possible to get a high figure of merit for any of the three design approaches.

Given that rois24_24uP is a new architecture, a simple or minimal implementation is the lowest risk and fastest to implement choice.  A minimal implementation also offers the greatest insight into LUT count versus instruction set complexity or cycle time.

  - Use a single pipeline stage

This mode of operation uses LUT RAM with asynchronous read for fetching operands.  Thus in one clock cycle an instruction is read, is decoded, the operands are read, the operation performed and the results along with the updated PC registered at the beginning of the next clock cycle.

A pipeline design with additional stages will result in a higher clock speed and greater performance.  However, there is greater complexity which is best left to when after the single pipeline stage design is up and running.  A full pipelined “barrel processor” design, with one processor per pipeline stage, is a straight forward way to get maximum clock rate.

  - Do not use a data path

For a simple processor implementation with single pipeline stage, the instruction decode is a large case statement with each case condition expressing the instruction calculation.

Normally one would code the data path and have the instruction decode generate the control signals for the data paths.  For sizeable instruction sets, this saves LUTs and results in a faster clock cycle.

  - Use an inferred data path

It is straight forward to promote shared use of adders by coding the adders separate from the instruction case condition coding.  The tools will then use the limited number of adders with a big reduction in LUT count (50% or more) and improved Fmax (20%).  A designed data path may or may not improve the LUT count?

  - Condition code register (CCR)

The conditional branch instructions specify a condition code.  There are arguments for and against condition code registers.  Here, it was decided to stay with a full condition code register.  Thus, the CCR contains carry, signed overflow, zero and sign for each accumulator.  Also included is a least significant bit (even/odd).  And four bits for: all ones, all zeroes for both the floating point exponent and the floating point mantissa fields (the overall zero bit becomes redundant).

Interrupt enable and Interrupted bits also present.

  - Condition code register contains the ALU result

It takes additional logic to generate the some condition code bits from the ALU result (such as the zero bit).  Instead save the entire ALU result and do the additional logic during the logic for conditional branches.  To save the CCR, the ALU result is reduced.  To restore the CCR, the all zeros and all ones bits are “unreduced”.  CCR saved/restored via IO instructions.

  - Keep the accumulators in RAM (LUT or Block)

Keeping the accumulators in RAM eliminates the complexity of updating discrete accumulator registers and pushing them to RAM.

  - Use block RAM for instruction memory

The smallest block RAM allows for several hundred instructions, a reasonable number.  Reading instructions from a synchronous RAM is compatible with a single pipe stage implementation.

  - Use dual port or quad port LUT RAM

A Xilinx series 6 or 7 FPGAs support quad port LUT RAMs (three asynchronous read ports and one asynchronous read/synchronous write port).  For the rois24_24uP instruction set one can use three ports: a read port for one operand of an instruction, a read port for the second operand and a write port for the result data.

Some Altera FPGAs support a simple dual block RAM “MLABs” (with asynchronous read and a separate synchronous write).  By operating two MLABs in parallel one can obtain two asynchronous read ports and a single synchronous write port (always writing the same location and data to the two MLABs simultaneously).

Micro Semiconductor, formerly Actel, and Lattice Semiconductor FPGAs provide similar capabilities as the Xilinx and Altera FPGAs (block RAM and LUT or MLAB RAM).

  - Use block RAM for all operands and results

With a two stage pipeline and a full dual port RAM, the sequence is: 1st pipeline stage reads the next instruction and writes the previous result.  2nd pipeline stage reads the two operands and computes the result.  Address calculation fitted into first stage.  ALU operation fitted into second stage.  Delay exceptions or resource constraints force additional pipeline stages.  A conditional branch can be fitted into a single pipeline stage if it uses CCR results from completed instructions.

  - Using instruction memory for data memory

Given that instruction memory is block RAM and therefore has synchronous reads, and supports dual port operation (first port for instruction reads, second port used for data RAM read/write), the single stage pipe design does not allow a block RAM data read to occur in the same cycle as the instruction read.  However a block RAM data write can occur at the end of the cycle.

Given that it takes one cycle to post the read address and the next cycle to read from the block RAM, a variety of arrangements can be made for data read: Use two instructions, one to post the read address and the next instruction to process the data.  Or use a memory pointer register (in IO space) which auto increments after each data read and fetches data at the next location.

  - Interrupts

Interrupts shall push sufficient information (program counter, condition code register, etc.) to transparently resume the program at the point of interrupt.

- Architecture Unification

  - Architecture character:

The market being addressed is soft-core processors using FPGA fabric.  As such a number of small processors running at high clock rates can be preferable to a larger, more powerful processor running at a slower clock rate and using more resources.  Indeed, the fixed size FPGA resources for RAM create sweet spots for program and data capacity.  Ideally one wants to use only one or a few Block RAMs per processor.  Use of a DSP block for the ALU can lead to low logic resource usage and to high clock speeds.

The architecture character herein is oriented towards a generic programming language style, that being that of algebraic languages with evaluation of expressions and subroutines.  This leaves out those special features for functional programming or event driven architectures.  And the architecture character is oriented towards a simple mapping between algebraic language operations and those operations in the instruction set.

The overriding architecture character is that a range of processors are possible using a range of resources and implementation complexities.  At its core this class of architectures uses accumulators and stacks.  And IO registers to connect to the outside world.  As memory needs increase one adds data memory and instruction memory.  As these memories get larger, they migrate from LUT RAM to Block RAM(s).  As performance needs increase, data widths increase and co-processors are added.

The instruction set is very fluid: by using prefix chunks, both long-complex and short-simple instructions can be handled efficiently.  As the instruction register can always be made larger, adding additional parameters and op-codes can be done without changing much besides instruction decoding.

Short instructions use default values for things such as data size, operand and destination locations (e.g. on a stack for minimal instruction size).  Larger word size versions can be upwards compatible with their smaller brothers by including an instruction field for data size.

There is a second aspect to instruction set encoding.  As the IR is zeroed after instruction execution and can be made as large as needed, the order of bits in the IR can be adjusted so that op-code bits interleave with mode bits and offset bits.  One can examine the compiler generated code and optimize the IR bit ordering so that the most necessary bits occur last in the instruction stream.  This minimizes the number of prefix chunks in the instruction stream, e.g. minimizes either static or dynamic program length.

  - Instruction character:

The basic instruction has a small, say 8, 12 or 16-bit “chunk”.  Instruction can be lengthened by prefix chunks and is gathered into the instruction register by a shift and insert sequence.  The entire instruction register is cleared after each instruction.  The entire instruction register is always in use.  The basic instructions are short with additional parameterization being zero.  Thus a short format load immediate op-code may only load zero but with additional prefixes can load a succession of larger data values up to or greater than the accumulator size.  This gives instruction memory economy for the simple modes for various op-codes.

The instruction memory can be made wider than the chunk size so that prefix look ahead can gather all the prefixes in fewer clock cycles.  And instruction memory can have a bit size different from the data granularity.

Processing efficiency follows from each instruction doing several things.  Thus the simple operations use an accumulator and one of the stacks or other accumulators for the operands and put the result back into the accumulator.  More elaborate operations access data memory via Thread and Frame pointers and do replace operations to the selected memory location.  A subroutine return bit can be included in the instruction so that a distinct return operation is seldom needed.  The Thread and Frame pointers can be operated in auto-advance (T bit) mode for sequencing through data memory.

  - Growth pattern: from state machine to general purpose programmable processor

    - Starting very basic:

- The simplest processor is a small state machine.  States are given names by enumeration and each state has the expressions for computing and for going to the next state.  State machines without an ALU will be characterized as “small”.  There is no instruction set.

- No instruction memory, only the ALU and registers. This is the realm of micro-code and medium size state machines.  E.g., there is no instruction set other than the micro-code signals.  Without a formal ALU it can’t be considered general purpose.  If a register file (with say two read ports and one write port) is used instead of an accumulator one has a register oriented computer instead of a memory oriented computer.

- At some point the need for subroutines and organized processing, with intermediate data, brings in an accumulator with data and control stack RAMs.

- At some further point, need for code compactness leads to an instruction decoder and instruction memory.

- At some further point the need for subroutine working space leads to adding a Frame pointer.  It could point to within the control stack.  Design complexity favors a distinct memory for frame data and/or global data.

- Adding a Thread pointer allows multi-threading with thread changes requiring multiple sets of Thread, Frame, Return stack and Data stack pointers along with the other state such as the PC, Accumulators and Status register.

    - Starting with no Thread or Frame pointers: All addressing is global

For some embedded applications, a global data pool is possible.  An example is when there is no subroutine recursion, so each subroutine can have a section of global memory dedicated for its parameters, results and local data.

    - Starting with no co-processors

Typical.

    - Starting with no Return stack

For some embedded applications, only one or two levels of subroutine calls occur.  It is then efficient to allocate D flip-flops for the return address as opposed to a return stack RAM and its pointer register.

    - Starting with no Global data memory, use data stack only

Rare, insignificant.

    - Starting with no Data stack or Return stack

Changes the basic nature of the instruction set.  All operations are to/from data memory (e.g. an accumulator design).

- Compilers and optimization

  - For embedded applications with small amounts of code, global variables are simpler than stack frame variables.  Compiler should be bright enough to decide between the various forms.

  - A sufficiently bright compiler should be able to inline subroutine calls on a case by case basis (e.g. if the parameters of the call allow simplification of the subroutine code and if there are few calls of this sort – e.g. do a cost benefit analysis).  Also in the bright category is choosing between using a stack frame and not using a stack frame (and thereby avoiding the stack frame overhead).

- Benchmarks

The intent is to provide a list of benchmark functions easily coded in assembler.  For each function and its evaluation at a particular input, report code size and performance in clock count.

Results will become extensive with a variety of word sizes and register encodings and instruction set additions.  For instance, if multiply is not provided, then single bit shifts and rotates are very useful.  For deeply recursive benchmarks the effect of register file size and its management become important.

  - Recursive Fibonacci

Return stack size limits max result.  Is a test of call/return overhead.

  - Iterative Fibonacci

48-bit result.  Is a test of multi-word arithmetic.

  - Recursive Ackerman function

  - Optimized Ackerman function

  - Polynomial evaluation

Polynomial in memory.

  - Matrix multiply

  - FFT

  - State Machine

  - Compress/decompress ASCII text

  - Bit mashing: pack and unpack a data structure

  - Software multiply and/or divide and/or square root

  - Software floating-point

- Debug and Test Sequence

The development sequence is to implement a few instructions and implement a corresponding test.  On power-up the sequence of tests is executed with a halt at the end of any failing test.  The halt displays either the instruction address or the fault code (to an eight-bit or more LED string).  Sequencing beyond the halt is via a push button (there being a “reset” and a “continue” push button).

The initial implementation is for a Zynq “Zed Board” as this allows use of 6LUTs, quad port LUT RAM and the DSP ALU for logic as well as arithmetic.  The LS9 micro-board (Spartan-6) is also usable.

At each step record resource utilization (LUT count, etc.) and performance (F- max).

  - Hello World for FPGAs (completed 2/11/2016)

A 48-bit counter is implemented in two registers.  Only four instructions are required: ADDI, ADC, JMPsN and OUT (output most significant word to some LEDs on the development board).  Instructions (e.g. the program) encoded by CASE statement.  Utilization: 184 LUTs including 96 for quad port register file.  Fmax = 110MHz on Spartan6 -3.

  - Coding of remaining instructions (completed 2/12/2016)

Hello World loop placed in block RAM, all instructions except INsN coded.  Utilization: 1119 LUTs including 96 for quad port register file.  Fmax = 83MHz on Spartan6 -3.  Bugs: Must invert block-RAM address inputs.  Program starts at location 0x1 instead of 0x0.

  - Inferred data path, coding of condition codes (completed 3/27/2016)

Hello World loop placed in block RAM, all 30 instructions and 32 condition codes implemented.  Utilization: 493 LUTs including 96 for quad port register file.  Fmax = 107MHz on Spartan6 -3.  Bugs: Must invert block-RAM address inputs.  Program starts at location 0x1 instead of 0x0.

  - Use push button as continue (completed ?)

A 48-bit counter is implemented in two registers.  Instructions required are: ADI, ADC, BR, OUTsN and ANDI.

  - Test rig: Place continue mechanism, e.g. push button loop in a subroutine

Provide a subroutine to do a comparison test, if no match, wait for push button to continue, if a match, single step via push button if switch on.

  - Five bit register #s, return bit & CC update enable bits

  - Stack encoding for register #s

  - Two stage pipe: better control of ALUs

First stage reads the instruction; registers pointers, imm’s & control signals for 2nd stage

Multiple states for some inst., flag to indicate CCR about to change

Second stage reads the LUT RAM, does the ALU & registers/writes the results

Both stages interface to separate ports of block RAM

  - Test full instruction set (completed ? using LX9 micro-board)

Do a variety of tests on each op-code with a trap to a common routine that displays a fault code or the return address until user push button pressed and released.  The common routine de-bounces the push button.

  - Optimize data path (completed ? using Zynq ZedBoard)

Create data path multiplexors, use DSP ALU, enumerate the control signals and re-write the VHDL to generate the control signals.

Not treated herein are variants for data and/or address sizes of 12 and 48 bits.

Further expansion of the “case statement as the program” approach is covered in the paper: “Lutiac – Small Soft Processors for Small Programs”, Galloway and Lewis, 2010.  http://www.eecg.utoronto.ca/~jayar/FPGAseminar/2010/lutiac---small-processors-2.html

Instruction work is the average amount of computation done by a single instruction, typically normalized to a VAX MIPS (e.g. the Drystone MIPS or the CoreMark).  The target performance metric value is several hundred.  To a first approximation instruction work is pro-rated by word size: 8:33%, 16:67%, 24:83%, 32:100%, 48:150%, 64:200% and by average clocks per instruction.  Further pro-rating if instruction set is weak, average or strong.

PDP-11, x86, MSP430 and ARM Thumb instruction sets are considered some of the densest.

One can always add special instructions that facilitate functional or event operations.

The condition code register (CCR) contains the default value for data size.

E.g., Harvard architecture: data memory separate from program memory and may have different word size.

Multi-port register files are available in FPGAs and simple register oriented instruction sets (ROIS) are easy to implement.  A compiler for ROIS must be more complicated to do efficient register allocation.  Instruction and data memory are usually shared and implemented in Block RAM.

Historically the choice was to add a single, all purpose, read/write memory.  Instruction fetch and data load/store occurred to the same memory.  Many instructions took several memory cycles to complete.

With FPGAs and their variety of memory resources, dedicated memories for each purpose make sense.  And most instructions can compute in a single clock cycle.  Herein the major memory types are: data stack, return stack, data memory and program memory.  Add register files if co-processors are warranted.

Why four memory sections when one will do?  It’s a matter of total memory bandwidth: Here, a given instruction can interact with the four memories simultaneously.  As opposed to current architectures, famous for the “von Neumann bottleneck”, where instruction and data caches are needed to improve on the single path to a large memory and to overcome the slow speed and long access time of that large memory.