Register Oriented Instruction Set

Rois24_24 variant

James Brakefield

Contents:

Instruction Formats	Op-code List	Variations	Condition Codes	Glossary

- Instruction Formats

  - 24-bit canonical form

Register to register				XXXXXX DDDDDD RRRRRR SSSSSS

Immediate1				XXXXXX DDDDDD RRRRRR NNNNNN

Immediate2				XXXXXX DDDDDD sNNNNN NNNNNN

Immediate3				XXXXXX NNNNNN NNNNNN NNNNNN

X:    Op-code		D:    Result register

R:    1st operand register	S:    2nd operand register

N:    immediate operand	s:     N’s 2’s complement sign bit

  - 24-bit form with 5-bit register selectors

Register to register				XXXXXX rDDDDD cRRRRR xSSSSS

Immediate1				XXXXXX rDDDDD cRRRRR NNNNNN

Immediate2				XXXXXX rDDDDD sNNNNN NNNNNN

Immediate3				XXXXXX NNNNNN NNNNNN NNNNNN

X:    Op-code		D:    Result register

R:    1st operand register	S:    2nd operand register

N:    immediate operand	s:     N’s 2’s complement sign bit

r:	Return enable bit	c:     Condition code update enable bit

  - 24-bit form with one memory reference

Register to register				XXXXXX mmDDDD mmRRRR rcSSSS

Immediate1				XXXXXX mmDDDD mmRRRR NNNNNN

Immediate2				XXXXXX mmDDDD sNNNNN NNNNNN

Immediate3				XXXXXX NNNNNN NNNNNN NNNNNN

X:    Op-code		D:    Result register

R:    1st operand register	S:    2nd operand register

N:    immediate operand	s:     N’s 2’s complement sign bit

r:	Return enable bit	c:     Condition code update enable bit

mmmm: =0: register mode; /=0: {D, R or S} X {indirect, indirect++, --indirect, indexed, indexed++}

If mmmm /=0 and a second D,R or S matches the {D,R,S} choice, then multiple memory references

  - 32-bit form with memory references

Register to register			XXXXXrc0 szmmDDDD znmmRRRR oommSSSS

Immediate1			XXXXXrc1 szmmDDDD znmmRRRR NNNNNNNN

Immediate2			XXXXXrc1 oommDDDD sNNNNNNN NNNNNNNN

Immediate3			XXXXXXX1 NNNNNNNN NNNNNNNN NNNNNNNN

X:    Op-code		D:    Result register

R:    1st operand register	S:    2nd operand register

N:    immediate operand	s:     N’s 2’s complement sign bit

r:	Return enable bit	c:     Condition code update bit

szz:	memory size code (non-memory, 8, 12, 16, 24, 32, 48, 64)

mm: data ptr1, data ptr2, return ptr, frame ptr relative

n:	D/R has memory operand address

oo:  register indirect, indirect auto inc, indirect auto dec, indirect w/offset

  - Register renumbering

Assuming two data stacks, a return stack and a frame pointer (using 5 or 6-bit codes):

Relative to top of stack		xxnnnn: pointer “xx” with offset “nnnn”

Top of stack with pop/push		xx1111: pop on source, push on result

xx = fp and nnnn = 1111: reference register zero (zero source, drop result)

Assuming data stack, a return stack and a frame pointer (using 4-bit codes):

Eight codes for data stack and frame pointer, four codes for return stack.

8th code for data stack is pop/push, 8th code for frame pointer is zero register, 4th code for return

stack is pop/push.

Assuming data stack, a return stack and a frame pointer (using ~3-bit codes, 10 bits total):

Four codes for data stack and frame pointer, two codes for return stack.

4th code for data stack is pop/push, 4th code for frame pointer is zero register, 2nd code for return

stack is pop/push.

- Op-code List

Register 0 always reads as zero

PFX instruction loads 18-bits into prefix register.

Used for concatenation to a 6-bit “N” in a following instruction.

Immediate2’s are sign extended.  Immediate1’s & 3’s not sign extended.

Three address arithmetic:

ADD	Add

SUB	Subtract

ADC	Add with carry

SBC	Subtract with borrow

AND	And

ANDN	And with second operand complemented

OR		Or

XOR	Exclusive Or

MUL	Multiply, least significant 24 bits

MULU	Multiply, most significant 24 bits, unsigned

MULUS	Multiply, most significant 24 bits, signed

DIV	Divide start, (D & R) / S

FADD	Floating point add

FSUB	Floating point subtract

FMUL	Floating point multiple

FDIV	Floating point divide

Immediates (immediate 1 format):

ADDI, SUBI, ADCI, SBCI, ANDI, ANDCI, ORI, XORI,

MULI, MULUI, MULUSI, DIVI, FADDI, FSUBI, FMULI, FDIVI

Immediate (immediate 2 format):

LDI		Load register with sNNNNN NNNNNN

Immediate (immediate 3 format):

PFX	Load prefix register with 18 bits from NNNNNN NNNNNN NNNNNN

Load/Store:

LD		Load register D with memory location R+S

ST		Store register D at memory location R+S

Load/Store (immediate 1 format):

LDN	Load register D with memory location R+NNNNNN

STN	Store register D at memory location R+NNNNNN

IN		Place contents of input port R+NNNNNN into D

OUT	Transfer D to output port R+NNNNNN

Relative branches (immediate2 format):

BRCC	Branch to PC+sNNNNNNNNNNN if condition DDDDDD true

CALLR	Branch to PC+sNNNNNNNNNNN and save PC at register D

Three address branches:

CALL	Branch to address R+S, save PC+1 at register D

CALLN	Branch to address R+NNNNNN, save PC+1 at register D

JMPCC	Jump to R+S if condition DDDDDDD true

JMPCCN	Jump to R+NNNNNN if condition DDDDDDD true

- Variations

The main constraints are LUT RAM for the register file(s) and Block RAM for program and data memory.  In the larger memory space versions block RAM becomes a cache for main memory (which is external to the FPGA).  LUT RAM for a single register file is constrained to have a single write port and may have as many read ports as needed.

The variation dimensions are RTL description (pipeline-depth, data-path, control signals), instruction encoding (op-codes, modes, number of register fields), instruction bit size (24, 16 or 12-bit) and addressable unit (8-bit, 12-bit or word).

All the variations listed below assume that one or more instructions are processed per clock.  The single stage pipe versions of load and store instructions take two clocks.  Two stage pipe versions of conditional instructions take two clocks if the condition code is modified by the pervious instruction.  To the extent that they implemented; multiply, divide and floating-point instructions may take more than two clock cycles depending on the pipeline implementation and result delay interlocks.

  - Minimal implementation (MI)

Only a few instructions are implemented and the program is expressed by a case statement which allows further optimization (program and instruction logic get folded together, optimization includes removing unused instructions and modes).  Fmax very good and LUT count is minimal.

  - Basic implementation (BI)

The basic architecture is a single stage pipe, triple port LUT RAM, single port block RAM.  Instruction implementation is a simple case statement, one entry for each op-code.  Then there is a second case statement to handle the second clock cycle for memory read/write.  Program typically in block RAM.

Synthesis results in many adders and multiplexors which get optimized by translate & map. Clock speed is OK and LUT count is very high due to large number of adders and multiplexors inferred and not optimized away.

The great advantage of BI is the ease of adding or removing instructions (one can simply comment out unused instruction case entry).

  - Data path with control signals

Number of signal names increases and debugging is more difficult.

    - Inferred data path (IDP)

Adders with similar operands are combined greatly improving LUT count over BI.  Fmax also improves.  It is less difficult than planning and implementing a specific data path.

    - Explicit data path (DP)

Data path diagram created and control signals defined.  LUT count is dramatically lower than that of BI.  Good control over synthesis results.

  - Two stage pipeline

The case statement for instruction specification now generates control signals rather than data operations.  E.g., the first stage reads the instruction and generates control signals and operands for the second stage.  The second stage generates the result and condition codes.  With dual port block RAM the second stage does memory read/writes in parallel with the fetch of the next instruction.

A hazard occurs if a conditional branch instruction follows an instruction that generates condition codes.  The conditional branch instruction stalls for one or more clocks.

Another hazard occurs if a second instruction depends on the results of the previous instruction.  It is possible to eliminate this hazard by forwarding the result directly to the next instruction operand register.  Also possible is doing the register file reads in the second pipe stage.

    - With single port block RAM

A rational for a single port block RAM is if the other port is used for communication with, for instance, another processor.  Loads and store take two clock cycles unless there is an instruction buffer.

    - With dual port block RAM

This variation is expected to give the best performance metric (Instruction work per LUT per millisecond).  Use of more than two pipe stages likely to give marginal improvement to Fmax and result in increased complexity and LUT count.

  - Five bit register designators

    - Additional instruction bits

With three five bit register designators there is room for nine instruction bits.  Planned usage for the additional bits are: return bit and condition code save bit.  The third bit remains for six-bit immediates.  The condition code save bit is the MSB of the D register field and return bit is the MSB of the R register field.

    - Register designation recode

Rather than have 32 fixed location registers it is possible to provide a number of register offsets that correspond to: return address stack, data stack(s) and frame pointer.  Thus one can specify a register relative to these pointers.

Additionally it is useful to specify modification of the offset pointers.  Thus an instruction can specify stack pops (from a source stack) and pushes (to a destination stack).  This allows more IW to be done by a given instruction.

At this time there are no plans to automatically spill and restore overflow/underflow of any of the register file “stacks”.

  - Code compression

The MIPS instruction set uses up to three five bit register designators and a variable size instruction.  Here, the baseline instruction is three six bit register designators and a six bit op-code.  By being 75% of the MIPS instruction size of 32-bits, it is 33% more compact and within the realm of “densely coded” instruction sets.

  - Addressable unit size

    - 12-bit

Original design is for 12, 24 or 48 bit word sizes.  With 12-bit data addressability the functionality closely matches 8/16/32 uP.  Practical considerations may necessitate 8-bit addressability for UTF-8 Unicode (however, UTF-8 could be emulated by a UTF-12).

    - 8-bit

One can dispense with 12-bit items being addressable.  Thus instructions would be either 24 or 16-bit and memory would be 8, 16, 32 or 64-bits wide.  Memory would be in two banks with separate address adders for each bank.  Instruction readout for 24-bit instructions requires a shift circuit to align the instruction.

  - Register file (LUT RAM) word size

    - 12-bit

The intent of a 4K memory space with 12-bit words is small micro-controllers.

    - 24-bit

The intent of a 16M item memory space is embedded systems with program and data that fit into 16M.  There is a cost savings over 32-bit uP.

    - 48-bit

The intent of a 48-bit uP is all applications short of very large super-computers.  Its 48-bit floating-point is adequate for many applications.  It holds the sweet spot between 32-bit and 64-bit microprocessors (uP).

    - 16-bit

This is the traditional embedded processor category.  This uses 16-bit addresses; 8 or 16-bit data.  Many such processors have outgrown their memory space.  Efforts to enlarge the address space using segmentation or address mapping have had little long term success.

    - 32-bit

The 32-bit uP provides 4GB of address space, sufficient for most general purpose computing including full operating systems and large libraries of code.

- Implementation Decisions

  - Go for a minimal design

There are arguments for going for either the smallest possible design or going for the highest performance design.  The figure of merit is DMIPS per LUT.  Or equivalently, DMIPS per unit silicon area, which factors in the area taken by multiplier/DSP units and block RAMs.  Of course LUTs from different FPGA families are not necessarily equivalent.  The ultimate figure of merit is DMIPS per dollar using the prorated chip area.

The DMIPS per dollar and DMIPS per unit silicon area metrics also allow comparison with FPGAs with hard core processors such as the PowerPC, ARM Cortex A9 or Cortex A3, or with non-FPGA embedded processors.

Naturally some applications call for very little data RAM or instruction memory.  Other applications call for a full 32-bit processor with floating-point, caches and/or memory management.

Interestingly enough there are examples of high performance FPGA soft core processors at the minimal level, at the single processor pipelined level and at the multi-processor/barrel approach.  Thus it is possible to get a high figure of merit for any of the three design approaches.

Given that rois24_24uP is a new architecture, a simple or minimal implementation is the lowest risk and fastest to implement choice.  A minimal implementation also offers the greatest insight into LUT count versus instruction set complexity or cycle time.

  - Use a single pipeline stage

This mode of operation uses LUT RAM with asynchronous read for fetching operands.  Thus in one clock cycle an instruction is read, is decoded, the operands are read, the operation performed and the results along with the updated PC registered at the beginning of the next clock cycle.

A pipeline design with additional stages will result in a higher clock speed and greater performance.  However, there is greater complexity which is best left to when after the single pipeline stage design is up and running.  A full pipelined “barrel processor” design, with one processor thread per pipeline stage, is a straight forward way to get maximum clock rate.

  - Do not use a data path

For a simple processor implementation with single pipeline stage, the instruction decode is a large case statement with each case condition expressing the instruction calculation.

Normally one would code the data path and have the instruction decode generate the control signals for the data paths.  For sizeable instruction sets, this saves LUTs and results in a faster clock cycle.

An intermediate data path form is to code adders for common sets of operands and invoke one of the sums as needed in the large case statements.  This has the effect of inferring a data path with a reduced number of multiplexors and a reduced number of adders.  In addition: the map, place and route tools have greater freedom than when a fixed data path is coded.

  - Condition code register (CCR)

The conditional branch instructions specify a condition code.  There are arguments for and against condition code registers.  Here, it was decided to stay with a full condition code register.  Thus, the CCR contains carry, signed overflow, zero and sign for each accumulator.  Also included is a least significant bit (even/odd).  And four bits for: all ones, all zeroes for both the floating point exponent and the floating point mantissa fields (the overall zero bit becomes redundant).

Interrupt enable and Interrupted bits also present.

  - Condition code register contains the ALU result

It takes additional logic to generate the some condition code bits from the ALU result (such as the zero bit).  Instead save the entire ALU result and do the additional logic during the processing for conditional branches.  To save the CCR to a register or memory, the ALU result is reduced.  To restore the CCR, the all zeros and all ones bits are “unreduced”.  CCR saved/restored via IO instructions.

  - Use block RAM for instruction memory

The smallest block RAM allows for several hundred instructions, a reasonable number.  Reading instructions from a synchronous RAM is compatible with a single pipe stage implementation.

  - Using instruction memory for data memory

Given that instruction memory is block RAM and therefore has synchronous reads, and supports dual port operation (first port for instruction reads, second port used for data RAM read/write), the single stage pipe design does not allow a block RAM data read to occur in the same cycle as the instruction read.  However a block RAM data write can occur at the end of the cycle.

Given that it takes one cycle to post the read address and the next cycle to read from the block RAM, a variety of arrangements can be made for data read: Use two instructions, one to post the read address and the next instruction to process the data.  Or use a memory pointer register (in IO space) which auto increments after each data read and fetches data at the next location.

  - Divide and Floating-point Instructions

One possibility is to have the divide and floating-point instructions start a coprocessor and use an input instruction later to read the result.  The input instruction would block until the result is available.

  - Interrupts

Interrupts shall push sufficient information (program counter, condition code register, etc.) to transparently resume the program at the point of interrupt.

- Condition Code List

Standard codes:

BRZ/BRNZ		Branch zero/non-zero

BRCS/BRCC		Branch carry set/clear

BRMI/BRPL		Branch minus/plus

BRVS/BRVC		Branch overflow set/clear

BRLE/BRGT		Branch less than or equal/greater than

BRGE/BRLT		Branch greater than or equal/less than

BRLS/BRHI		Branch unsigned result low or same/high

Non-standard codes:

BROD/BREV		Branch odd/even

BR1/BRN1		Branch equal to one/not equal

BRM1/BRNM1	Branch equal to minus one/not equal

BRM2/BRNM2	Branch equal to minus two/not equal

BR01/BRN01	Branch equal to zero or one/not equal

BR0M1/BRN0M1	Branch equal to zero or minus one/not equal

BR01M1/BRN01M1	Branch equal to zero, one or minus one/not equal

BR01M12/BRN01M12 Branch equal to zero, one, minus one or minus 2/not equal

Alternate names:

BRA/NOP		Branch always/never (same as CALLR with D=0 and sN=0 or 1)

BREQ/BRNE		Branch equal/not equal (same test as BRZ/BRNZ)

BRLO/BRHS		Branch unsigned low/high same (same test as BRCC/BRCS)

- Abbreviations and Glossary

BI		Basic implementation: soft core uP implemented with a single pipe stage using inferred adders, multiplexors and control logic.  The RTL implements the entire instruction set in a single case statement.

Block RAM	Usually available with two independent read/write ports.  Word size on each port independently specified (18K bit RAM may offer 18-bit by 1K words, 9-bit by 2K words, 4-bit by 4K words, etc.).  Reads are synchronous in that address is clocked in and data appears memory access time later.  Usually there are a variety of options including registered data out.

CCR		Condition code register: The results of a logic or arithmetic operation are summarized in a small number of bits, typically: carry, zero, sign and overflow.  The condition code bits are saved in a register for use by following conditional branch instruction(s).

CoreMark	EEMBC benchmark for embedded processors.  CoreMarks per MHz are an indication of instruction work per clock cycle.  Values range from 0.1-.2 for 8-bit processors, ~.5 for 16-bit processors to as high as 5 for processors that issue more than one instruction per clock.

DP		Data path: soft core uP implemented in RTL such that the adders and multiplexors are explicit with explicit control signals.  Often there is a system diagram showing these elements along with the memory units.  See IDP.

DMIPS		Dhrystone MIPS.  DEC VAX 11/780 has a DMIPS of 1.0.  Not as accurate as CoreMark.

DSP unit	Some FPGA families contain multiply units.  In more recent families there is also an accumulator.  These units are optimized for high speed signal processing.

FPGA		Field Programmable Gate Array: a chip composed of a sea of LUTs, Block RAM, DSP units and other units interconnected by programmable wiring.

IDP		Implicit data path: soft core uP implemented in RTL such that adders and multiplexors with identical inputs are shared.  See DP.

IW		Instruction work: is the average amount of computation done by a single instruction.  Herein, to a first approximation instruction work is pro-rated by word size: 8:33%, 16:67%, 24:83%, 32:100%, 48:150%, 64:200% and by average clocks per instruction.  Further pro-rating if instruction set is weak or strong.

LUT		Lookup table: basic component of an FPGA, typically four to six inputs and one or two outputs.  Other names are “tile” (~2.5 tiles equivalent to one 4LUT) and ALUT.  (~1.5 4LUTs equivalent to one 6LUT or one ALUT).

LUT RAM	Register file built from FPGA LUTs, may also be a small block RAM.  Many (most) LUT RAMs offer asynchronous reads and a registered write.

MI		Minimal implementation: simplest possible version of a soft core uP using a minimum number of instructions and memory.  Herein, enough instructions are implemented in RTL and used in a short program to implement a long binary counter and output some of its bits to drive LEDs.

MIPS		Has two meanings: “millions of instructions per second” and the name of a RISC uP vendor.

RISC		Designator for uP with instructions containing an op-code, two source and one destination register numbers.  Main memory accessed via load and store instructions.

RTL		Register transfer language: typically code written in Verilog, VHDL or System Verilog that specifies the registers and the logic between the registers.  FPGAs typically use D flip-flop registers.

Soft Core uP	The implementation of a processor using the resources within an FPGA.  Usually coded in a RTL.

uP		Microprocessor

UTF-8		The dominate representation for Unicode character strings.  UTF-8 uses a variable length code consisting of 8-bit bytes.  The 95 ASCII characters are encoded in a single byte.

Further expansion of the “case statement as the program” approach is covered in the paper: “Lutiac – Small Soft Processors for Small Programs”, Galloway and Lewis, 2010.  http://www.eecg.utoronto.ca/~jayar/FPGAseminar/2010/lutiac---small-processors-2.html

Instruction work (IW) is the average amount of computation done by a single instruction, typically normalized to a VAX MIPS (e.g. the Drystone MIPS or the CoreMark).  The target performance metric value is several hundred.  Herein, to a first approximation instruction work is pro-rated by word size: 8:33%, 16:67%, 24:83%, 32:100%, 48:150%, 64:200% and by average clocks per instruction.  Further pro-rating if instruction set is weak or strong.

PDP-11, x86, MSP430 and ARM Thumb instruction sets are considered some of the densest.

The author’s opinion is that it is better to have a uniform instruction set across various word size register files.