STOC1132: 32-bit stack machine in the “key” of PDP-11

James C. Brakefield 2021

July 6, 2021

Op-code constants for ASTOC11_16MIN

On before subroutine entry setup new frame pointer, save previous FP, save restore SP

On return restore FP also sets trimmed SP.

July 4, 2021

The two unused codes for M = stack reference and data size not largest can be repurposed for alternate offset sizes for M = stack reference + offset.

June 30, 2021

Version with 9-bit bytes (stoc1136 or stoc1118) has almost all the desired features;

Return bit, 64X op-codes, choice of three data sizes and five address modes

Three data sizes and five modes leave one code for immediate and a five bit stack reference.

Today:

Two byte only ISA:		ssmmddnnn	rPRxxxxxx	64*2*2*2*3*5*4*8

Immediate variant:		1111nnnnn	rPVxxxxxx	64*2*2*2*3*5*32

Two byte control:	 	nnnnnnnnn	nnn000xxx	6 of 64*4096

S		Three data sizes (9, 18 and 36 bits)

M		Five address modes, no modifications to stack pointers

reg-file(D+n)				Operand/result is the stack element at D+n

mem(reg-file(D+n))			Operand/result location is given by stack element

mem(reg-file(D+n)++)		Increment stack element by data size after useage

mem(--reg-file(D+n)) 			Decrement stack element by data size before useage

mem(reg-file(D+n)+18-bit-offset)	Memory location is stack element + offset

D is one of either: data stack pointer, return stack pointer, frame pointer and global area

Offset is 18 bits only?  Or comma coded 8x bits?

Built-in registers: Top of data stack, Top of return stack (AKA PC), Residue register, Status/Interrupt enable register, IO ports

Single byte instructions: RTN, BRK, NOP, DUP, DROP, SWAP, LOOP1 (count down) LOOP2 (count up to limit), LOOP3 (count up to limit with delta), While loop.  Start loop (four variants) saves next PC as head of loop and also zero to four items from the data stack (count, limit, delta) plus current value of count.  Small constants?

(6) Two byte control instructions: Call, BR, BRZ, BRNZ, LDI and Prefix additional 12 “n” bits

The one and two byte instructions have 64 op-codes available:

(23) Load/Store(3), Add, Sub, Mul, Div And, Or, Xor, Cmp, Test, Fadd, Fsub, Fmul, Fdiv, Fcmp, Extract(2), Insert, ASR, LSR, LSL

X	op-code bit

r	subroutine return enable bit (pop return stack into PC)

M	mode (stack ref, indirect ref, indirect ++ ref, indirect – ref, indirect+offset ref)

DN	register file pointer select and offset to pointer

P	push/pop TOS

If results to TOS (R is 0), P is enable to replace TOS with results otherwise push TOS

If results to stack (R is 1), P is pop TOS enable

R	results to stack reference or to TOS

V	Swap immediate with other operand

June 29, 2021

The idea appeared in a flash late at night, after comp.arch discussion on a PDP-11 modernization, some discussion on code density and Brad Eckert’s Chad version of J1 (on comp.lang.forth):  Merge the minimal set of PDP-11 addressing modes into my alt_stkoc32_8 design by pruning the ISA to one byte stack operations and two byte op + stack reference instructions.  Would be nice if one can find a bit for the return enable?  Looks like data size field will need to go?  Also how to include immediates?  How to do indexed load/store?

Today:

Two byte only ISA:		smmddnnn	sPRxxxxx	32*2*2*3*5*4*8

Immediate variant:		111nnnnn	1PVxxxxx	32*2*2*3*5*4*8

Two byte control:	 	nnnnnnnn	nnx000xx	6*1024

One byte stack mode instructions:  		xxx0000x	2*1024

Unary instructions?		xxxxxxxx	xxx00010

Ternary instructions?		xmmddnnn	xmmddnnn	xxx00011

Other instructions are ad-hoc formats from five remaining unused op-codes

X	five op-code bits

P	push/pop TOS enable

R	results to stack reference enable

S	data size (three data sizes, byte and long are most common, 3 sizes suffice?)

M	mode (stack ref, indirect ref, indirect ++ ref, indirect – ref, indirect+offset ref)

DN	register file pointer select(D) and offset to pointer(N)

D:	SP, RSP, FP & TP (data stack, return stack, frame & thread-global)

N:	0-6: offset from register file pointer, 7: push/pop at offset zero

N	Five bit immediate value or immediate size

Three data sizes and five address modes leaves one code for five bit immediate operand?

V	Swap immediate with other operand, or do a return?

No single byte instructions

Except RTN, BRK, NOP, DUP, DROP, SWAP, LOOP1 & LOOP2?

(6) Two byte control instructions: Call, BR, BRZ, BRNZ, LDI and Prefix 10x bits

The two byte instructions have 32 op-codes available:

(23) Load/Store(3), Add, Sub, Mul, Div And, Or, Xor, Cmp, Test, Fadd, Fsub, Fmul, Fdiv, Fcmp, Extract(2), Insert, ASR, LSR, LSL

Alternate encoding of DN: only four offset values for Thread and RSP, six offset values for TOS and seven offset values for Thread allowing eight values for M (that is three codes for indirect+offset, e.g. three offset sizes)?

June 28, 2021

Use RISC’s register file via data stack, return stack, frame and base/thread register file pointers

Use five PDP-11 addressing modes

Use Brakefield’s ISA methodology to devise a compact stack machine ISA

Postpone stack spilling and refilling until “min” version working

Little endian

One byte stack mode instructions:  			0XXXXXXX

Two byte instructions: 		rmmddnnn	1PRXXXXX 5*4*6 + 8

Two byte instructions: 		rmmmddnn	1PRXXXXX 8*4*4

Two byte with immediate: 		rmmddnnn	0PVXXXXX

Two byte control:	 		nnnnnnnn	0XXXXnnnn

Other instructions:			xxxxxxxx … 	xXXXXXXX

X	op-code bit

R	subroutine return enable bit (pop return stack into PC)

M	mode (stack ref, indirect ref, indirect ++ ref, indirect – ref, indirect+offset ref)

DN	register file pointer select and offset to pointer

P	push/pop TOS enable

If results to TOS (R is 0), P is enable to replace TOS with results otherwise push TOS

If results to stack (R is 1), P is pop TOS enable

R	results to stack reference or to TOS

V	Swap immediate with other operand

MMDDnnn encodes five memory reference modes, four register file pointers and register file offsets.  The 128 codes, 5*4*25+3, sp & frame get 8 offsets, rsp gets five offsets, and base or thread gets four offsets.  One of the data stack and one of the return stack offsets is pop/push mode access to top.  The remaining three of 128 codes are immediate, relative and absolute modes.

The visible registers are the top of data stack (TOS), top of return stack (PC), the residue register (MY 66000 carry mechanism), a status/control register (interrupt processing and status bits not contained in the result or residue) and a prefix register for accumulating long immediates.

TBD the exact division between one and two byte instructions

TBD the handling of immediate operands, most likely by reserving some mmddnnn codes

The two byte instructions have 32 op-codes available:

(26) Load/Store(6), Add, Sub, Mul, Div And, Or, Xor, Cmp, Test, Fadd, Fsub, Fmul, Fdiv, Fcmp,

Extract(2), Insert, ASR, LSR, LSL

(6) Two byte control instructions: Call, BR, BRZ, BRNZ, LDI and Prefix 12 bits

Leaves 160 one byte stack mode op-codes