Project Log for alt-11

Augmenting PDP11 for more & bigger registers, more address space

James Brakefield

4/23/2025	Culled entries from uP_log…docx relating to alt-11

There is many more files and log entries on alt430 and alt780 than on alt-11.

Current preference is to reduce number of addressing modes and thereby increase the number of registers.  Us as much of the original PDP-11 ISA as possible, e.g. provide as much compatibility between the PDP-11 instructions and all-11 instructions.  Repurpose the PDP-11 floating-point instruction allocation to an escape to 24-bit instruction thereby supporting more data types and more data sizes. The alt-11 16-bit instructions provide for two data sizes (byte and max), five addressing modes, 12+ registers and 2’s complement arithmetic.  Variable length immediate values are preferred.

Log entries for STOC11_16 and ASTOC11 are stack machines using alt-11 addressing modes.

04/19/2025	DSO8U_16X_def	no alt-11 doc files

Located at _jcb_uP, and nowhere else?

STOC1132_210706.docx located in C:\_jcb_uP\_2012_CA_directories\astoc is stack machine

Original MMMMMM mixture:

Five memory addressing modes on twelve registers (60 codes)

reg, (reg), (reg++), (--reg), (reg + delta)

Address modes on special registers (four codes)

Immediate, Direct, Stack pointer auto inc/dec, Frame pointer + Immediate

Current mixture: change (reg) to (reg + delta8) and (reg + delta) to (reg + delta_max)

Avoids variable length immediate value formatting, such as comma coding

Special registers: PC as a destination, PC, SP & FP in addition to 12 other regs?

Use of inferred index register instead of auto-increment & auto-decrement:

reg, (reg+imm8), (reg+scaled_index), (reg+imm_max), (reg+scaled_index+imm_max)

Residue register as part of register file?

uP_log_161026.doc goes all the way back to 2004 and includes DSO (Data Size Orthogonal)

uP_log 09/21/2021, 06/11/2021, 01/18/2021 have links to usenet/comp/arch where alt-11 is discussed (by Mitch Alsup & others), e.g. using floating-point instruction space for expansion

11/08/2022	legacy uP ISA issues

LCX11: Twelve registers, five modes and 4*12 unused codes become Two immediate sizes (per above), an index register reference and a residue register reference.  Effectively 14 registers and immediates

09/21/2021	“Alt” uP list

Alt-11		PDP11 variant with 12 registers and four data sizes

Floating-point instruction allocation used as escape to 24-bit instructions

16-bit instructions remain, as is, and supporting two data sizes

Addressing modes are R, (R++), (--R), (R+byte-disp), (R+word-disp)

Remaining four codes (64-60) are imm, relative, absolute & ?

07/08/2021	Generic ASTOC11:

ASTOC11 stands for Accumulator/Stack oriented computer with PDP-11 addressing modes.

An ASTOC11 ISA has the following instruction fields:

Instruction length can be implied or encoded by a length field.  Typically one byte (dual stack op-code), two byte (op-code and stack reference) and three bytes for three operand instructions with two stack references.  The top of data stack (TOS) is usually one of the operands.

Typically there is a five or six bit op-code.  Additional op-code bits on three byte instructions.

A replace bit to indicate result goes to the stack reference rather than to the TOS.

A push/pop bit which can push the result onto the data stack or pop the data stack if the result goes to the stack reference.

A return bit to enable a subroutine return in addition to the other work of the instruction.

A data size field is needed.  Four sizes are preferred and two sizes on reduced implementations.  A stack reference mode field uses a subset of the PDP-11 addressing modes.  Four modes is the minimum and eight or more is possible.

A stack reference field which contains a two bit stack indicator and a two or more bit offset from the associated stack pointer.  The stacks are: the return stack, the data stack, a frame pointer and a global/thread/base area.  The frame area should support 16 frame entries, the data stack eight entries, the return stack four and the global area also four.

Some combinations of data size and addressing mode may indicate that the stack reference field is an immediate value operand.  In that case most of the stack reference field will encode small constants and the remaining codes used to indicate the number for following bytes containing the full immediate value.

The individual ISA layout is highly dependent of the byte size (8, 9, 10 or 12 bits), the degree to which a full featured ISA is desired, and the number of codes in each instruction field.  Combining instruction fields is possible, say, three data sizes and five addressing modes.  Eliminating fields is also possible, for instance the return enable bit.  In general, a two byte instruction will support an op-code and a stack reference.  Three byte instructions are very useful for three operand instructions.

The intent of the architecture is to support high code density that is compatible with FPGA and RISC micro-architecture; including multiple instruction issue, pipelining and out of order instruction completion.  It is expected that a FPGA implementation will use a multi-ported LUT RAM for the stack areas offering single clock per instruction execution.  Most likely the TOS, the PC and the residue register (RR) will be virtual in the sense that a DFF implementation is present and virtually the TOS is part of the data stack, PC is part of the return stack and Residue is part of the global area.

The RR captures signed and unsigned overflow, upper half of a multiply operation, remainder from a divide and round-off error from floating-point operations.  Any instruction can access the residue.  Three operand instructions are ideal for utilizing the RR.

At this point, issues of stack spill and refill, interrupt handling, memory caching and virtual memory management are not specified.

07/05/2021	STOC11_16: 16-bit stack machine in the “key” of PDP-11

There are too many ways to do the ISA on a stack machine with PDP-11 addressing modes.  The best way to fit into 8X instruction sizes appears to be for the 16-bit instructions not have addressing modes.  Use 24-bit instructions for a full featured instruction.

Currently think is to have a skinny version with small SM and DN fields that fit into 16-bits.  And have “pro” version with full size SM & DN fields that fits into 20+ bits.

Choices:

Op-code bits: five and up

P & R bits: always present

Return bit: optional as can always be done by an 8-bit instruction

DN bits: four to six bits, Livermore Loops can utilize 16 frame values

S: two, three or four data sizes

M: Five to eight codes

TBD: potential for multiple instruction issue?

SAOC16_8	smmddnnn rPRxxxxx		no register indirect

07/04/2021	STOC11_18: 18-bit stack machine in the “key” of PDP-11

Switched to 9/18/27-bit instructions:

Data can still remain 8X

Leaves room for return bit and sufficient 27-bit three operand instructions.

xxx00000x  one byte inst.

nnnnnnnnn nnn000xxx  2 byte inst with 12-bit imm.

ssmmddnnn rPRxxxxxx	 generic 2 byte inst.

1111nnnnn rPVxxxxxx	 2 byte inst. With imm. value

xxxxddnnn ssmmddnnn rPRxxxxxx	 three byte inst.

When M is stack reference, size is register size and unused S bit codes indicate size of offset field.  So the 16 SM codes are oversubscribed.

Currently, 16-bit instructions are a subset of 18-bit instructions.  Even then the SM field could be expanded to five bits.  So, there are many ways to select and encode S, M, D and N fields with resulting instructions of 16 or more bits.

06/28/2021	STOC1132: 32-bit stack machine in the “key” of PDP-11

Use RISC’s register file via data stack, return stack, frame and base/thread register file pointers

Use five PDP-11 addressing modes

Use Brakefield’s ISA methodology to devise a compact stack machine ISA

Postpone stack spilling and refilling until “min” version working

Little endian

One byte stack mode instructions:  			0XXXXXXX

Two byte instructions: 		rmmddnnn	1PRXXXXX 5*4*6 + 8

Two byte instructions: 		rmmmddnn	1PRXXXXX 8*4*4

Two byte with immediate: 		rmmddnnn	0PVXXXXX

Two byte control:	 		nnnnnnnn	0XXXXnnnn

Other instructions:			xxxxxxxx … 	xXXXXXXX

X	op-code bit

R	subroutine return enable bit (pop return stack into PC)

M	mode (stack ref, indirect ref, indirect ++ ref, indirect – ref, indirect+offset ref)

DN	register file pointer select and offset to pointer

P	push/pop TOS enable

If results to TOS (R is 0), P is enable to replace TOS with results otherwise push TOS

If results to stack (R is 1), P is pop TOS enable

R	results to stack reference or to TOS

MMDDnnn encodes five memory reference modes, four register file pointers and register file offsets.  The 128 codes, 5*4*25+3, sp & frame get 8 offsets, rsp gets five offsets, and base or thread gets four offsets.  One of the data stack and one of the return stack offsets is pop/push mode access to top.  The remaining three of 128 codes are immediate, relative and absolute modes.

The visible registers are the top of data stack (TOS), top of return stack (PC), the residue register (MY 66000 carry mechanism), a status/control register (interrupt processing and status bits not contained in the result or residue) and a prefix register for accumulating long immediates.

TBD the exact division between one and two byte instructions

TBD the handling of immediate operands, most likely by reserving some mmddnnn codes

The two byte instructions have 32 op-codes available:

(26) Load/Store(6), Add, Sub, Mul, Div And, Or, Xor, Cmp, Test, Fadd, Fsub, Fmul, Fdiv, Fcmp,

Extract(2), Insert, ASR, LSR, LSL

(6) Two byte control instructions: Call, BR, BRZ, BRNZ, LDI and Prefix 12 bits

06/11/2021	comp.arch on alt/revised PDP-11

# https://groups.google.com/g/comp.arch/c/U-RGNrDhmnI	PDP-11-like ISA

# https://groups.google.com/g/comp.arch/c/o-a7wowxJUk	Squeezing Those Bits: Concertina II

Discussions on several ways to modify PDP11 into 64-bit machine.

Mitch wants to stick with 8 registers and use (Rn+offset) to reference a cache of 64-bit values.

I want to add additional data sizes and room for additional op-codes

Mitch seems to want to not use byte/word instruction bit.  I suggested using 17XXXX floating-point op-code space as an escape to a longer instruction format.  Original two RM fields stay put.

Discussion on dropping some addressing modes to allow disp16, disp32 and disp64 offsets.

Or adding other features including indexing?

A VAX mode is possible with an escape to a 32-bit instruction format having three complete register and mode fields (RM) and 16 registers.  Not much interest in increasing number of registers.  (VAX supports disp8, disp16 and disp32)

A variation of my previous alt-vax is to use a 32-bit instruction with three complete RM fields and 16 or 32 registers (offers simple decode with up to three memory references per instruction as opposed to previously documented 24-bit format with single memory reference).

So, currently there is interest in doing away with two addressing modes: @(--Rn) and @(Rn+offset).  Possibly @(Rn++) as well (which I want to keep for ITC Forth and Fortran coding?

Discussion on whether to use instruction bit to enable update of status register.  Most PDP11 instructions update the SR, including the MOV instruction.

01/18/2021	Alt PDP-11

Eliminate the unused address modes: @(--R) and @(R+disp) leaving six address modes.  Six goes into 64 ten times.  Remaining four codes used for imm, abs, relative and PC itself.  Nets eleven registers instead of just eight (the four necessary modes for the PC register and six modes for ten other registers).  @(R++) is used (needed?) for parameter address lists following subroutine call instructions in Fortran and for indirect threading.

It would be nice to support larger address space and additional data sizes. One approach would be to repurpose one of the floating-point op-code spaces as an escape to a 24 bit instruction.  Also would be useful to go with variable length immediate fields.

05/30/2018	Alt-11

Modify to 12 registers with five addressing modes + imm, abs, relative & frame.

Four sections of unused op-code space (after RORB,ROLB,ASRB,ASLB moved to 075400-075777 or FADD,FSUB,FMUL,FDIV moved to 106400-16777) used as prefix codes (e.g. 32-bit instructions) at 007xxx, 076xxx, 106xxx & 107xxx  providing additional op-code bits and a third register/address field and four data sizes.  Two default sizes are 8-bit and register size.  Otherwise ISA stays the same!

PDP-11 has R, a register reference, (R) a register indirect reference, (R++) a register indirect reference with post auto-increment, (--R) a register indirect reference with pre memory access data size decrement, (R+offset) a register indirect reference with a displacement field suffixed to the instruction and three other modes not considered here.