TROC24_8234_ISA_250515.docx

Tagged Register Oriented Computer

James Brakefield

Instruction set architecture for 24-bit Register Oriented Computer supporting 16, 24 and 32-bit instructions on thirty-two 24-bit registers.  It is a 24-bit rendition of the full TROC ISA 8-bit aligned instructions and data.  There are four sections:

- 16-bit instructions using full encoding of immediate values, that is small values within the five bit register field, 12-bit values using an additional byte and larger values using an additional two to four bytes.  The same encoding used for 24 and 32-bit instructions.

- 24-bit instructions supporting the majority of the instruction set.

- 32-bit instructions providing three operand instructions and three additional op-code bits.

- 8-bit instructions from the 32-bit instruction code space for Trap, Breakpoint, Word align and Cache line align.

- Remaining encoding space reserved for 40 and 48-bit instructions.

The 32-register 24-bit register file is augmented with two “tag” bits for data type and two “tag” bits for additional exponent and four “tag” bits for additional mantissa on each register.  The data types are: unsigned, signed, floating-point and a fourth data type which could be a second floating-point format or fixed point logarithm or something entirely different.  The program counter (PC) and a “residue” register are part of the register file (registers 31 and 28) but implemented as distinct registers.  The residue register captures the carry from unsigned add/subtract, the signed carry/overflow from signed add/subtract, the upper half of multiply and the remainder from divide.

Floating-point arithmetic and conversion between memory and register formats not yet formally defined.  Expectation is that several float formats will be supported.  The other “floating-point” type may instead be fixed-point logarithm or pointers for variable length arithmetic.

The instruction formats are:

16-bit instruction specifies N or S

sssss ddddd xxxxx1

nnnnn ddddd xxxxx1

24-bit instruction has an immediate enable bit

rrrrr 0 xx sssss ddddd xxxx10

nnnnn 1 xx sssss ddddd xxxx10

32-bit instruction has three additional op-code bits

ttttt xxx rrrrr 0 xx sssss ddddd xxxx00

nnnnn xxx rrrrr 1 xx sssss ddddd xxxx00

The X are the op-code bits.  The least significant instruction bits are arranged so that an all zero instruction is a trap, e.g. the processor has encountered an invalid instruction.

D, S, R and T are five bit register selectors.

N is a five bit immediate value with multiple interpretations.

For TROC24 and above, it indicates a value from -4 to 7, a 12-bit value via an additional instruction byte, or indicates the number of additional bytes containing the value.

Using two port register file

16-bit Op-code chart by binary encoding

LSBs:	  xxx001		  xxx011      xxx101   xxx111

MSBs		 n	  N		 n	  N		 S	  n	  N

000xx1	INN	 LDN		SHFTN EXTCTN	ADD	 ADDI ibid

001xx1	OUTN	 STN		FUNCT INSRTN	SUB	 SUBI ibid

010xx1	BSRN	 ibid	AOBN	 ibid	MUL	 MULI ibid

011xx1	MOV   S->D	SOBN	 ibid	DIV	 DIVI ibid

100xx1	BRMN	 ibid	LDI	 ibid	AND	 ANDI ibid

101xx1	BRPN	 ibid	LDIS	 ibid 	OR	 ORI  ibid

110xx1	BRZN	 ibid	LDIF  ibid	XOR	 XORI ibid

111xx1	BRNZN ibid	LDIF2 ibid	CMP	 CMPI ibid

8-bit Instruction descriptions:

Op-code     R?	Mnemonic	Precise description (R? column indicates residue update)

“00000000”  N	p1TRAP		Illegal instruction trap, save current PC in internal save register

Branch to illegal instruction handler location

“01000000”  N	p1BKPT		Debug trap, save current PC in internal save register

Branch to debug handler location

“10000000”  N	p1ALGNW	Skip to next word aligned instruction.  Word size is implementation

defined.

“11000000”  N	p1ALGNC	Skip to next cache line or DRAM line aligned instruction

16-bit Instruction descriptions:

If the destination register is residue, the normal residue result is not recorded.

Op-code    R?	Mnemonic	Precise description (R? column indicates residue update)

“000001”   N	p2INN		Store value of input port N into D if N is in the range from -4 to +7.

Negative N signifies internal ports, positive N external ports. D’s tag bits set to unsigned.

“000001”   N	p2LDN		When n is outside the range from -4 to +7 load D from absolute memory

location N.  D’s tag bits unchanged, for floats memory value is converted to register format

“001001”   N	p2OUTN	Place D into output port N if N is in the range from -4 to +7.

Negative N signifies internal ports, positive N external ports. D’s tag bits set to unsigned.

“001001”   N	p2STN		When n is outside the range from -4 to +7 store D into absolute memory

location N.  For floats D’s value is converted to memory format

“010001”   N	p2BSR		Branch to PC + N, save address of next instruction in D

“011001“   N	p2MOV		Move register S to D, tag bits moved as well

“100001”   N	p2BRZN	Branch to PC + N if D is zero

“101001”   N	p2BRNZN	Branch to PC + N if D is non-zero

“110001”   N	p2BRPN	Branch to PC + N if D is positive (MSB is zero)

“111001”   N	p2BRMN	Branch to PC + N if D is negative (MSB is one)

“000101”   Y	p2ADD		Add S to D, if types differ is currently unimplemented trap

Residue register captures the carry, for signed ADD, carry can be either zero, +1 or -1

For floating-point ADD residue register captures round-off error

Floating-point ADD makes use of and may modify the exponent tag bits

“001101”   Y	p2SUB		Subtact D from S, if types differ is currently unimplemented trap

Residue register captures the carry, for signed SUB, carry can be either zero, +1 or -1

For floating-point SUB residue register captures round-off error

Floating-point SUB makes use of and may modify the exponent tag bits

“010101”   Y	p2MUL		Multiply D by S and store in D, if types differ is unimplemented trap

Residue register captures upper half of product for signed or unsigned multiply

For floating-point MUL residue register captures round-off error

Floating-point MUL makes use of and may modify the exponent tag bits

“011101”   Y	p2DIV		Divide D by S and store in D, if types differ is unimplemented trap

Residue register captures remainder for signed or unsigned divide

For floating-point DIV residue register captures a floating-point remainder

Floating-point DIV makes use of and may modify the exponent tag bits

“100101”   N	p2AND		Logical AND between S and D to D irrespective of data types

“101101”   N	p2OR		Logical OR between S and D to D irrespective of data types

“110101”   N	p2XOR		Logical exclusive or between S and D to D irrespective of data types

“111101”   ~Y	p2CMP		S subtracted from D and placed in residue register

“000111”   Y	p2ADDI		Add I to D, I converted to register format of D before add

Residue register captures the carry, for signed ADDI, carry can be either zero, +1 or -1

For floating-point ADDI residue register captures round-off error

Floating-point ADDI makes use of and may modify the exponent tag bits

“001111”   Y	p2SUBI		Subtract D from I, I converted to register format of D before SUB

Residue register captures the carry, for signed SUBII, carry can be either zero, +1 or -1

For floating-point SUBI residue register captures round-off error

Floating-point SUBI makes use of and may modify the exponent tag bits

“010111”   Y	p2MULI		Multiply D by I and store in D, if types differ is unimplemented trap

Residue register captures upper half of product for signed or unsigned multiply

For floating-point MULI residue register captures round-off error

Floating-point MULI makes use of and may modify the exponent tag bits

“011111”   Y	p2DIVI		Divide D by I and store in D, if types differ is unimplemented trap

Residue register captures remainder for signed or unsigned divide

For floating-point DIVI residue register captures a floating-point remainder

Floating-point DIVI makes use of and may modify the exponent tag bits

“100111”   N	p2ANDI		Logical AND between I and D to D irrespective of data types

“101111”   N	p2ORI		Logical OR between I and D to D irrespective of data types

“110111”   N	p2XORI		Logical exclusive or between I and D to D irrespective of data types

“111111”   ~Y	p2CMPI		D subtracted from I and placed in residue register

“100011”   N	p2LDI		load unsigned immediate, set tag bits to unsigned

“101011”   N	p2LDIS		load signed data immediate, set tag bits to signed

“110011”   N	p2LDIF		load float data immediate, convert N to register format

“111011”   N	p2LDIF2	load float2 data immediate, convert N to register format

“010011”   N	p2SOBN	Decrement D and branch to PC + N if D not zero

“011011”   N	p2AOBN	Increment D and branch to PC + N if D not zero

“000011”   Y	p2SHFTN	If N is in the range from -4 to +7 shift or adjust exponent by signed N.

For signed or unsigned D shift left or right per sign of N.  For floating-point add signed N

to exponent.

“000011”   N	p2INSRTN	If N is not in the range from -4 to +7 insert field from residue into D per

N (uu000wwwww0mmmmm)

“001011”   ~Y	p2EXTCTN	If N is not in the range from -4 to +7 extract field from D and place in

Residue per N (uu000wwwww0mmmmm).  Operation is irrespective of tag bits.  U is data

type of the result.  W is the width of the field extracted.  If W is “00000” then width is

24.  M is the starting bit of the field.  For unsigned, bit field extracted from D rotated

right by M bits. For signed data type, bit field sign extended to register width.

“001011”	p2FUNCT	If N is in the range from -4 to +7 do a one operand function fn(D) => D

“001011”	“11111”	p2CVTUS	If n is “00000” convert D to unsigned

“001011”	“11110””	p2CVTS		If n is “00001” convert D to signed

“001011”	“11101”	p2CVTFLT	If n is “00010” convert D to float

“001011”	“11100”	p2CVTFLT2	If n is “00011” convert D to float2

“001011”   N	“00000”	p2SETUS	If n is “00000” set D type to unsigned

“001011”   N	“00001”	p2SETS		If n is “00001” set D type to signed

“001011”   N	“00010”	p2SETFLT	If n is “00010” set D type to float

“001011”   N	“00011”	p2SETFLT2	If n is “00011” set D type to float2

“001011”	“00100”	p2ABS		If n is “01000” take the absolute value of D

“001011”	“00101”	p2NABS	If n is “01001” take the negative absolute value of D

“001011”   Y	“00110”	p2TAG		If n is “01001” place D’s tag bits into residue

“001011”	“00110”			Room for one more function

24-bit Instruction descriptions:

For TROC24 with byte addressing and 24-bit instructions the index register R is multiplied by the data byte size and added to the base register S. If R is specified as the PC (register 31) a value of zero is used.  It is OK to use the PC as a base register.  Loading the PC (i.e. D is register 31) causes a branch.

The 24-bit instructions use the full TROC interpretation of N. If N is specified instead of R then N is added to the base register S without scaling.

For TROC24 the 24-bit instructions that load/store 32-bit data are repurposed for load/store of tagged values.

The 24-bit instructions split the op-code into two parts: Bits 5 down to 0 contain xxxx01 where the x’s are the LSB op-code bits.  Bits 17 down to 16 contain the most significant two bits.  When “00” it is a load instruction.  When “11” the op-code space is reserved for 40-bit instructions.

Op-code     R?	Mnemonic	Precise description (R? column indicates residue update)

"00 000010"  N	p3LD8U		Load byte unsigned, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + R

"00 000110"  N	p3LD16U	Load half-word unsigned, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 2 * R

"00 001010"  N	p3LD24U	Load 24-bit word unsigned, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 3 * R

"00 001110"  N	p3LD32U	Load word unsigned, 24-bit instruction, includes tag bits

If immediate (bit 18 set), address is register S + N, otherwise address is S + 4 * R

"00 010010"  N	p3LD8S		Load byte signed, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + R * ds

"00 010110"  N	p3LD16S	Load half-word signed, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 2 * R

"00 011010"  N	p3LD24S	Load 24-bit word signed, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 3 * R

"00 011110"  N	p3LD32S	Load word signed, 24-bit instruction, includes tag bits

If immediate (bit 18 set), address is register S + N, otherwise address is S + 4 * R

"00 100010"  N	p3LD8F		Load byte unsigned, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + R

"00 100110"  N	p3LD16F	Load half-word float, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 2 * R

"001001010"  N	p3LD24F	Load 24-bit word float, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 3 * R

"00 101110"  N	p3LD32F	Load word float, 24-bit instruction, includes tag bits

If immediate (bit 18 set),  address is register S + N, otherwise address is S + 4 * R

"00 110010"  N	p3LD8F2	Load byte float2, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + R

"00 110110"  N	p3LD16F2	Load half-word float2, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 2 * R

"00 111010"  N	p3LD24F2	Load 24-bit word float2, 24-bit instruction

If immediate (bit 18 set), address is register S + N, otherwise address is S + 3 * R

"00 111110"  N	p3LD32F2	Load word float2, 24-bit instruction, includes tag bits

If immediate (bit 18 set), address is register S + N, otherwise address is S + 4 * R

"01 000010"  N	p3ST18		Store byte converting register format to memory format

If immediate (bit 18 set), address is register S + N, otherwise address is S + R

"01 000110"  N	p3ST16		Store half-word converting register format to memory format

If immediate (bit 18 set), address is register S + N, otherwise address is S + 2 * R

"01 001010"  N	p3ST24		Store 24-bit word converting register format to memory format

If immediate (bit 18 set), address is register S + N, otherwise address is S + 3 * R

"01 001110"  N	p3ST32		Store word converting register format to memory format

If immediate (bit 18 set), address is register S + N, otherwise address is S + 4 * R

"01 010010"  N	p3LEA8		Place memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + R

"01 010110"  N	p3LEA16	Place memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + 2 * R

"01 011010"  N	p3LEA24	Place memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + 3 * R

"01 011110"  N	p3LEA32	Place memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + 4 * R

"01 100010"  Y	p3ADD, p3ADDI R,I + S => D, if types differ is currently unimplemented trap

Residue register captures the carry, for signed ADD, carry can be either zero, +1 or -1

For floating-point ADD residue register captures round-off error

Floating-point ADD makes use of and may modify the exponent tag bits

"01 100110"  Y	p3SUB, p3SUBI	R,I - S => D, if types differ is currently unimplemented trap

Residue register captures the carry, for signed SUB, carry can be either zero, +1 or -1

For floating-point SUB residue register captures round-off error

Floating-point SUB makes use of and may modify the exponent tag bits

“01 101010”   Y	p3MUL, p3MULI R,I * S => D, if types differ is unimplemented trap

Residue register captures upper half of product for signed or unsigned multiply

For floating-point MUL residue register captures round-off error

Floating-point MUL makes use of and may modify the exponent tag bits

“01 101110”   Y	p3DIV, p3DIVI	S / R,I => D, if types differ is unimplemented trap

Residue register captures remainder for signed or unsigned divide

For floating-point DIV residue register captures a floating-point remainder

Floating-point DIV makes use of and may modify the exponent tag bits

"01 110010"  Y	p3AND, p3ANDI Logical AND between R,I and S to D irrespective of data types

"01 110110"  Y	p3ORI, p3ORII	Logical OR between R,I and S to D irrespective of data types

"01 111010"  Y	p3XOR, p3XORI	Logical exclusive or between R,I and S to D irrespective of data types

"01 111110" N	p3CMP, p3CMPI S subtracted from R,I and placed in register D

"10 0xxx10"

INSRT, INSRTI

EXTRCT, EXTRCTI

SHL, SHLI

MAX, MAXI

MIN, MINI

FUNC

INTP-BLEND-LERP, FINTP

CMOV

"10 1xxx10"

JMPcc, CALLcc

BRcc, BSRcc

BBS0

BBS1

BBC0

BBC1

BRccRC, JMPccRC

CASE, CASEI

"11 xxxx10"	reserved for 40-bit instructions

32-bit Instruction descriptions:

For TROC24 with byte addressing and 32-bit instructions the scaled index register R added to the base register S and to the T register or immediate value. The three additional op-code bits in 32-bit instructions modify the scaling of the index register.  The three bits indicate the number of data grouped items (“gs” from one to eight).  Thus scaling of R can range from one to eight times the data size.  If R is specified as the PC (register 31) a value of zero is used.  It is OK to use the PC as a base register.  Loading the PC (i.e. D is register 31) causes a branch.  The symbol “T,I” represents value of register T or immediate value I depending on if bit 18 is set.

The 32-bit instructions use the full TROC interpretation of N.

For TROC24 the 32-bit instructions that load/store 32-bit data are repurposed for load/store of tagged values.

The 32-bit instructions split the op-code into two parts: Bits 5 down to 0 contain “xxxx00” where the x’s are the LSB op-code bits.  Bits 17 down to 16 contain the most significant two bits.  When “00” it is a load instruction.  When “11” the op-code space is reserved for 48-bit instructions.

Op-code     R?	Mnemonic	Precise description (R? column indicates residue update)

"00 000000"  N	p4LD8U		Load byte unsigned, 32-bit instruction

Memory address is register S + T,I + R * gs.  If R is PC then zero is used in place of PC.

"00 000100"  N	LD16U		Load half-word unsigned, 24-bit instruction

Memory address is register S + T,I + 2 * R * gs.  If R is PC then zero is used in place of PC.

"00 001000"  N	LD24U		Load 24-bit word unsigned, 24-bit instruction

Memory address is register S + T,I + 3 * R * gs.  If R is PC then zero is used in place of PC.

"00 001100"  N	LD32U		Load word unsigned, 24-bit instruction, includes tag bits

If immediate, address is register S + N, otherwise address is S + 4 * R * gs

"00 010000"  N	LD8S		Load byte signed, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + R * gs

"00 010100"  N	LD16S		Load half-word signed, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + 2 * R * gs

"00 011000"  N	LD24S		Load 24-bit word signed, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + 3 * R * gs

"00 011100"  N	LD32S		Load word signed, 24-bit instruction, includes tag bits

If immediate, address is register S + N, otherwise address is S + 4 * R * gs

"00 100000"  N	LD8F		Load byte unsigned, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + R * gs

"00 100100"  N	LD16F		Load half-word float, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + 2 * R * gs

"00 101000" N	LD24F		Load 24-bit word float, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + 3 * R * gs

"00 101100"  N	LD32F		Load word float, 24-bit instruction, includes tag bits

If immediate, address is register S + N, otherwise address is S + 4 * R * gs

"00 110000"  N	LD8F2		Load byte float2, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + R * gs

"00 110100"  N	LD16F2		Load half-word float2, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + 2 * R * gs

"00 111000"  N	LD24F2		Load 24-bit word float2, 24-bit instruction

If immediate, address is register S + N, otherwise address is S + 3 * R * gs

"00 111100"  N	LD32F2		Load word float2, 24-bit instruction, includes tag bits

If immediate, address is register S + N, otherwise address is S + 4 * R * gs

"01 000000"  N	ST18		Store byte converting register format to memory format

If immediate, address is register S + N, otherwise address is S + R * gs

"01 000100"  N	ST16		Store half-word converting register format to memory format

If immediate, address is register S + N, otherwise address is S + 2 * R * gs

"01 001000"  N	ST24		Store 24-bit word converting register format to memory format

If immediate, address is register S + N, otherwise address is S + 3 * R * gs

"01 001100"  N	ST32		Store word converting register format to memory format

If immediate, address is register S + N, otherwise address is S + 4 * R * gs

"01 010000"  N	LEA8		Place byte memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + R * gs

"01 010100"  N	LEA16		Place half-word memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + 2 * R * gs

"01 011000"  N	LEA24		Place 24-bit memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + 3 * R * gs

"01 011100"  N	LEA32		Place word memory address into D, type set to unsigned

If immediate, address is register S + N, otherwise address is S + 4 * R * gs

"01 100000" ~Y	p4ADD, p4ADDI T,I + R + S => D, if types differ is currently unimplemented trap

Residue register captures the carry, for signed ADD, carry can be either zero, +1 or -1

For floating-point ADD residue register captures round-off error

Floating-point ADD makes use of and may modify the exponent tag bits

Bit 24 is residue update enable, bit 25 is sign change for S and bit 26 is sign change for R

"01 100100" ~Y	p4SUB, p4SUBI	T,I - R - S => D, if types differ is currently unimplemented trap

Residue register captures the carry, for signed SUB, carry can be either zero, +1 or -1

For floating-point SUB residue register captures round-off error

Floating-point SUB makes use of and may modify the exponent tag bits

Bit 24 is residue update enable, bit 25 is sign change for S and bit 26 is sign change for R

“01 101000” ~Y	p4MUL, p4MULI T,I * S + R => D, if types differ is unimplemented trap

Residue register captures upper half of product for signed or unsigned multiply

For floating-point MUL residue register captures round-off error

Floating-point MUL makes use of and may modify the exponent tag bits

Bit 24 is residue update enable, bit 25 is sign change for S and bit 26 is sign change for R

“01 101100” ~Y p4DIV, p4DIVI	R|S / T,I => D, if types differ is unimplemented trap

For signed and unsigned, R is the remainder from a previous divide (R must be < T,I)

and is concatenated to S with appropriate sign handling.

Residue register captures remainder for signed or unsigned divide

For floating-point DIV residue register captures a floating-point remainder

Floating-point DIV makes use of and may modify the exponent tag bits

Bit 24 is residue update enable, bit 25 is sign change for S and bit 26 is sign change for R

"01 110000"  N	p4AND, p4ANDI Logical AND between T,I and R and S to D irrespective of data types

Bit 24 is complement enable for T,I; bit 25 is one’s complement enable for S and bit 26 is

complement enable for R

"01 110100"  N	p4ORI, p4ORII	Logical OR between T,I and R and S to D irrespective of data types

Bit 24 is complement enable for T,I; bit 25 is one’s complement enable for S and bit 26 is

complement enable for R

"01 111000"  N	p4XOR, p4XORI	Logical exclusive or between T,I and R and S to D irrespective of data

types.  Bit 24 is zero, bit 25 is one’s complement enable for S and bit 26 is complement

enable for R.

"01 111000"  N	p4XMERGE, p4MERGEI T,I bit by bit selects bits from either R (if zero bit) or S (if one bit)

to D irrespective of data types.  Bit 24 is one, bit 25 is one’s complement enable for S

and bit 26 is complement  enable for R.

"01 111100" N	p4CMP, p4CMPI S + R is subtracted from T,I and placed in register D

Floating-point CMP makes use of and may modify the exponent tag bits

"10 0xxx00"

INSRT, INSRTI

EXTRCT, EXTRCTI

SHL, SHLI

MAX, MAXI

MIN, MINI

MEDIAN

INTP-BLEND-LERP, FINTP

CMOV

"10 1xxx00"

JMPcc, CALLcc

BRcc, BSRcc

BBS0

BBS1

BBC0

BBC1

VECT

LOOP, LOOPI

BRccRC, JMPccRC

CASE, CASEI

"01 01xx00"

MISC (MMOV, PCND, PCB1, STM, LDM, MUX, TBD, TBD)

P3OPS

EADD, EADDI

"11 xxxx00"	reserved for 48-bit instructions

An alternative code is a group size of one to four and an additional byte if the third bit is set, R multipliers then range for one (1 * 1 + 0) to 17 (4 * 4 + 1, data-size * 1..4 + 0..1))