BROCC_log

James Brakefield

09/03/2025	Very simple forth uP

Initial idea

Single port return stack, dual port data stack (32x16-bits each)

No stack address adder: increment, decrement or change the three pointers as needed

RS-pointer, TOS-pointer & SOS-pointer

for best Fmax, eliminate some instruction decode and LUT RAM address adder time

8-bit instructions:

Op-codes: 6-bit plus return bit

Immediate values: Continuation/comma-bit & 7-bit value (concatenate until op-code)

2’s complement only

References

j1a & j1b dedicate an instruction bit to indicate a 15-bit literal

Zpu dedicates an instruction bit to push/append a 7-bit literal and supports five bit stack offsets

Only ~six ALU instructions   31 “emulate” instructions (usual 32-bit stack ops)

Plan B

Keep the stacks in block RAM, do the TOS, SOS (or stack offset reference), return address access at same time as instruction fetch.  Updates take place in next clock cycle, overlapped.  Requires four read ports and two write ports?  Ugh.

General architecture

To be called: BROCC (Block RAM Oriented Concatenative Computer).  Two dual-port block RAMs with single write to both to maintain coherence? Stack address pointers for TOS, ROS and AOS (Adjoined register On Stack), AOS address generated by displacement from TOS pointer.  Also there is the PC pointer.  If these block-RAMs encompass the entire address space it is a new variant of the stack machine?  There are five memory pointers: PC, TOS pointer, ROS pointer, Frame pointer and AOS pointer. And for registers: TOS, AOS, ROS, Index and Residue which are temporary copies of block-RAM contents?

Micro architecture

Instruction and return address block RAM with current instruction and return address always output.  Therefore address registers for PC and top of return stack,  Data and data stack block RAM with next on stack and adjoined stack location always output.  Therefore address registers for NOS (next on stack is the data stack item adjacent to TOS) and AOS.  In addition there are registers for TOS, index, residue and frame pointer.  Single cycle execution of usual Forth operators is provided.  TOOSM and OOSM ISA to be supported via shadow LUT RAMs for top of return stack, top of data stack, frame area and global or thread area.  LUT RAM expected to track changes to any of the stack areas.

ISA

Eight or nine bit instructions patterned after ZPU ISA.  One bit reserved to indicate push/append literal and one bit of non-literals reserved to indicate a longer instruction (64 one byte op-codes max).  With support of shadow LUT RAM single cycle OOSM and TOOSM ISAs can be supported.

Key features

Return and data stacks can be implemented entirely in LUT RAM, yielding the same single cycle performance.  The block RAM version supports addressable items on the stacks and much larger stacks.  The block RAM version supports single cycle execution by read-out of all useful stack items.  The ISA provides means to move the AOS pointer to nearby stack locations and therefore TOOSM/OOSM features.  The index register and the residue register are dedicated registers that avoid frequent movement to/from the stacks.  The return address (or can be a loop top pointer) and the TOS are also directly available, e.g. can be updated without competition with the block RAM data ports.

Instructions and data can byte aligned whereas the stack items are word aligned.  Implementation of byte alignment can be implemented by even/odd block RAM banks.

Having a loop top address on the top of the return stack allows increment/decrement index and loop if non-zero instructions.  Other instructions such as pruning the data stack after each loop also possible.  It is expected that a frame area also include a pointer to the previous frame pointer.  Likewise the locations before the frame pointer can be used for return results and/or subroutine parameters, this implying that offsets from the frame pointer can be both positive and negative.