STack/Accumulator Oriented Computer Architecture (STAOC) Description

James Brakefield ©2020

Preface:

STAOC is part of a family of computer architectures.  There are commonalities within the family: Little Endian, A register file, typically of 16 or 32 registers, and a main memory.  The register bit length defines the largest data size and the maximum addressable main memory.  Each family member supports three or four data sizes.  The smallest addressable memory unit is called a byte and can be eight, nine or twelve bits in size.  Distinct instruction sets exist for each member and each instruction set provides instructions of several sizes.

Immediate and displacement values needed by an instruction have a byte length field of three bits within the instruction and the value follows the instruction (for small values the value is coded within the instruction).

The family currently includes a RISC machine (ROC32_8), a stack/accumulator machine (STAOC32_8) and the multi-denominational ALT-A machine (combining stack, RISC and x86 style ISAs within a single architecture).  Closely related are alternative variations on the MSP430, the VAX and the x86 (alt_430, alt_vax and alt_x86) ISAs.

Introduction:

STAOC has several variants that could be implemented.  Byte size could be 12-bits leading to possible data sizes of 12, 24, 36 and 48 bits.  More commonly byte size is 8-bits and the four data sizes are 8, 16, 24 and 32 bits; or 8, 16, 32 and 64 bits.  The nomenclatures for these variations are: STAOC48_12, STAOC32_8 or STAOC64_8.  Herein STAOC32_8 will be described.

STAOC32_8 has four instruction sizes of 8, 16, 24 and 32 bits with any immediate value following the instruction.  An immediate can be of any size from one to eight bytes.  An immediate of -8 thru 15 is encoded within the instruction. In general, instructions are byte aligned, however by choosing a suitable combination of instruction and immediate lengths 16-bit or 32-bit alignment can be maintained.

STAOC uses a register file for data and return stacks, frame area and a global/thread area.  Stack spill and restore mechanism is undefined at this time.  With current FPGA technology stacks can be from 32 to 256 registers.  For embedded applications this may be sufficient and allows a simple single cycle design with one instruction executed per clock.  Using Block RAM instead of LUT RAM for the stacks/register file with a lookaside mechanism should be doable within a high throughput, pipelined design.

The STAOC ISA has four instruction sizes and each larger size supports additional features.  The one byte instructions are the usual stack operators that take two operands from the data stack and return one result.  There is room for 64 such instructions.  Two byte instructions have a five bit field for a stack reference, a one bit field for stack pop/push, a one bit field for replace operations (result put back into the stack reference), and a one bit field to indicate an immediate instead of a stack reference.  Again there is room for 64 such instructions.  The three byte instructions add a predicate generate bit that updates a condition code register and a five bit condition select field to cause the instruction to execute.  The three and four byte instructions include additional op-code bits and augment the stack reference to a larger set of stack references.

The stack reference in the short format (two byte instruction) provides access into the data stack or the frame area (12 locations each) and 4 locations into return stack and global registers.  In the longer formats (three and four byte instructions) four times as many locations of each are accessible.

The five bit stack reference field when used as an immediate uses the 32 possible values in three ways: A zero to fifteen positive immediate, a minus one to minus eight negative immediate, or to indicate that a one to eight byte immediate follows the instruction.

Instruction Formats:

Single byte			00xxxxxx

Two byte generic		01xxxxxx iprnnnnn

Two byte with stack reference	01xxxxxx 0prnnnnn

Two byte with immediate	01xxxxxx 1pvnnnnn

Three byte generic		10xxxxxx iprnnnnn nnuccccc

Three byte with stack reference	10xxxxxx 0prnnnnn nnuccccc

Three byte with immediate	10xxxxxx 1pvnnnnn xxuccccc

Four byte generic		11xxxxxx iprnnnnn nnuccccc xxxxxxxx

Four byte withstack reference	11xxxxxx 0prnnnnn nnuccccc xxxxxxxx

Four byte with immediate	11xxxxxx 1pvnnnnn xxuccccc xxxxxxxx

00, 01, 10 or 11	Instruction length bits

X	op-code bit

P	push/pop TOS (meaning depends on R bit: =0 push result, =1 pop TOS)

R	Replace operation to stack reference register

N	Five to seven bit stack reference field or immediate bit field and additional op-code bits

U	Update condition-code register

V	swap operands enable, where useful, additional op-code bit otherwise

C	Five predicate condition bits (if condition matches instruction performed otherwise a NOP)

I	Immediate flag bit (MSB of second byte)

0nnnn	immediate of 0 thru 15

11nnn	immediate of -8 thru -1

10nnn	signed immediate of one to eight bytes follows (considered part of the instruction)