Multi-Accumulator Oriented Computer: A modern VAX ISA

James Brakefield 2026

Abstract:

This is a legacy microprocessor effort which tries to capture the ambiance of the VAX ISA whilst not suffering from the VAX ISA complexity.  The design supports 16 general purpose registers including the PC, SP, frame pointer and a general purpose Residue register.  Four data sizes and four data types are supported.  Each register can hold any combination of data size and type.

In line with other ISAs there are four data types and four data sizes.  Of course VAX had this already.  Here we embellish the registers with a type code and additional bits for canonical floats (additional exponent and fraction bits).  Type-codes help reduce the number of op-codes.  Implied coercions between operand types may or may not be supported.

This ISA is designed so there is at most one data memory location per instruction.  There is support for a single indirection of the VAX addressing modes (register, register indirect, register indirect post increment, register indirect pre-decrement, register indirect with offset).  24-bit encoding is as op-code, three register fields, addressing mode field (one of eight) and register location (either first source or the destination) of its register field.  Using the PC as a source register implies an immediate operand.

The ISA uses a variety of instruction sizes.  Basic sizes are 16-bits for single operand/result instructions and 24-bits for two operand/one result instructions.  Immediate and displacement fields encode their length which can be additional bytes.  Two such fields are possible: a register displacement and an immediate value or absolute or relative location.

As a 32-bit an embedded computer a single DRAM chip suffices for entire memory address space.  Thus, page mapping and other virtual memory infrastructure is unnecessary.  Cache support is much simpler.

ISA preliminaries:

Four combinations of data type and date size result in sixteen variants of the basic two operand instructions: ADD, SUB, MUL and DIV.  The move, compare and Boolean instructions make use of the spare addressing mode bits to reduce the number of their code points.  Booleans do not need a data type, only the data size.  Four Boolean op-codes provided: AND, OR, XOR and TBD. The MOV instruction has two addressing mode fields and does not need a data type.  Compare instructions are similar to MOV with only two register fields.

An eight-bit prefix instruction specifies an index register scaled by data size.

Immediate fields contain a size code:

4-bit register field: value of 0..9 or a following 8/16/24/32/48/64-bit signed immediate

8X displacement field: -64 to +63 value or a 14/22/32/48/64-bit signed immediate

Single operand/destination instructions in 16-bits: CLR, INC, DEC, TST.  As CLR does not need a type the unused code points are allocated to Set to NaR, Set to all one bits, Set to plus one.

8-bit encoding:

Index register prefix				XXXXRRRR

Enable predicate register use			XXXXXXXX

Breakpoint					XXXXXXXX

Trap						XXXXXXXX

NOP						XXXXXXXX

16-bit encoding:

Single memory/register location			XXXXXXXXRRRRmmms

8 bits	Op-code (also encodes data size)

4 bits	Register

3 bits	Addressing mode: Register, register indirect, (R++), (--R), (R+delta8), (R+delta16), (R+delta32)

1 bit	Partial data size

The CLR instruction (set to zero) needs four sizes and no type, the other 12 combinations are set to NaR, set to All ones or set to Plus one

24-bit encoding:

8 bits	Op-code (also encodes data size)	XXXXXXXXRRRRSSSSDDDDmmmx

4 bits	Register 1

4 bits 	Register 2

4 bits	Register 3

Addressing mode: Register, register indirect, (R)++, (--R), (R + offset); no double indirection

1 bit	Memory reference location (first or third register)

ISA Summary:

(20) 8-bit instructions

(32) 16-bit single operand code points (4 op-codes, 4 types, 4 sizes, less one bit)

(84) 24-bit two operand code points

(64) ADD, SUB, MUL, DIV (4 types, 4 sizes each)

(16) CMP (4 type settings, 16 size combinations less two bits)

(4) MOV (16 size combinations less two bits)

Leaving (120) code points for other instructions