Alternative VAX ISA

Jim Brakefield

Abstract:

This is a legacy microprocessor effort which tries to capture the ambiance of the VAX ISA whilst not suffering from the ISA complexity.  The design supports 16 general purpose registers including the PC, SP and frame pointer.  Four data sizes are supported along with an optional predicate field.  One bit of the 8-bit op-code is used to enable the predicate mechanism.

The ISA uses a variety of instruction sizes.  Basic sizes are 16-bits for single operand/result instructions and 24-bits for two operand/one result instructions.  Invoking the predicate field lengthens the instruction by eight bits.  Immediate and displacement fields encode their length which can be additional bytes.  Two such fields are possible: a register displacement and an immediate value or absolute or relative location.

The ISA is designed so there is at most one memory location per instruction.  Support a single indirection of the VAX addressing modes (register, register indirect, register indirect post increment, register indirect pre-decrement, register indirect with offset).  24-bit encoding is as op-code, three register fields, addressing mode field (one of five) and register location (one of three) of its register field.  Using the PC as a source register implies an immediate operand.

16-bit prefix instruction specifies a scaled index register; 0 to 15 shift amount factor is S register field.

Immediate fields contain a size code:

4-bit register field: value of 0..9 or a following 8/16/24/32/48/64-bit signed immediate

8X displacement field: -64 to +63 value or a 14/22/32/48/64-bit signed immediate

Single operand/destination instructions in 16-bits: CLR, SET, INC, DEC, TST, FTST

Single operand with immediate in 16 bits: ADDI, LDI, etc.  unused location field used as part of the op-code.

The predicate bit enables an addition 8-bits allocated to a four bit predicate generate field, a three bit predicate shift register examine field and a one bit shift register bit polarity.  If the shift register bit matches the polarity bit the instruction is not executed.

8-bit codes

Index register prefix				XXXXRRRR

16-bit encodings:

Single memory/register location			XXXXXXXpRRRRmmmi

8 bits	Op-code (also encodes data size)

4 bits	Register

3 bits	Addressing mode: Register, register  indirect, (R++), (--R), (R+delta8), (R+delta16), (R+delta32)

Scale factor prefix instruction			XXXXXXXpRRRRssss

8 bits	Op-code

4 bits	Register

4 bits	Scale factor (shift count)		(use one bit for a 3X scaling?)

24-bit encoding:

8 bits	Op-code (also encodes data size)	XXXXXXXpRRRRSSSSDDDDmm-p

4 bits	Register 1

4 bits 	Register 2

4 bits	Register 3

Log2(5)	Addressing mode: Register, register  indirect, (R)++, (--R), (R+offset); no double indirection

Log2(3)	Memory reference location (first, second or third register)

Reg1 or reg2 is PC: 8X immediate field, both reg1 and reg2 are PC: PC used for reg2

Predicate byte:					PPPPSSSp

32-bit encoding:

8 bits	Op-code (also encodes data size)

4 bits	Predication field

1 bit	Status register update enable

5 bits	Register 1

5 bits 	Register 2

5 bits	Register 3

Log2(5)	Addressing mode: Register, register  indirect, (R)++, (--R), (R+offset); no double indirection

Log2(3)	Memory reference location (first, second or third register)

(Rule needed for the case when two or more register fields have the same register number)

32-bit encoding with scaled index register:

8 bits	Op-code (also encodes data size)

5 bits	Register 1

5 bits 	Register 2

5 bits	Register 3

5 bits	Index register (scaled by data size)

Log2(5)	Addressing mode: Register, register  indirect, (R)++, (--R), (R+offset); no double indirection

Log2(3)	Memory reference location (first, second or third register)