Alt_ISA Status

Common to all:	Memory:	Four 8-bit wide block RAMS with 32-bit byte rotate,

adr & adr+4 fed to four 2:1 address muxes

Alignment:	Instructions and data byte aligned, byte addressing

Endian:		Little

Status register:	Present

Basic instructions: ADD, ADC, SUB, SBC, AND, OR, XOR, Bcc, CALL, RTN, LD, ST,

MUL, MULU, MULUS, ROT, ASR, LSR, LSL plus immediate versions

DIV & IDIV defined, load & store for all date sizes signed/unsigned

Floating-point defined: FADD, FSUB, FMUL, FDIV and separate floating-point LD/ST inst.

Variable length immediate: two or three bit size field for number of immediate bytes

Immediate values and/or offsets directly follow instruction

Comments:	Floating point departs from IEEE-754 & supports gradual overflow

Alt_MSP430:	Docs: 		Business case, op-codes, instruction formats

Choices:	24 or 32-bit registers, addresses & ALU,

R0: PC, R1: SR, R2: SP, R3: FP

Three data sizes, 16-bit instructions

TBDs:		Prefix instruction formats & prefix op-codes;

Detailed ISA doc

Comments:	Relatively well defined, not fully compatible addressing modes

Alt_RISC:	Docs:		Business case, op-codes, instruction formats

Choices:	32 registers; 24 or 32-bit registers, addresses & ALU

Three data sizes, 24-bit instructions

Done:		ROIS24_24: 24-bit data size only implementation

ROIS24_32: 8, 16 & 32-bit data, complete definition

TBDs:		Stack mode register file implementation

Load/store op-codes defined for floating-point data

Comments:	Relatively well defined

Six bit register fields lead to insufficient op-code space

Alt_VAX:	Docs:		Business case, instruction formats

Choices:	Prefix instruction formats, full set of alt_VAX op-codes

Four data sizes, 16 & 24-bit instructions (four bit register fields)

TBDs:		Detailed ISA doc

Comments:	Large number of VAX op-codes

Two register field instructions unnecessary, code space can be used for

instructions with immediate

Alt_x86:	Docs:		Business case, instruction formats

Choices:	Prefix instruction formats, full set of alt_x86 op-codes

Four data sizes

24 & 32-bit instructions

TBDs:		Detailed ISA doc

Comments:	Huge number of X86-64 op-codes

Quad_ISA:	Docs:		Business case, instruction formats, op-code lists

Choices:	Register files have 32 registers

8(macro), 16(stack), 24(RISC) & 24/32(X86)-bit instructions

TBDs:		Detailed ISA doc

Comments:	Relatively well defined except for the Macro facility

Immediate size field in uniform location

Four data sizes supported throughout

Limiting to 16 registers allows all instructions to be 24-bits

Common Details:

Byte addressing for instructions and data

All ISAs support three or four data sizes (such as 8/16/32 or 8/16/24/32)

Register file word size is as a minimum the largest data size

Register file size varies between different ISAs with 16 and 32 words typical

Register to register operations use an ALU width of the largest data size

Floating-point register values may have additional hidden bits in the register file (for performance optimization)

Temporary alignment restrictions for 16 and 32-bit data and instructions

For convenience in development and debug

May or may not have a condition code register

Condition code register exists in two forms:

A form that captures all of ALU results including overflow

A reduced form for save/restore which contains bits for sign, least significant bit (even/odd), exponent bits all ones and exponent bits all zeros and similarly for the mantissa, integer overflow, floating-point result inexact

Little Endian

Binary representation of unsigned integers

Two’s complement representation of signed integers

Floating-point departs from IEEE-754

Register format has additional exponent bits to accommodate gradual underflow and gradual overflow.

Register format has additional status bits to indicate exact versus inexact

For 24-bit and 32-bit floating point numbers

Sign, biased exponent, mantissa with hidden leading one bit and an inexact bit

If result of an operation on exact values does not give an exact result then the result becomes inexact

Inexact rounding rounds to the interval between exact values; with intervals alternating open and closed so that unbiased rounding can be done without incrementing or decrementing the truncated mantissa.

Least exponent value signifies underflow value (number of trailing zeros in mantissa is the amount to reduce the exponent value to get true exponent).

Greatest exponent values signifies overflow value (number of trailing zeros in mantissa is the amount to increase the exponent value to get true exponent).

There may be (design choice) more than one greatest or least exponent value.  The number of such values is always a power of two.

Support for 16-bit floating point values with either short or full size exponent

Predication, conditional execution and predicate shift register

An instruction bit is used to signify the predication fields which increase the instruction length by an eight bit field.  The first four bits identify the predicate to be placed on the shift register.  Besides the usual carry, sign, zero, odd, overflow choices there is an allocation for none (no bits placed on the shift register), within bounds (>=0 and < comparison value, >0 and <= comparison value), floating point inexact (the odd predicate), floating point normal (exponent bits not all ones nor all zeros), predicate is one and save extended result into destination register plus one.

The next three bits indicate the shift register bit to examine (shift register is eight bits long, referencing the last bit is a pass situation – the instruction executes normally.  The last bit is the polarity test on the shift register bit, if it matches the instruction executes normally, otherwise the instruction is a NOP.

Standard set of instruction operations

There are two operands and one result.  The instruction decode process identifies the register or memory locations of the operands and results.  One of the operands can be encoded into the instruction as an “immediate” value.  The size of the immediate in bytes is also encoded into the instruction (and lengthens the instruction).  A short immediate shall be sign extended to operand length.

(18) Standard two operand operations (compare instructions have a destination register)

ADD	Two’s complement addition supporting either signed or unsigned interpretation

SUB 	Two’s complement subtraction supporting either signed or unsigned interpretation

ADC	Two’s complement addition with carry supporting either signed or unsigned interpretation

SBC 	Two’s complement subtraction with “carry” supporting either signed or unsigned interpretation

CMP	Two’s complement subtraction with no register result (result provided either by condition code register or predicate processing)

AND	Binary bit by bit logical AND

OR	Binary bit by bit logical OR

XOR	Binary bit by bit logical exclusive or

ANDN	Binary bit by bit logical AND with the logical complement of the second operand

MUL	Least significant half of the product of signed or unsigned operands

MULU	Most significant half of the product of unsigned operands

MULUS	Most significant half of the product of signed operands

DIV	Quotient of dividing the first unsigned operand by the second unsigned operand

DIVS	Quotient of dividing the first unsigned operand by the second unsigned operand

EXTRCT	Second operand has extract bit location in the first operand and also (by bit wise concatenation) the bit length of the field extracted.  The extracted field is zero extended to result size.

EXTRCTS Second operand has extract bit location in the first operand and also (by bit wise concatenation) the bit length of the field extracted.  The extracted field is sign extended to result size.

INSRT	Second operand has insert bit location in the first operand and also (by bit wise concatenation) the bit length of the field inserted into the result.

FADD	Add two floating point operands producing a floating point result

FSUB	Subtract two floating point operands producing a floating point result

FMUL	Multiply two floating point operands producing a floating point result

FADD	Divide two floating point operands producing a floating point result

FCMP	Compare two floating point operands with no register result (result provided either by condition code register or predicate processing)

LSR	Logical shift right

LSL	Logical shift left

ASR	Arithmetic shift right

ROTL	Rotate left

BRcc	Conditional branch using the condition code register.  The first (destination) register field specifies the condition and the second field is the relative branch displacement.

JMPcc	Conditional branch using the condition code register.  The first (destination) register field specifies the condition and the second field register is added to the third field immediate to generate the branch address.

BSR	The first (destination) register field specifies the register to get the address of the next instruction and the second field is the relative branch displacement.

JSR	The first (destination) register field specifies the register to get the address of the next instruction and second field register is added to the third field immediate to generate the branch address.

PRED	Predicate from three predicate bits

Standard one operand operations

LDZCT	Leading zeros count

TRZCT	Trailing zeros count

LDOCT	Leading ones count

TROCT	Trailing ones count

INC

DEC

CLEAR

SET

SQRT	Unsigned integer square root

FSQRT	Floating point square root

FEXP	Floating point natural exponential

FLN	Floating point natural logarithm

FLOAT	Convert integer to floating point

INT	Extract integer portion of floating point value

EXPON	Extract exponent portion of floating point value

Trigonometric with operand or result in 360 degree rotations:

SINPI, COSPI, ATANPI

(18) Load and Store operations

LDI		Load immediate

LDB		Load byte

LDSB		Load signed byte

LDH		Load half

LDSH		Load signed half

LDW		Load word

LDSW		Load signed word

LD		Load long

STB		Store byte

STH		Store half

STW		Store word

ST		Store long

FLDH		Floating load half (float16 & float with 8-bit exponent?)

FLDW		Floating load word

FLD		Floating load long

FSTH		Floating store half (float16 & float with 8-bit exponent?)

FSTW		Floating store word

FST		Floating store long